3GPP TSG-SA2 Meeting #156E	S2-2305297r02

# Elbonia, 17th – 21th April, 2023	(was S2-23xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4544 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Session Management Associations with multiple PINs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 5.44.3, for a UE with PEGC, there are three cases to support PDU session association for a PIN: - Case 1: Associate one PIN per PDU session with same DNN and N-SSAI (not supported in SoH) - Case 2: Case 2: Associate multiple PINs for sharing one PDU session (merge into 4175) - Case 3: Associate one PIN to multiple PDU session Acording to S2-2305293, the following Editor’s Notes can be resolved and removed. Editor's note: One PIN served by more than one PDU sessions in PEGC is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to clarify Case 3 : Based on operator’s policy, URSP rules needs to enhance so that one PEGC serving one PIN may use different PDU Sessions for different groups of PINEs using different applications. The PEGC should be able to identify a group of PINEs in the same PIN that share the same PDU Session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of multiple PDU sessions association for PEGC with one or more PINs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******Start of changes*******

#### 5.44.3.1	PDU Session Establishment for PIN

When a PDU Session associated with a PIN is established by UE with PEGC, an SMF is selected according to clause 4.3.2.2.3 of TS 23.502 [3]. The UE with PEGC may use IP address allocation methods as specified in clause 5.8.2.2 (e.g. IPv6 Prefix Delegation feature).

One PEGC may serve more than one PINs and in this case the PEGC shall have at least one PDU Session for each PIN if the PIN traffic is via PEGC/5GC.

One PDU Session may be shared by a group of PINEs within a PIN served by the PEGC and in this case, one PIN may be served by more than one PDU sessions in the PEGC.

Editor's note:	How and whether to handle the case where PINs share a PDU session and local switching is FFS.

Editor's note:	The handling of the PEMC in 5GC in relation with PIN is FFS.

******* End of changes *******