3GPP TSG-SA2 Meeting #156-e	S2-2305344

# Online, , 17th Apr 2023 - 21st Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0253 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility procedures for RRC inactive UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The following EN needs to be resolved: Editor's note: It is FFS whether more details about messages to transfer MBS session information and MBS assistance information are required for UEs in RRC_IDLE state, where the connection setup procedure is applicable. The EN relates to the following text: When the UE in RRC_INACTIVE receiving multicast MBS session data, if the UE moves out of the RA, the UE initiates the Mobility Registration Update procedure to activate the associated PDU session(s) as specified in clause 4.2.2.2.2 of TS 23.502 [6]. According to TS 23.502, clause 4.2.2.2.2 during mobility registration update with PDU session activation, steps 5ff of UE initiated service request procedure are executed. TS 23.247 already documents impacts to service request procedures in clause 7.2.8, but it only list other trigger conditions and thus gives the impression that the additions to the service request procedure in clause 7.2.8 (to provide MBS session related information from SMF to NG RAN) do not apply if the service request procedure is triggerted as part of the mobility registration update procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Parts of theService Request procedure can also be triggered by the Mobility Registration Update procedure, and the related MBS specific additions in Clause 7.2.8 to provide the MBS session related information from SMF to NG RAN then apply. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved open issue for Mobility of UE in RRC_INACTIVE state receiving MBS data out of RA remains, During Mobility Registration Update no MBS session information might be provided to NG RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.8.1, 7.2.3.8.4, 7.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 7.2.8	Service request procedure

If the multicast MBS session is in Inactive state, the UE can go to CM-IDLE state or the user plane of the associated PDU Session may be deactivated even when the UE is in CM-CONNECTED state. When user plane of the associated PDU session is activated again, the SMF sends the MBS session information to NG-RAN during Service Request procedure as specified in TS 23.502 [6]. The MBS session information indicates that this MBS session is in Inactive state; and:

-	For the MBS supporting NG-RAN node, the NG-RAN establish the shared tunnel with the MB-UPF as usual. However, as the MBS session is in Inactive state, the NG-RAN node will not allocate related radio resource.

-	For the non-MBS supporting NG-RAN node, the unicast QoS flow associated with the MBS session are not established.

The service request procedure can also be executed while the multicast MBS session is in Active state:The UE can go to CM-IDLE state due to AN release as specified in TS 23.502 [6]. In addition, to activate the associated PDU session(s) parts of the service request procedure can also be executed during the Mobility Registration Update procedure as specified in clause 4.2.2.2.2 of TS 23.502 [6].  When the service request procedure is executed while the multicast MBS session is in Active state, the SMF provides the MBS session information indicating that the UE joined the MBS session to the NG-RAN within N2 SM container for the associated PDU Session.

NOTE:	Network triggered Service Request, if triggered for other reason, can also be used by the SMF to provide that MBS session information to NG-RAN.

End of changes