3GPP TSG-WG SA2 Meeting #157 	S2-2306776

Berlin, Germany, May 22 – 26, 2023	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Update on Ranging SL Positioning UE discovery

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes some updates and to remove ENs for the discovery.

1. Introduction/Discussion

For Ranging/Sidelink Positioning UE discovery procedure, this paper proposes two updates and remove two ENs.

1. The authorized Ranging/SL Positioning role is included in policy/parameter provisioning to UE. So this paper proposes that the UE needs to check whether the corresponding role is authorized or not during the discovery procedure.

2. It is agreed that the capabilities and the serving PLMN of the candidate Located UEs are used for Located UE selection in clause 5.2.2. This paper proposes that the capabilities and the serving PLMN of the candidate Located UEs can be obtained by target UE during the discovery message, so the target UE can use them for the selection.

3. For the following two ENs, it proposes that User Info ID is same to the one defined in TS 23.304, and can be configured by the network or assigned by application server.

Editor's note: Which information is included in User Info ID, especially for Located UE discovery, is FFS.

Editor's note: Which information is included in User Info ID and Target Info, especially for Located UE discovery, is FFS.

For Ranging/Sidelink Positioning UE Discovery & Selection functionality, this paper proposes to remove the ENs and update the description for alignment.

Editor's note:	Whether ProSe identifier is used during Ranging/SL Positioning UE direct discovery is FFS.

Proposal 1: ProSe identifier is needed, since the peer UE needs to know the ranging for which service, and also User Info ID of UE needs associate with a certain ProSe identifier.

Editor’s Note: It is FFS how Target UE and the discovered SL Reference UEs know the serving network not supporting Ranging/SL Positioning.

Editor’s Note: It is FFS whether and how LMF can indicate using “UE-only” operation to Target UE.

Proposal 2: The Ranging/SL Positioning Support is configured in the AMF according to network capabilities. AMF includes Ranging/SL Positioning Support indicator in the Registration Accept message to indicate that the serving network supports Ranging/SL Positioning. And based on agreement in clause 5.3.1 “A SL Positioning Server UE can be discovered and selected for result calculation, method determination, assistant data distribution and SL Reference UE selection in case of out-of-coverage or for UE-only Operation if the serving network does not support Ranging/SL Positioning.”, by using the mechanism of Ranging/SL Positioning Support capability, LMF does not need to indicate using “UE-only” operation to Target UE.

Editor’s Note: It is FFS how a Target UE can discover whether or not SL Reference UEs are currently not served by a network supporting Ranging/SL Positioning.

Proposal 3: SL Reference UEs can indicates the serving network supporting Ranging/SL Positioning during the discovery.

Editor’s Note: It is FFS whether a Target UE can/should use SL Positioning Server UE can also be determined by policy.

Proposal 4: Ranging/SL Positioning Support capability (the serving network supporting/not supporting Ranging/SL Positioning) is concluded and works well, and thus no need of policy enhancement.

Proposal 5: Some updates and alignments includes

- Network assisted ranging and SL positioning is not correct, and is changed to UE Positioning assisted by Sidelink Positioning and involving 5GC.

- Redundant description on a list of candidate Located UEs pre-configured in LMF, and proposes to remove one.

- On the located UE selection part, it is agreed that LMF can decide that LMF or target UE selects Located UEs. And Target UE needs to know the decision, to select by itself or not.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586.

# * * * * First change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Located UE: A SL Reference UE of which the location is known or is able to be known using Uu based positioning. A Located UE can be used to determine the location of a Target UE using Sidelink Positioning.

Network-based Operation: Operation of Ranging/Sidelink Positioning with the involvement of 5GC NFs for the service request handling and result calculation.

Positioning: A functionality, which detects a geographical location and optionally, velocity (of e.g. a mobile terminal).

Ranging: Refers to the determination of the distance between two UEs or more UEs and/or the direction of one UE (i.e. Target UE) from another UE (i.e. Reference UE) via PC5 interface.

Ranging/SL Positioning Application Identifier: A globally unique identifier identifying a specific Ranging/SL Positioning application, which can be mapped to a V2X service type or a ProSe identifier.

Relative position: An estimate of the UE position relative to other network elements or relative to other UEs.

Relative velocity: An estimate of the UE velocity relative to another UE.

SL Reference UE: A UE, supporting positioning of target UE, e.g. by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc. using sidelink.

NOTE 1:	SL Reference UE is understood as "Anchor UE" in RAN WGs.

Sidelink Positioning: Positioning UE using PC5 to obtain absolute position, relative position, or ranging information.

SL Positioning Client UE: A third-party UE, other than SL Reference UE and Target UE, which initiates Ranging/Sidelink positioning service request on behalf of the application residing on it.

NOTE 2:	The SL Positioning Client UE does not have to support Ranging/Sidelink positioning capability, but a communication between the SL Positioning Client UE and SL Reference UE/Target UE has to be established, either via PC5 or via 5GC, for the transmission of the service request and the result.

SL Positioning Server UE: A UE offering method determination, assistant data distribution and/or location calculation functionalities for Sidelink Positioning and Ranging based service. It interacts with other UEs over PC5 as necessary in order to determine Ranging/SL Position method, distribute assistant data and calculate the location of the Target UE. Target UE or SL Reference UE can act as SL Positioning Server UE if any of the functionalities is supported.

Target UE: A UE whose distance, direction and/or position is measured with the support from one or multiple SL Reference UEs using Sidelink in the Ranging based service and Sidelink positioning.

UE-only Operation: Operation of Ranging/Sidelink Positioning in which the service request handling and result calculation are performed by UE.

NOTE 3:	For UE-only Operation, the communication among UEs are over PC5.

User Info ID: The User Info ID is configured for Ranging/SL Positioning UE Discovery based on the policy of the HPLMN or via the Ranging/SL Positioning application server that allocates it. The definition of values of User Info ID is out of scope of this specification.

# * * * * Second change * * * *

## 5.2	Ranging/Sidelink Positioning UE Discovery & Selection

### 5.2.1	General

Both Model A and Model B discovery as defined in clause 6.3.2 of TS 23.304 [7] are supported for the 5G ProSe capable UEs discovery (including commercial and public safety use cases). During Ranging/SL Positioning UE direct discovery, the ProSe identifier indicates "Ranging/Sidelink Positioning"; During Ranging/SL Positioning Group Member Discovery, the Application Layer Group ID indicates a Ranging/Sidelink Positioning group that the UE belongs to.

Procedures for V2X communication over PC5 reference point as defined in TS 23.287 [6] are supported for the V2X capable UEs discovery. When unicast mode V2X communication procedure as defined in 6.3.3 of TS 23.287 [6] is used, the Service Type in Layer-2 link establishment procedure indicates "Ranging/Sidelink Positioning"; when broadcast mode or groupcast mode V2X communication procedure as defined in 6.3.1 or 6.3.2 of TS 23.287 [6] is used, Service Type indicates "Ranging/Sidelink Positioning".

The role(s) of the discovered UE (e.g. Target UE, SL Reference UE, SL Positioning Server UE, Located UE) is (are) included in discovery message (for 5G ProSe capable UE) and unicast link establishment messages (for V2X capable UE) as RSPP specific meta data. More details on this and other information conveyed during discovery can be found in clause 6.4.

The procedures for Ranging/Sidelink Positioning UE Discovery & Selection are defined in clause 6.4.

### 5.2.2	Located UE Discovery & Selection

When Sidelink Positioning is applied for Target UE in a 5GC-MO-LR or 5GC-MT-LR procedure, Located UE(s) is discovered and selected:

- When LMF determines to apply Sidelink Positioning for Target UE, LMF triggers the Target UE to perform discovery of the Located UE.

- When the Target UE determines to apply Sidelink Positioning, the Target UE triggers the discovery of the Located UE.

The LMF may be pre-configured with the list of candidate Located UEs including e.g. the capability, stationary information and location information of the Located UE(s), e.g. RSU or the operator deployed Located UE. The LMF may provide to the Target UE a list of candidate Located UE(s) as assistance data.

The discovery of Located UEs follows the same principles as specified in clause 5.2.1. The UE can indicate its role “Located UE” in its list of supported roles during discovery, if it is authorized to be a Located UE in a given PLMN as per the Authorization and Provisioning for Ranging/SL positioning service as specified in clause 5.1.

Editor's note:	Privacy aspects of sharing the location of a Located UE (e.g. to any Target UE) and exposing information about such privacy aspects during discovery (e.g. to facilitate Located UE selection by a Target UE) needs to be aligned with SA WG3.

A Target UE may discover and select one or more Located UEs to be used in the Ranging/SL positioning procedures as specified in clauses 5.3 through 5.5.

Multiple candidate Located UEs may be discovered, in that case, the Located UE(s) is selected from the candidate Located UEs list. The Located UE(s) is selected based on:

- Candidate list of Located UE(s), if available

- Capabilities of the candidate Located UE(s), e.g. the supported Sidelink Positioning methods

- The required positioning QoS

- Whether the serving PLMN of candidate Located UE(s) is same with serving PLMN of Target UE

When LMF determines SL positioning for target UE and trigger the discovery of the Located UE, LMF can decide that LMF or target UE selects Located UEs. If the decision is LMF selecting Located UEs, Target UE sends the multiple discovered candidate Located UEs to the LMF for the selection based on the indication of LMF selecting Located UE from LMF. After the LMF determines the selected Located UE(s), the LMF sends the selected Located UE(s) to the Target UE. Or Target UE selects Located UE(s) based on the indication of Target UE selecting Located UE from LMF

In this release of the specification, for UE Positioning assisted by Sidelink Positioning and involving 5GC, the Target UE shall discover and select Located UEs that are in the same serving PLMN of the Target UE.

LMF needs to provide located UE for discovery when AF request ranging between two UEs. Additionally, LMF may be provisioned with Located UEs that may be sent to target UE as candidate list of located UEs.

NOTE: 	The role of being “Located UE” is dynamic and can change over time, in particular if the Located UE is moving. Hence, the discovery results need to be refreshed if there is a (significant) delay between discovery and initiating of a ranging procedure with a discovered Located UE. How often this is done is up to UE implementation.

Editor's note:	The criteria to determine if the location of a UE is sufficiently stable and accurate in order for the UE to announce itself as a “Located UE” needs to be aligned with RAN WGs.

### 5.2.3	SL Positioning Server UE Discovery & Selection

SL Positioning Server UE Discovery & Selection is performed by the Target UE, when it meets one or more of the following criteria:

- The Target UE and the discovered SL Reference UEs are currently not served by a network supporting Ranging/SL Positioning (e.g. because they are out-of-coverage or the serving network does not support Ranging/SL Positioning).

- The Target UE is not capable of performing SL Positioning Server UE functionalities.

The Ranging/SL Positioning Support is configured in the AMF according to network capabilities. If AMF receives Ranging/SL Positioning Capability from UE, AMF may include Ranging/SL Positioning Support indicator in the Registration Accept message to indicate that the serving network supports Ranging/SL Positioning. SL Reference UEs can indicate the serving network supporting Ranging/SL Positioning during the discovery.

The discovery of SL Positioning Server UE follows the same principles as specified in clause 5.2.1. The UE can indicate its role “SL Positioning Server UE” in its list of supported roles during discovery, if it is authorized to be a SL Positioning Server UE in a given PLMN as per the Authorization and Provisioning for Ranging/SL positioning service as specified in clause 5.1.

The Target UE shall discover and select a SL Positioning Server UEs that are in the same or different serving PLMN of the Target UE and the Reference UE(s).

Editor’s note: Other criteria for the selection of SL Positioning Server UE can be coordinated with RAN WGs.

# * * * * Third change * * * *

### 6.4.2	Ranging/SL Positioning UE discovery with 5G ProSe capable UE

#### 6.4.2.1	Ranging/SL Positioning UE direct discovery

As defined for the Model A and Model B discovery in TS 23.304 [7]:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

Figure 6.4.2.1-1 illustrates the procedure for Ranging/SL Positioning UE discovery with 5G ProSe capable UE using Model A discovery.

Figure 6.4.2.1-1: Ranging/SL Positioning UE discovery in Model A

1.	The Announcing UE (UE-1) sends a Ranging/SL Positioning Announcement message. The Ranging/SL Positioning Announcement message includes the Type of Discovery Message, security protection element, RSPP metadata information, User Info ID of Announcing UE.

Announcing UE sends the Announcement message only if it is authorized to be the corresponding UE role in RSPP metadata information.

For the Announcing UE acting as Located UE, Ranging/SL Positioning Announcement message also includes the serving PLMN of Announcing UE, and RSPP metadata information includes the capability of the Announcing UE (e.g. the supported Sidelink Positioning methods).

The Destination Layer-2 ID used to send the Ranging/SL Positioning Announcement message is configured in clause 5.2.

The Source Layer-2 ID to send the Ranging/SL Positioning Announcement message is self-assigned by the Announcing UE

A Monitoring UE determines the Destination Layer-2 ID for signalling reception based on the configuration in clause 5.2.

A Monitoring UE selects the Announcing UE based on the information received in step1.

Editor's note:	The RSPP metadata information (e.g., the role(s) of the Announcing UE) is included as the metadata in the Announcement message, which value is determined by RAN2.

Editor's note:	How the RSPP metadata information is used to convey that a UE is a “Located UE” needs to be aligned with RAN2.

Figure 6.4.2.1-2 illustrates the procedure for Ranging/SL Positioning UE discovery with 5G ProSe capable in Model B discovery.

Figure 6.4.2.1-2: Ranging/SL Positioning UE discovery in Model B

1.	The Discoverer UE (UE-1) sends a Ranging/SL Positioning Solicitation message. The Ranging/SL Positioning Solicitation message includes the Type of Discovery Message, security protection element, User Info ID of Discoveree UE, User Info ID of Discoverer UE and optionally RSPP metadata information.

The Destination Layer-2 ID used to send the Ranging/SL Positioning Solicitation message is configured in clause 5.2.

The Source Layer-2 ID to send the Ranging/SL Positioning Solicitation message is self-assigned by the Discoverer UE

A Discoveree UE determines the Destination Layer-2 ID for signalling reception based on the configuration in clause 5.2.

Editor's note:	The RSPP metadata information (e.g., the specific Role(s) to be discovered) is included as the metadata in the Solicitation message, which value is determined by RAN2.

2	The Discoveree UE that matches the Ranging/SL Positioning solicitation message (e.g., RSPP metadata information) responds to the Discoverer UE with the Ranging/SL Positioning Response message. The Ranging/SL Positioning Response message includes Type of Discovery Message, security protection element, RSPP metadata information, user Info ID of the Discoveree UE.

Discoveree UE sends the Response message only if it is authorized to be the corresponding UE role in the solicitation message.

For the discovered Role being Located UE, Ranging/SL Positioning Response message also includes the serving PLMN of Discoveree UE, and RSPP metadata information includes the capability of the Discoveree UE (e.g. the supported Sidelink Positioning methods).

The Source Layer-2 ID used to send the Ranging/SL Positioning Response message is specified in clause 5.2.

The Destination Layer-2 ID is set to the Source Layer-2 ID of the received Ranging/SL Positioning Solicitation message.

Editor's note:	The RSPP metadata information (e.g., the role(s) of the Discoveree UE) is included as the metadata in the Announcement message, which value is determined by RAN2.

#### 6.4.2.2	Ranging/SL Positioning group member discovery with 5G ProSe capable UE

Figure 6.4.2-1 illustrates the procedure for Ranging/SL Positioning group member discovery with 5G ProSe capable in Model A discovery.

Figure 6.4.2.2-1: Ranging/SL Positioning group member discovery in Model A

1.	The Announcing UE sends a Ranging/SL Positioning Group member discovery Announcement message. The Announcement message includes the Type of Discovery Message, User info ID of the Announcer UE, Group ID and RSPP metadata information.

Announcing UE sends the Announcement message only if it is authorized to be the corresponding UE role in RSPP metadata information.

For the Announcing UE acting as Located UE, Ranging/SL Positioning Group member discovery Announcement message also includes the serving PLMN of Announcing UE, and RSPP metadata information includes the capability of the Announcing UE (e.g. the supported Sidelink Positioning methods).

The Destination Layer-2 ID used to send and receive the Ranging/SL Positioning Group member discovery Announcement message is described in clause 5.8.1.2 of TS 23.304 [7].

The Source Layer-2 ID used to send and the Ranging/SL Positioning Group member discovery Announcement message is self-assigned by the Announcing UE.

Editor's note:	The RSPP metadata information (e.g., the role(s) of the Announcing UE) is included as the metadata in the Announcement message, which value is determined by RAN2.

Figure 6.4.2.2-2 illustrates the procedure for Ranging/SL Positioning Group member discovery with 5G ProSe capable in Model B discovery.

Figure 6.4.2.2-2: Ranging/SL Positioning Group member discovery in Model B

1.	The Discoverer UE (UE-1) sends a Ranging/SL Positioning Group member discovery Solicitation message. The Solicitation message includes the Type of Discovery Message, User Info ID of Discoverer, Group ID, optionally User Info ID of Discoveree UE and optionally RSPP metadata information.

The Destination Layer-2 ID used to send and receive the Solicitation message is described in clause 5.8.1.2 of TS 23.304 [7].

The Source Layer-2 ID to send the Ranging/SL Positioning Solicitation message is self-assigned by the Discoverer UE

Editor's note:	The RSPP metadata information (e.g., the specific Role(s) to be discovered) is included as the metadata in the Solicitation message, which value is determined by RAN2.

2	The Discoveree UE that matches the values of the parameters (including Group ID and User Info ID of Discoveree UE) contained in the Ranging/SL Positioning Group member discovery solicitation message, responds to the Discoverer UE with the Ranging/SL Positioning Group member discovery Response message. The Ranging/SL Positioning Response message includes Type of Discovery Message, the Discoveree info, the RSPP metadata information and the Group ID.

Discoveree UE sends the Response message only if it is authorized to be the corresponding UE role in the solicitation message.

For the discovered Role being Located UE, Ranging/SL Positioning Group member discovery Response message also includes the serving PLMN of Discoveree UE, and RSPP metadata information includes the capability of the Discoveree UE (e.g. the supported Sidelink Positioning methods).

The Source Layer-2 ID used to send the Ranging/SL Positioning Group member discovery Response message is self-assigned.

The Destination Layer-2 ID is set to the Source Layer-2 ID of the received Ranging/SL Positioning Group member discovery Solicitation message.

Editor's note:	The RSPP metadata information (e.g., the role(s) of the Discoveree UE) is included as the metadata in the Announcement message, which value is determined by RAN2.

# * * * * End of changes * * * *