SA WG2 Meeting #S2-157	S2-2306311

22 - 26 May, 2023, Berlin, Germany

3GPP TSG-RAN WG3 Meeting #119bis-e	     R3-232084

E-meeting, 17 – 26 April, 2023

Title:	LS on the enhancements to restricting paging in a limited area

Response to:

Release:	Release 18

Work Item:	Netw_Energy_NR-Core

Source:	RAN3

To:	RAN2, SA2

Cc:

Contact Person:

#### Name:	Håkon Helmers

E-mail Address:	hakon.helmers@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 is currently working on enhancements to restrict paging in a limited area as described in TR 38.864 clause 6.5.4, and has made the following agreements for RAN-initiated paging:

a)	It is gNB’s implementation to decide to which UEs the paging enhancement technique is applied.

b)	Introduce the recommended paging SSB list in the F1AP PAGING message sent from the gNB-CU to the gNB-DU.

c) Introduce the recommended paging SSB list in the F1AP UE CONTEXT RELEASE COMPLETE message sent from the gNB-DU to the gNB-CU.

According to above agreements, paging for UEs in RRC INACTIVE state may be restricted to a limited number of beams within a cell. RAN3 would respectfully like to ask RAN2 the following question:

Q1: Whether RAN3’s agreements have any impact on, or could be impacted by, TS 38. 304 clause 7 Paging, e.g., related to the following paragraph.

TS 38.304 clause 7.1:

“In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.”

RAN3 also discussed the enhancements to restrict paging in a limited area for UEs in RRC IDLE state based on the following mechanism:

- The signalling procedure of the paging enhancement for UEs in RRC_IDLE state is given as follows:

- The last serving gNB reports a list of recommended beams (e.g., last few served SSBs) of the gNB to the AMF in UE CONTEXT RELEASE COMPLETE message over NGAP

- The AMF stores the list of recommended beams while UE is in RRC_IDLE state

- During idle mode paging, the AMF sends the stored recommended beams back to the last serving gNB in the NGAP Paging message.

- To assist in NGAP Paging, it is expected that the gNB shall provide the list of recommended beams to the CN only for UEs that are known to be stationary, but it is up to gNB’s implementation to decide to which UEs the paging enhancement technique is applied.

- RAN3 assumes that the list of recommended beams will be transparent to AMF.

Q2: RAN3 would respectfully like to ask SA2 if there are any issues from CN perspective with this mechanism.

2. Actions:

To RAN2: 	RAN3 kindly asks RAN2 to consider RAN3’s agreements listed above and provide answer to Q1.

To SA2:		RAN3 kindly asks SA2 to provide answer to Q2.

3. Dates of Next TSG-RAN WG3 Meetings:

3GPP TSG RAN WG3#120		22 - 26 May, 2023				Incheon, Korea

3GPP TSG RAN WG3#121		21 - 25 August, 2023			Toulouse, France