3GPP TSG-WG SA2 Meeting #157 	S2-2306379

# Berlin, Germany, May 22-26, 2023	(revision of S2-2304014, S2-2306065)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1284 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting UE centric AR Telephony Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | WID NG_RTC has objective to support AR telephony communication in IMS network, which has been concluded in TR 23.700-87. This CR reflects the TR conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause for both the UE centric AR Architecture and network centric AR architecture based on the DC architecture. This CR includes the UE centric architecture. Additions in this revision: Moved the architecture to a separate clause since it applies to both jUE centric and networ centric AR. Added text to introduce the new clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.X(new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * *

# AC.X Support of AR Communication

## AC.X.1 General

This clause  describes the enhancements to the IMS architecture supporting data channel services to additionally support  AR services. Two modes for AR services are supported; UE centric and network centric.

Editor’s Note: The definition of ‘AR media’ is not specified in this Release.

## AC.X.2 Architecture

Figure AC.X.2-1 shows the architecture to support UE Centric AR communication. This architecture supports both UE centric mode and network assisted AR mode. The AR related functionality in the architecture will be described when the modes are specified.

Figure AC.X.2-1: Architecture to support UE Centric AR communication

NOTE 1:	The AR Application Server is a specialized DC Application Server for AR and is not specified in this Release.

### AC.X.2.1 UE Centric AR

For the UE centric case, AR traffic is transparent to the IMS network and is exchanged between the two peer UEs via the deployed media functions (e.g. IMS AGW/TrGW).

Figure AC.X.2.1-1 shows the architecture to support UE Centric AR communication.

Figure AC.X.2.1-1: Architecture to support UE Centric AR communication

## AC.X.3 Procedures

### AC.X.3.1 UE Centric Procedure

Figure AC.X.3.1-1 depicts a typical call flow procedure to establish a UE centric AR IMS session from UE-A perspective. The main steps in the call flow are as follows:

1.	UE-A initiates an IMS communication with UE-B, including establishment of bootstrap data channel.

2.	The user of UE-A upgrades the IMS session with AR experience. 
UE-A initiates a re-INVITE adding media descriptors required by the AR application to be established E2E, e.g. RTP and/or application data channels. Application data channel may be anchored in the MF/MRF.

In this scenario, the UE performs AR media rendering based on the AR media from its local application.

3.	UE-A captures and encodes AR media.

4.	UE-A sends the AR media to the peer through the established media connection(s).

5.	UE-A receives AR media from the peer through established media connection(s).

6.	UE-A decodes and displays the received  AR media on its screen.

Figure AC.X.3.1-1: Establishment of UE Centric AR communication session

# * * * * End of changes * * * *