3GPP TSG-SA2 Meeting #157	S2-2306507

Berlin, May 22 – 26 2023	(merge of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4176 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for Multi-member AF Session Revoke |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, ETRI, Huawei (?), Ericsson (?),Samsung(?) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedures for supporting AF’s request for multiple members’ AF session for Create and Update have been defined. However, the Revoke procedures is missiong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the “Revoke” procedure to support Multi-session AF session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing procedure to support the AF request to remove the multiple members’ AF session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.13.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

##### 4.15.6.13.X	Procedure for Revoking a Multi-member AF session with required QoS for a set of UEs

Figure 4.15.6.13.2-1: Procedure for revoking a Multi-member AF session with required QoS for a list of UEs

1.	The AF sends a request to revoke the allocated resources for the traffic flows for the communication between a set of UEs and an AF, using Nnef_AFsessionWithQoS_Revoke request message (Transaction Reference ID).

2.	The NEF authorizes the AF request. If the authorisation is not granted, all steps (except step 8) are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

3-4.	The NEF refers to the list of UE addresses associated with the Transaction Reference ID and finds BSF serving the UE address(es) using NRF.   For each UE address, the NEF uses Nbsf_Management_Discovery service operation, providing the UE address, to discover the responsible PCF for each of the PDU Sessions.

Steps 5-7 apply for each UE address in the list of UE addresses.

5.	The NEF provides the UE address and the received parameters in step 1 to the PCF in the Npcf_PolicyAuthorization_Delete request as described in clause 5.2.5.3.4 excluding TSCTSF related info for the corresponding UE.

6.	The PCF requests SMF to proceed the SM Policy Association Termination procedure as defined in clause 4.16.6.

7.	The PCF sends the Npcf_PolicyAuthorization_Delete response message to the NEF.

8.	The NEF aggregates the authorization responses from the PCFs and sends a Nnef_AFsessionWithQoS_Revoke response message (Transaction Reference ID) with the aggregated authorization responses to the AF. Result for list of UE addresses includes whether the authorization was successful or has failed for every UE address in the list. The NEF stores the list of UE addresses for which the authorization was successful.

Steps 9-10 apply for each UE address in the list of UE addresses.

9 – 10. 	The NEF initiates the Npcf_PolicyAuthorization_Unsubscribe operations towards the respective PCF(s) for every UE in the list of UE address(es) associated with the Transaction Reference ID and PCF(s) confirm the the unsubscribe.

11.	The NEF sends Nnef_AFsessionWithQoS_Notify message with the event reported by the PCF to the AF to confirm the revocation of the Multi-member AF session.

* * * End of Changes * * *