3GPP TSG-WG SA2 Meeting #157	S2-2306566

# Berlin, ED, May 22-26, 2023		(revision of S2-2305749)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0322 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ranging and Sidelink Positioning MO-LR procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Qualcomm Incorporated], Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is concluded in FS_Ranging_SL study (TR 23.700-86) that the LMF may be used to support network assisted SL Positioning when one of the UEs involved in Ranging and Sidelink Positioning is in coverage. This CR provides the MO-LR based procedure for UE Positioning assisted by Sidelink Positioning and involving 5GC. This CR attempts to resolve ENs of the previous revision. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add MO-LR based procedure for UE Positioning assisted by Sidelink Positioning and involving 5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support on UE Positioning assisted by Sidelink Positioning and involving 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2; (new) 6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

APN	Access Point Name

APN-NI	APN Network Identifier

EDT	Early Data Transmission

E-SMLC	Evolved Serving Mobile Location Centre

GMLC	Gateway Mobile Location Centre

HGMLC	Home GMLC

LCS	LoCation Services

LDR	Location Deferred Request

LIR	Location Immediate Request

LMF	Location Management Function

LPI	LCS Privacy Indicator

LRF	Location Retrieval Function

MBSR	Mobile Base Station Relay

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

NI-LR	Network Induced Location Request

PMD	Pseudonym mediation device functionality

POI	Privacy Override Indicator

SDT	Small Data Transmission

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TWAP	Trusted WLAN Access Point

VGMLC	Visited GMLC

SL-MO-LR	Sidelink Mobile Originating Location Request

# * * * * next change * * * *

6.x	Network assisted Ranging/SL Positioning procedures

6.x.z	SL-MO-LR Procedure

Figure 6.x.z-1 illustrates a procedure to enable a UE  to obtain sidelink positioning/ranging location results using one or more other UEs with the assistance of an LMF in a serving PLMN for UE1.

This procedure applies to the following use cases of UE Positioning assisted by Sidelink Positioning and involving 5GC when the service request is initiated at the UE:

- Network based SL Positioning for UE with NAS connection, in which UE1 is the Target UE as defined in TS 23.586 [40] and UE2/UE3/ … /UEn is the Reference/Located UE as defined in TS 23.586 [40];

- Network based SL positioning for UE without NAS connection, in which UE1 is the Reference/Located UE as defined in TS 23.586 [40] and one of UE2/UE3/ … /UEn is the Target UE as defined in TS 23.586 [40];

- UE based SL Positioning for UE with NAS connection, in which UE1 is the Target UE as defined in TS 23.586 [40] and UE2, UE3, … UEn is the Reference/Located UEs as defined in TS 23.586 [40];

- UE based SL Positioning for UE without NAS connection, in which UE1 is the Reference/Located UE as defined in TS 23.586 [40] and one of UE2/UE3/ … /UEn is the Target UE as defined in TS 23.586 [40].

The Ranging/SL Positioning location results may include absolute locations, relative locations or distances and directions, depending on the service request.

Figure 6.x.z-1: SL-MO-LR Procedure

Precondition:

UE1 is in coverage and registered with a serving PLMN. UEs 2 to n may or may not be in coverage and, if in coverage, may or may not be registered with the same serving PLMN as UE1.

1.	The procedures and signalling specified in clause 6.2 of TS 23.586 [40] may be used to provision the Ranging/SL positioning service authorization and policy/parameter provisioning to UE1 and one or more of UEs 2 to n, when in coverage. If LMF supporting Ranging/SL positioning is not available, an indication of UE-only operation may be included as the policy/parameter provisioned to UE.

2.	UE1 determines whether SL-MO-LR or UE-only operation is to be performed based on Ranging/SL positioning service authorization and policy/parameter received at step 1.

3. If UE1 determines to perform UE-only operation at step 2, procedures of Ranging/Sidelink Positioning control as defined in clause 6.8 of TS 23.586 [40] is performed. All the rest steps are skipped.

4.	If UE1 determines to perform SL-MO-LR at step 2 and if UE1 is in CM-IDLE state, UE1 instigates a UE triggered Service Request in order to establish a signalling connection with the serving AMF of UE1.

5.	UE1 sends a supplementary services SL-MO-LR request to the serving AMF in an UL NAS TRANSPORT message. The SL-MO-LR request indicates the other UEs 2 to n (using application layer ID, e.g. GPSI), indicates any assistance data needed, indicates whether location calculation assistance is needed, indicates if a user plane connection between UE and LMF is established and indicates whether location results should be transferred to an LCS client or AF.  For transfer to an LCS Client or AF, details of the LCS Client or AF (and possibly of a GMLC) and application layer IDs for all UEs are also included. For location calculation assistance from the LMF, the preferred type of sidelink positioning/ranging location results (e.g. absolute locations, relative locations or distances and directions between pairs of UEs) and the required QoS are included.

Editor’s note: It is FFS whether indication about the need of assistance data from UE.

6.	The serving AMF selects an LMF serving UE1/2/…/n (e.g. an LMF that supports sidelink positioning/ranging) and sends an Nlmf_Location_DetermineLocation service operation towards the LMF with the information from the SL-MO-LR Request.

NOTE 1: If the serving AMFs of UE1/2/…/n are different, the serving AMF of UE1 informs the other AMF(s) the ID of the selected LMF, and the other AMF(s) selects the same LMF for UE1/2/…/n.

7. If UE1 requests to establish UP connection with LMF at step 5, UP connections are established between UE1 and LMF using the Positioning via a User Plane Connection between UE and LMF initiated by UE procedure as defined in clause 16.8.2. If UE1 doesn’t requests to establish UP connection with LMF at step 5, and LMF determines to establish UP connection with UE1, Positioning via a User Plane Connection between UE and LMF initiated by LMF procedure as defined in clause 16.8.1 is applied.

8.	The LMF sends a request to UE1 for the capabilities of all UEs 1 to n.

9.	UEs 1 to n returns its capabilities to the LMF. UE1 may return the capabilities for the UEs obtained at step 5 to the LMF.

10. UE1 may send a request for specific assistance data to the LMF.

11.	LMF sends the requested assistance data to UEs 1 to n. The assistance data may assist UEs 1 to n to obtain sidelink location measurements at step 13 and/or may assist UE1 to calculate sidelink positioning/ranging location results at step 14.

NOTE 1:	Steps 10 and 11 can be omitted if UE1 includes a message containing the capabilities of UEs 1 to n in the SL-MO-LR request at step 5. Step 12 can be omitted if UE1 includes a message containing the request for specific assistance data in the SL-MO-LR request at step 5.

Editor's note: Whether step 8-11 are needed will be aligned with RAN WGs.

12.	If the SL-MO-LR request at step 5 indicated location calculation assistance is needed and/or indicated transfer of sidelink positioning/ranging location results to an LCS Client or AF, the LMF sends a request for location information to UE1 and may also send a request for location information to UEs 2 to n. If LMF determines to apply UE based SL Positioning, LMF includes in the request the indication of UE based SL Positioning.

Editor's note:How the request for location information is sent to UE1/2/../n will be aligned with RAN WGs.

13.	UE1 instigates a sidelink positioning/ranging procedure as defined in clause 6.8 of TS 23.586 [40] among UEs 1 to n in which UEs 1 to n obtain sidelink location measurements and UEs 2 to n transfer their sidelink location measurements to UE 1 and/or to the LMF (depending on the assistance requested).

14. If UE1’s absolute location information is required at step 5 and if absolute locations of UE2…UEn are not available, UE2…UEn may perform 5GC-MO-LR procedure to acquire their absolute location.

15.	UE1 calculates sidelink positioning/ranging location results for itself and for each of UEs 2 to n based on the sidelink location measurements obtained at step 13 and possibly using assistance data received at step 11. The sidelink positioning/ranging location results can include absolute locations, relative locations or ranges and directions related to the UEs 1 to n.

16.	If UE1 received a request for location information at step 15, UE1 sends a response to the LMF and includes  the sidelink location measurements obtained at step 13, the sidelink positioning/ranging location results obtained at step 15 if step 15 was performed, or UE2/…/UEn absolute location obtained at step 14.

17. If UE1’s absolute location information is required at step 5 and if UE2/…/UEn absolute location is not received at step 16, LMF triggers 5GC-MT-LR procedure to the GMLC to acquire the absolute location of UE2/…/UEn.

18.	The LMF calculates sidelink positioning/ranging location results for UEs 1 to n from the sidelink location measurements received at step 16 and absolute location of UE2…UEn at step 17. The sidelink positioning/ranging location results can include absolute locations, relative locations or ranges and directions related to the UEs 1 to n.

19.	The LMF returns an Nlmf_Location_DetermineLocation service operation response to the AMF and includes any sidelink positioning/ranging location results received at step 16 or calculated at step 18.

204.	If sidelink positioning/ranging location results were received at step 10, the AMF performs steps 7-12 of clause 6.2 to send the sidelink positioning/ranging location results to the GMLC and to an AF or LCS Client if this was requested at step 5. The sidelink positioning/ranging location results include the global identities for UEs 1 to n received at step 5.

NOTE 2:	Sending location results and global identities for UEs 1 to n to an AF or LCS Client may require privacy verification from UEs 1 to n and/or from the HPLMNs of UEs 1 to n.

21.	The LMF returns a supplementary services SL-MO-LR response to UE1 in a DL NAS TRANSPORT message and includes any sidelink positioning/ranging location results calculated at step 18 if step 18 was performed. UE1 may then transfer any sidelink positioning/ranging location results to UEs 2 to n.

# * * * * End of changes * * * *