3GPP TSG-SA2 Meeting #157	S2-2306683

# 22 – 26 May 2023, Berlin, Germany												(revision of S2-230)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4609 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing default PDU-set importance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  |  | Date: |  | 2023-05-10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In case the UPF receives a PDU that does not belong to a PDU Set based on the N4 rule the UPF may include the PDU as a single PDU in a PDU Set. In that case, it is unclear how the UPF determines the PDU Set importance for the scenario where a PDU Set includes a single PDU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that when the PCF determines that an application session requires PDU set QoS handling the PCF, in addition to, PSDB, PSER and PSIHI determines also a default PDU set importance. The PCF provides the default PDU set importance within a PCC rule to the SMF. The SMF in turn includes a default PDU Set importance parameter in the N4 rule to the UPF. The UPF includes the default PDU Set importance in GTP-U for a PDU Set if the UPF determines that the PDU does not belong to a PDU Set already and marks it with PDU Set information. In addition, it is also proposed the AF to include a default PDU Set importance value to the PCF in case the AF is aware that the application session contains PDUs that do not match the protocol description of a service data flow. The information by the AF may be used by the PCF to determine appropriate PCC rules that includes the default PDU Set importance value provided by the AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How UPF handled PDU Set importance for PDUs that are not part of a PDU Set is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

************************************ START OF CHANGE *********************************

### 5.37.5	PDU Set based QoS Handling

#### 5.37.5.1	General

A PDU Set is comprised of one or more PDUs carrying an application layer payload such as, e.g. a video frame or video slice. The PDU Set based QoS handling by the NG-RAN is determined by PDU Set QoS parameters specified in clause 5.7.7 and PDU Set information provided by the PSA UPF as described in clause 5.37.5.2.

Editor's note:	The applicability and details of PDU Set handling in uplink direction is pending RAN WG's progress.

In addition to the PDU related service information, the AF may provide PDU Set related assistance information for dynamic PCC control. One or more of the following PDU Set related assistance information may be provided to the NEF/PCF using the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3].

-	PDU Set QoS parameters as described in clause 5.7.7

-	Protocol Description: Indicates protocol and payload type used by the service data flow.

-	Default PDU Set importance value, in case, the application session may contain PDUs that do not match the Protocol Description and payload type used by the service data flow.

NOTE X: The default PDU Set importance value depends on agreements between AF and PLMN network operator.

When the PCF determines that PDU Set based QoS Handling is to be performed for an application service flow, the PCF generates a PCC rule containing the PDU Set QoS parameters (PSER, PSDB and PSIHI) and the SMF determines a QoS Profile for the QoS Flow. The PCC rule may also contain a default PDU Set importance value if provided by the AF or based on internal decision by the PCF. The SMF includes the default PDU Set importance value in an N4 rule to the UPF.

PSA UPF identifies PDUs that belong to PDU Sets. If the UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, then the UPF includes this PDU as a single PDU in a PDU et. The PDU Set importance value for such PDU Set is set to the default PDU Set importance value provided by the SMF.

Editor's note:	Whether a standardized S-NSSAI (SST) is defined for XRM or whether non-standardized S-NSSAI is used is FFS.

************************************ END OF CHANGE ****************************************