3GPP TSG-SA2 Meeting #157	S2-2306687

# 22 - 26, May 2023, Berlin, Germany

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4610 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU Set QoS handling: non-homogenous support (Alternative 1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PDU set based QoS handling feature needs not be homogenously supported in the network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added description on non-homogenous support of PDU set based QoS handling in NG-RAN Clarified that PDU Set QoS handling is enabled by sending at least one of the PDU Set QoS Parameters to the NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SMF cannot know whether the PDU set based QoS handling is enabled in NG-RAN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.7.1, 5.37.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR .... |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 5.7.7.1	General

PDU Set QoS Parameters are used to support PDU Set based QoS handling in the NG-RAN. At least one among the PDU Set QoS Parameters shall be sent to the NG-RAN to enable PDU Set based QoS handling.

The followings are the PDU Set specific QoS characteristics:

1.	PDU Set Delay Budget (PSDB).

2.	PDU Set Error Rate (PSER).

3.	PDU Set Integrated Handling Information (PSIHI).

Editor's note:	Usage of PSIHI is FFS.

For a QoS Flow supporting PDU Set based QoS handling, the QoS Profile includes the PDU Set QoS Parameters described in this clause (see clause 5.7.1.2). The PCF determines the PDU Set QoS Parameters based on information provided by AF and/or local configuration. The PDU Set QoS parameters are sent to the SMF as part of PCC rule. The SMF sends them to NG-RAN as part of the QoS Profile.

If the NG-RAN receives PDU Set QoS Parameters and supports them, it applies PDU Set QoS Parameters as described in this clause.

Editor's note:	[XRM] The applicability and details of PDU Set handling in uplink direction is pending RAN WG's progress.

# >>>>NEXT CHANGE<<<<

#### 5.37.5.x	Non-homogenous support of PDU set based QoS handling in NG-RAN

The PDU set based QoS handling does not need to be homogeneously supported in NG-RAN. When the SMF serving the PDU session supports PDU set based QoS if a PCC rule requiring PDU set based QoS is active, the SMF provide the PDU Set QoS parameters to NG-RAN and enables the PDU Set based QoS handling for the downlink traffic in the UPF.

NOTE 1: 	If the receiving NG-RAN node does not support the PDU set based QoS handling then the NG-RAN node ignores the PDU Set QoS parameters.

When an existing QoS flow that did not initially support the PDU Set based QoS handling is modified to support it, the SMF instructs the PSA UPF to activate the PDU Set based QoS handling for the downlink traffic. In that case, the PSA UPF shall start marking PDU Sets over N3 from a complete PDU Set (i.e., it shall not mark PDU Sets for which some PDUs were already delivered previously unmarked).

NOTE 2:	In case of handover from a non-supporting NG-RAN node to a supporting NG-RAN node, the target NG-RAN node needs to temporarily handle both unmarked PDUs and marked PDUs within the same QoS flow.

NOTE 3:	In case of handover from a non-supporting NG-RAN node to a supporting NG-RAN node, the target NG-RAN node can be configured (e.g., based on the 5QI values associated to the QoS Flow) to provide a positive response to the source NG-RAN node in the handover preparation but to perform admission control after the QoS flow is modified, via PDU session modification, to support PDU Sets.

# >>>>END OF CHANGES<<<<