3GPP TSG-WG SA2 Meeting #157	S2-2306787

Berlin, Germany, May 22 – 26, 2023

Title:	Reply LS on IAB Authorization

Response to:	LS (S2-2306278/R3-231010) on IAB Authorization from RAN3

Release:	Rel-16

Work Item:	NR_IAB-Core, IABARC

Source:	SA2

To:	RAN3

Cc:	CT1

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.401 (R16 CR3728, R17 CR3729, R18 CR3730), TS 23.501 (R16 CR4388, R17 CR4389, R18 CR4390)

1. Overall Description:

SA2 would like to thank RAN3 for their LS on IAB Authorization (S2-2306278/R3-231010).

SA2 agreed the attached CRs that clarify the different options of AMF/MME behaviour when the IAB’s authorization is “not authorized”.

- Option 1: the AMF/MME rejects the IAB-UE’s registration/attach request or de-registers/detaches the IAB-UE.

- Option 2: the AMF/MME initiates UE Context setup/Modification procedure by providing IAB authorized indication with the value set to “not authorized” to the NG-RAN/E-UTRAN, but the IAB-UE is still registered.

2. Actions:

To RAN3 group.

ACTION: 	SA2 asks RAN3 group to take the above feedback into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #158		August 21 – 25, 2023			Goteborg, SE

TSG-SA2 Meeting #159		October 09 – 13, 2023			Xiamen, CN