3GPP TSG-WG SA2 Meeting #157 	S2-2306818

Berlin, Germany, May 22 – 26, 2023	(revision of S2-23xxxxx)

Source:	CATT

Title:	Resolve ENs on QoS handling

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes to resolve the ENs on QoS handling in TS 23.586.

1. Discussion

QoS handling of Ranging/SL Positioning is defined in clause 5.7 of TS 23.586, and it includes two aspects: Ranging/SL Positioning QoS and RSPP transport QoS.

The Ranging/SL Positioning QoS contains attributes defined in clause 4.1b of TS 23.273 with the some additions, and it is used for determination of Ranging method. Therefore, there is no need to provide the Ranging/SL Positioning QoS to NG-RAN.

For RSPP transport QoS, a new PQI value needs to be defined. Considering the Performance requirements in TS 22.261 (e.g. 10 ms), it is proposed to reuse PQI=55 for RSPP transport QoS.

Based on the above discussion, it is proposed to resolve the related ENs on QoS handling.

2. Text Proposal

It is proposed to capture the following changes into TS 23.586.

# * * * * First change* * * *

4.3.2	PCF

In addition to the functions defined in TS 23.501 [2] and TS 23.503 [4], the PCF includes functions to provision the UE with necessary policies and parameters to use Ranging/SL Positioning services, as part of the UE Ranging/SL Positioning Policy information as defined in TS 23.503 [4] clause 4.2.2:

-	Authorization policy and parameters for Ranging/SL Positioning over PC5;.

-	Authorization policy and parameters for Network assisted SL Positioning;

-	Authorization policy and Ranging/SL Positioning service exposure;

The PCF may update the Ranging/SL Positioning to the UE under certain conditions.

When receiving the Ranging/SL Positioning in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR, the PCF generates the PC5 QoS parameters used by NG-RAN corresponding to a UE as defined in clause 5.4.2 of TS 23.287 [6].

# * * * * Second change * * * *

4.3.3	AMF

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the AMF performs the following functions:

-	Select a PCF supporting Ranging/SL positioning Policy/Parameter provisioning based on indication of Ranging/SL positioning Capability as part of the 5G ProSe Capability/V2X capability in the "5GMM capability" in the Registration Request.

-	Store the Ranging/SL positioning Capability.

-	Select the LMF supporting Ranging/SL positioning and forward the Ranging/SL positioning Capability of Target UE to LMF.

-	Select a GMLC supporting Ranging/SL positioning and request the GMLC for Ranging/SL Positioning result.

-	Forward the Ranging/SL positioning Capability to PCF.

-	Obtain from UDM the subscription information related to Ranging/SL positioning and store them as part of the UE context data.

-	Provision the NG-RAN with indication about the UE authorization status about Ranging/SL Positioning over PC5 (i.e. as Ranging/SL positioning-enabled UE);

-	Provision the NG-RAN with PC5 QoS parameters related to Ranging/SL positioning over PC5.

# * * * * Third change* * * *

5.1.2	Authorisation and policy/parameter provisioning to NG-RAN

The "Ranging/Sidelink Positioning authorised" information and the RSPP transport QoS parameters shall be provided to the NG-RAN node for scheduled resource allocation mode resource management.

The "Ranging/Sidelink Positioning authorised" information includes one or more of the following:

-	whether the UE is authorized to act as a Reference UE;

-	whether the UE is authorized to act as a Target UE;

-	whether the UE is authorized to act as a Located UE;

-	whether the UE is authorized to act as a SL Positioning Server UE.

The RSPP transport QoS parameters include parameters defined in clause 5.7.3, which are generated by the PCF and are provided to the AMF when receiving the Ranging/Sidelink Positioning Capability in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR.

During registration, the UE includes the Ranging/Sidelink Positioning capability as defined in clause 4.3.1 as part of the "5GMM capability" in the Registration Request message. AMF determines whether the UE is authorised to use Ranging based services and sidelink positioning based on UE's Ranging/Sidelink Positioning Capability and the Ranging/Sidelink Positioning Service Authorisation included in the subscription data received from UDM. The AMF stores the Ranging/Sidelink Positioning capability in the UE context.

If the UE is authorised to use Ranging based services and sidelink positioning, the "Ranging/Sidelink Positioning authorised" information and the RSPP transport QoS parameters shall be provided from the AMF to the NG-RAN node in a NGAP message during Registration procedure, Service Request procedure and N2 Handover procedure, and from source NG-RAN node to target NG-RAN node over Xn during Xn Handover procedure as defined in clause 6.5 of TS 23.287 [6] and clause 6.6 of TS 23.304 [7].

# * * * * Fourth change* * * *

5.7.3	Handling of RSPP transport QoS

The V2X/ProSe layer handles the RSPP traffic as the V2X/ProSe application data packets for the QoS treatment. QoS handling mechanism of V2X as defined in TS 23.287 [6] or QoS handling mechanism of 5G ProSe as defined in TS 23.304 [7] is reused.

PQI value = 55 as defined in TS 23.287 [6] is reused for RSPP transport QoS.

# * * * * Fifth change* * * *

6.3.7	Delivery of PC5 QoS parameters related to Ranging/SL positioning to NG-RAN

The UE Policy Association Establishment procedure and UE Policy Association Modification procedure, as defined in TS 23.502 [5], is used to provide the AMF with PC5 QoS parameters related to Ranging/SL positioning used by NG-RAN. When receiving the Ranging/SL positioning service Capability in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR, the PCF generates the PC5 QoS parameters related to Ranging/SL positioning used by NG-RAN corresponding to a UE.

The (V-)PCF provides the information to the AMF as follows:

-	In the roaming case, the H-PCF includes the PC5 QoS parameters related to Ranging/SL positioning used by NG-RAN in the Npcf_UEPolicyControl_Create Response message or Npcf_UEPolicyControl UpdateNotify Request message sent to V-PCF in an N2 PC5 policy container, and V-PCF relays this N2 PC5 policy container as the N2 container in the Namf_Communication_N1N2MessageTransfer message sent to AMF.

-	In the non-roaming case, the PCF includes the PC5 QoS parameters related to Ranging/SL positioning used by NG-RAN in an N2 container in Namf_Communication_N1N2MessageTransfer message sent to AMF.

When the AMF receives the N2 PC5 policy container from (V-)PCF, the AMF stores such information as part of the UE context.

In the UE Configuration Update procedure triggered by UE Policy Association Establishment or UE Policy Association Modification:

-	The AMF forwards the PC5 QoS parameters related to Ranging/SL positioning in the NGAP message to the NG-RAN if a N2 PC5 policy container is received in the Namf_Communication_N1N2MessageTransfer message.

NOTE 1:	If the PC5 QoS parameters related to Ranging/SL positioning are provided to both NG-RAN and UE, both the N2 PC5 Policy Container and the UE Policy Container are included in the Namf_Communication_N1N2MessageTransfer message.

NOTE 2:	Non-UE specific PC5 QoS parameters, e.g. default PC5 QoS parameters, can also be locally configured in NG-RAN. How such configuration is performed is out of scope of this specification.

# * * * * End of changes* * * *