3GPP SA WG2 Meeting #157	S2-2306838

Berlin, Germany, May 22-26, 2023	(revision of S2-23xxxxx)

# Source:	Nokia, Nokia Shanghai Bell

# Title:	New SID on Support for Multiple Registrations in the 5G System

# Document for:	Information

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Support for Multiple Registrations in the 5G System

Acronym:	FS_MUREG

Unique identifier:	tbd

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  |  |  |  |
| Don't know | X |  | X |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| DualSteer | SA1 | tbd | WID on Upper layer traffic steering, switching and split over dual 3GPP access |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| xyz | New SID on Access Traffic Steering, Switching and Splitting support in the 5G system architecture; Phase 4 | Rel-19 Study Item |

# 3	Justification

"Simultaneous registration to 5GC via Multiple NR accesses" for a UE with single subscription is needed to support new use cases. In the past this was not supported or not possible as interference caused by the used radio bands was a showstopper. However, the situation has changed as 5G supports different bands from below 1 GHz up to mmWave. Some of the use cases that are enabled with "Multiple 3GPP access registrations" of a UE are explained below.

Use case 1: UE using different services offered by a PLMN and/or SNPN

Networks may support different services e.g., PLMN-A or SNPN-A support service-1 (data or Intranet access), PLMN-B supports service-2 (voice). A UE may require multiple service support (voice and data). In this case, the UE when connected to both PLMNs or to PLMN and SNPN can obtain access to the different services simultaneously.

Use case 2: Support for steering, splitting and switching of user data across two 3GPP accesses

The 3GPP SA WG1 is defining service requirements related to 5GS support of enhanced mechanisms for steering, splitting and switching of user data across two 3GPP networks (FS_DualSteer). There are various use cases documented in TR 22.841. One of the underlying assumptions for the work is that the UE is registered to the 3GPP network via more than one 3GPP access using a single subscription. This allows traffic aggregation on low and high NR bands or switch traffic from low band NR to high band NR.

Use case 3: FRMCS requirements

In the context of Future Railway Mobile Communication System (FRMCS) several scenarios are discussed where the train can be connected to two NR or one NR and one LTE access at the same time. This allows for traffic aggregation and increased resilience of the connections to the mission critical infrastructure (MCPTT, MCDATA, MCVOICE). The NR access can be provided by the railway network, which is a PLMN, while the second NR access or LTE access can be provided by a mobile network operating in the same country as the railway company or by a second railway company (e.g., when the train crosses borders). In addition, during stops in a train station the train may also be connected to local SNPN operated by the railway company or a 3rd party network provider.

# 4	Objective

Based on the above justification, following objectives will be studied:

1.	Study the architectural and procedural enhancements needed to support "Simultaneous registration to 5GC via Multiple NR accesses" for a UE. The UE can be registered to a single PLMN, to a single SNPN, to two different PLMNs or SNPNs, or to one PLMN and one SNPN. The UE may in addition be registered via one non-3GPP access to 5GC.

2.	Study how to support UE mobility between 5GS and EPS when 5GS supports "Simultaneous registration to 5GC via Multiple NR accesses" and EPS supports UE attach via only a single LTE access.

3.	Study the necessary enhancements to support mobile originating and terminating services (e.g., mobile data, voice, SMS, LCS) when the UE is registered to the 5GC network simultaneously via multiple NR accesses and potentially one non-3GPP access.

It is assumed that NR access refers to terrestrial or NTN access.

Within the scope of this study, it is also assumed that the UE has a single credential and a single subscription to register via the different 3GPP and non-3GPP accesses.

It is assumed that the UE can attach to EPC only via a single access path.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or is dependent on completion of other WTs |
| --- | --- | --- | --- | --- |
| 1. |  |  | Maybe | Self-contained |
| 2. |  |  | Maybe | Depends on 1. |
| 3. |  |  | No | Depends on 1. |

Total TU estimates for the study phase:

Total TU estimates for the normative phase:

Total TU estimates:

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xyz | Study on Support for Multiple 3GPP Access Registrations in the 5G System architecture | SA#xx | SA#yy | Gupta, Pallab, Nokia pallab.gupta@nokia.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Gupta, Pallab, Nokia, pallab.gupta@nokia.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Potential RAN impacts to be covered by RAN WGs.

Potential security impacts to be covered by SA3.

Potential charging and OAM impacts to be covered by SA5.

Stage 3 aspects covered by CT WGs.

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell