SA WG2 Meeting #SA2-157	S2-2306885

May 22 – 26 2023, Berlin, Germany

Title:	[Draft] Reply LS on carrier mapping for unicast SL CA

Response to:	LS on carrier mapping for unicast SL CA (S2-2306317/ R2-2304236)

Release:	Rel-18

Work Item:	NR_SL_enh2

Source:	SA2

To:	RAN2

Cc:	CT1

Contact Person:

Name:	Wen Wang

E-mail Address:	wen.wang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	xxxx

- Overall description

SA2 thanks RAN2 for their LS on carrier mapping for unicast SL CA (S2-2306321/ R2-2304506).

For the questions from RAN2, SA2 has discussed with following answers:

Question 1: According to TS 24.588, V2X layer is only provisioned with a mapping between service identifier and initial L2 address used for unicast. But service identifier is invisible to AS-layer, and the initial L2 ID will only be used in DCR and be replaced by a self-chosen L2 ID in PC5-S link establishment procedure. Then, after L2 ID changes, whether/how can UE's AS layer obtain the mapping between L2 ID and frequencies?

[SA2 Answer]: Current design is that the V2X layer passes the V2X frequencies associated with the V2X service identifier to the AS layer that is after the PC5 unicast link establishment, which can be referred to TS24.587 clause 6.1.2.12 as follows:

TS24.588: 6.1.2.12 PC5 QoS flow establishment over PC5 unicast link The UE shall also pass the one or more V2X frequencies associated with the V2X service identifier and the communication mode which is set to unicast mode for the V2X service identifier to the lower layers, if: a) the UE is configured with V2X service identifier to V2X frequency mapping rules for V2X communication over PC5 as specified in clause 5.2.3; and b) there is one or more V2X frequencies associated with the V2X service identifier in the current geographical area.

SA2 thinks that the V2X layer is aware of the changed L2 ID is associated to the initial L2 ID (default L2 ID) and passes the exact V2X frequencies to the AS layer.

SA2 would like to further state that the L2 ID changes also occurs after the PC5 unicast link establishment. In this case, V2X layer will provide the changed L2 ID to the AS layer, SA2 assumes that the AS layer maintains a mapping relationship between the V2X frequencies and the L2 ID, when the V2X layer provides the changed L2 ID to the AS layer, the old L2 ID will be replaced and the mapping relationship is updated accordingly.

Question 2: According to TS 24.587, PC5 unicast allows UEs to add/modify/remove V2X services/PC5 QoS flows to the same L2 ID pair. Then, given service info is invisible to AS layer, how can the UE ensure the modified V2X services to be transmitted only on the corresponding frequencies mapped in the V2X layer?

[SA2 answer]: PC5 unicast link modification procedure is used for the UEs to add/modify/remove V2X services/PC5 QoS flows to the same L2 ID pair. This procedure is achieved by the initiating UE requesting to modify the specific V2X services and the Target UE confirming which V2X services are accepted to be modified or not. It means that once this procedure is complete, both UEs will have the same understanding on the modified V2X service(s) that uses this PC5 unicast link and its corresponding frequencies to be transmitted.

2. Actions:

To CT1:

ACTION: 	SA2 kindly asks RAN2 to take the above feedback into account.

3. Date of Next SA2 Meetings:

3GPP TSG-SA2 Meeting #158	21-25 August 2023	Goteborg, SE

3GPP TSG-SA2 Meeting #159	09-13 October 2023	Xiamen, CN