3GPP TSG-SA2 Meeting #157	S2-2306915

# Berlin, Germany, May 22-26, 2023										           (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0833 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the ADRF service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| - Reason for change: |  | The Nadrf_MLModelManagement_ Retrieval service is defined, but it is not included in the ADRF service operations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adjust the general description of Analytics Data and ML Model Repository procedures - Add service operation of RetrievalRequest in Nadrf_MLModelManagement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ML Model retrieval from ADRF is not supported by ADRF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.1, 10.1, 10.3.1, 10.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Changes * * *

6.2B.1	General

Collected data and analytics may be stored in ADRF, using procedure as specified in clause 6.2B.2 and clause 6.2B.3. Collected data and analytics may be deleted from ADRF, using procedure as specified in clause 6.2B.4.

ML model may be stored in ADRF, using procedure as specified in clause 6.2B.5.

ML model may be deleted from ADRF, using procedure as specified in clause 6.2B.6. ML model(s) may be retrieved from ADRF, using procedure as specified in clause 6.2B.7.

* * *Second Changes * * *

10.1	General

Table 10.1-1 shows the ADRF services and ADRF service operations.

ADRF service operations may be used to store data or analytics in the ADRF, retrieve data or analytics from an ADRF, or delete data or analytics from an ADRF.

ADRF service operations may also be used to store ML model(s) or ML model address(es) in the ADRF, retrieve ML model(s) from an ADRF or delete ML model(s) from an ADRF.

Table 10.1-1: NF services provided by ADRF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadrf_DataManagement | StorageRequest | Request / Response | DCCF, NWDAF, MFAF |
|  | StorageSubscriptionRequest | Request / Response | DCCF, NWDAF |
|  | StorageSubscriptionRemoval | Request / Response | DCCF, NWDAF |
|  | RetrievalRequest | Request / Response | DCCF, NWDAF |
|  | RetrievalSubscribe | Subscribe / Notify | DCCF, NWDAF |
|  | RetrievalUnsubscribe |  | DCCF, NWDAF |
|  | RetrievalNotify |  | DCCF, NWDAF |
|  | Delete | Request / Response | DCCF, NWDAF |
| Nadrf_MLModelManagement | StorageRequest | Request / Response | NWDAF |
|  | RetrievalRequest | Request / Response | NWDAF |
|  | Delete | Request / Response | NWDAF |

* * *Third Changes * * *

10.3.1	General

Service Description: This service enables the consumer to store, retrieve and remove ML model(s) or ML model address(es) from an ADRF.

* * *Fourth Changes * * *

10.3.4	Nadrf_MLModelManagement_RetrievalRequest service operation

Service operation name: Nadrf_MLModelManagement_RetrievalRequest

Description: The consumer NF uses this service operation to request the ADRF to get the ML model(s) stored in ADRF.

Inputs, Required:

-	Analytics ID(s);

-	Storage Transaction Identifier; or

-	one or more tuples of unique ML Model identifier stored in ADRF.

Inputs, Optional: None.

Outputs Required:

-	address (e.g. URL or FQDN) of Model file stored in ADRF.

Outputs, Optional: Validity Period.

* * * End of Changes * * *