3GPP TSG-WG SA2 Meeting #157	S2-2306924

# Berlin , May 22 – 26, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0382 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of procedure of Cancellation of Reporting of Location Events with a User Plane Connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 23.273, as described in clause 6.12.2, for the procedure of cancellation of reporting of location events by an AF, an NF or External LCS Client or GMLC in clause 6.3.3,the user plane connection to the LCS Client or AF shall be released during the cancellation process if a user plane connection is used. As described in clause 5.10, the position messages can be routed to an established PDU Session or can trigger the establishment of a new PDU Session, to enable the release of user plane connection, the PDU session release/modification shall be added to the reporting cancellation procedures and performed according to whether the LCS Service use an established PDU Session or trigger the establishment of a new PDU Session. When the UE receives a location cancellation request from AMF, the resources may still be transferred via the user plane connection. To ensure the integrity of the resources transferred via the user plane connection, UE shall defer the request to release the user plane connection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a timer to the procedure of the cancellation of reporting of location events with a user plane connection. After receiving the Cancel Location request from AMF, UE initiates a deferred timer and release the user plane connection after the time expires instead of initiating a user plane connection release immediately. PDU session release/modification will be performed according to whether the LCS Service use an established PDU Session or trigger the establishment of a new PDU Session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When the UE receives a location cancellation request from AMF, how the user plane connection be released will be unclear. And as the resources may still be transferred via the user plane connection, UE release the user plane connection immediately will cause interruptions of the transfer of resources. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.16.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.16.2	Cancellation of Reporting of Location Events with a User Plane Connection

A UE may cancel a deferred 5GC-MT-LR procedure for periodic, or triggered location events where a user plane connection is used by the following the procedure defined in clause 6.3.2. After the UE receives the acknowledgment of the cancellation at step 6 in clause 6.3.2, the UE shall release the user plane connection to the LCS Client or AF if the LCS Client or AF has not yet initiated release of the user plane connection.

An AF or External LCS Client or a GMLC may cancel a deferred 5GC-MT-LR procedure for periodic or triggered location where a user plane connection is used by the following the procedure defined in clause 6.3.3. After the UE has returned the acknowledgment of the cancellation at step 9 in clause 6.3.3, the UE shall release the user plane connection to the LCS Client or AF if the LCS Client or AF has not yet initiated release of the user plane connection. After receiving the Cancel Location request from AMF in step 8, UE initiates a deferred timer and release the user plane connection after the time expires. If the LCS Service uses a dedicated PDU session, the UE requests to release the PDU session. If the LCS Service reuses the existing PDU session of another service, the UE requests to modify the PDU session.

# * * * * End of changes * * * *