3GPP TSG SA2 WG Meeting #157		S2-2307035

Berlin,Germnay, 22  - 26 May 2023

Title:	[Draft] LS on the Design of RTP Header Extension for PDU set handling

Response to:	S2-2306281/ S4-230419

Release:	Rel-18

Work Item:	XRM

Source:	[CATT] 3GPP SA2

To:	3GPP SA4

Cc:	3GPP RAN2

Contact person:	Chunshan Xiong

chushan.xiong@cictmobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall description

SA2 thanks SA4 for the liaison (S2-2306281/S4-230419) on the design of RTP Header Extension for PDU Set handling.

Regarding the questions raised by SA4, SA2 would like to provide following replies:

Q1: SA4 would like to ask if it is possible for a single QoS flow to include both PDUs marked with PDU set header extension and unmarked PDUs?

Answer:  SA2 agreed that a single QoS Flow includes either PDUs with GTP-U header extensions for PDU Sets or PDUs without such extension, but not both.

Q2: Does SA2 expect the application server to decide which PDUs belong to a data burst and signal related information in the RTP header extension?

Answer: SA2 thinks that the PSA UPF can correctly identify the End of data burst based on the RTP header information (e.g., the timestamp, sequence number and "M" bit). In a QoS Flow, all the RTP packets with the same RTP timestamp belong to the same data burst.SA2 kindly asked SA4 to provide guidelines on End of Data Burst Indication marked by the application server via RTP header information (e.g., the timestamp, sequence number and "M" bit) or define the 3GPP specific RTP header extension if the current RTP header cannot provide such information to determine End of Data Burst Indication.

2	Actions

To SA4:

ACTION: 	SA2 kindly asks SA4 to take the above into account and provide feedback if any.

3	Dates of next TSG SA WG2 meetings

TSG-SA2 Meeting #158		21-25 August 2023                     Goteborg, SE

TSG-SA2 Meeting #159		09-13 October 2023                    Xiamen,   CN