3GPP TSG-WG SA2 Meeting #157	S2-2307065

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4649 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clean Up of Hierarchical NSAC Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For hierarchical NSAC architecture the related service operations need be updated to add the concrete referred clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is replaced the “XYZ service” used in Clause 5.15.11.1.2 and 5.15.11.2.2 with the proper service operations and the corresponding referece clauses from TS23.502. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference service operation in Clauses 5.15.11.1.2 and 5.15.11.2.2 are not properly used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.1.2, 5.15.11.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause 4.2.11.X are define in 23.502 CR3930. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.15.11.1.2	Hierarchical NSAC architecture

In the hierarchal NSAC architecture, the NSACFs deployed in the service areas interacts with the primary NSACF when needed, and as explained below.

The main differences between the NSACFs deployed in a non-hierarchal architecture and NSACFs deployed in a hierarchal architecture is as follows:

-	When the AMF triggers an NSAC request to the NSACF, the AMF includes the UE already Registered indication if it can determine that the registered S-NSSAI has been registered in one service area before. The AMF determines the indication based on the received the Allowed NSSAI information from source AMF (in case of inter AMF handover) or from SMF+PGW-C (in case of mobility from EPS to 5GS)

-	There are two types of UE admission control: quota-based control or threshold-based control. A PLMN is configured to deploy only one type of UE admission control. Based on the type of UE admission control configured for the PLMN, the NSACF handles the NSAC request as described below:

-	For NSACFs supporting quota-based control, if upon receiving a request to increase the number of UEs and the local maximum number of UEs registered for a network slice has been reached, or upon receiving a request to decrease the number of UEs and no UE entry is in the NSACF, the NSACF interacts with the Primary NSACF for the handling of the NSAC request for the UE. The Primary NSACF may return in the response an updated local maximum number of registered UEs value to the NSACF based on the status of registered UEs for the network slice and which enables the NSACF to handle locally the request. Alternatively, and if the request includes the UE already Registered indication, the Primary NSACF may handle and store the UE entry to enable UE admission and allow for service continuity. The Primary NSACF may also reject the request.

-	For NSACF supporting threshold-based control, the NSACF is initially configured with the maximum number of Registered UEs to be admitted. If upon a receiving a request to increase the number of UEs and no UE already Registered indication and if UE admission threshold is at or above the threshold level configured at the NSACF, NSACF immediately rejects the NSAC request. If the received request includes the UE already Registered indication and if UE admission is at or above the threshold level configured at the NSACF, NSACF accepts the request to enable UE admission and allow for service continuity as long as the maximum number of Registered UEs have not been reached. If the local maximum number of registered UEs value have been reached, the NSACF interacts with the Primary NSACF for the handling of the NSAC request for the UE. The NSACF does not include the UE already Registered Indication in this case. The Primary NSACF may return an updated UE admission threshold value to the NSACF in the response which enables the NSACF to handle the request locally. Alternatively, the Primary NSACF may handle and store the UE entry. The Primary NSACF may also reject the request.

-	For both options, the Primary NSACF supports the following capabilities depending on the NSACF configuration:

-	Returning a new updated local maximum number of Registered UEs for the NSACF to admit if the NSACF is configured to support that feature; or

-	Returning a new updated UE admission threshold for the NSACF to apply if the NSACF is configured to support that feature;

-	The primary NSACF handles, stores entries only related to UEs which the NSAC request includes the UE already Registered indication, that are already admitted in an existing service area but cannot be admitted in the new service area due to no remaining local maximum number of Registered UEs, as long as the overall PLMN number of registered UEs at the Primary NSACF is not exhausted. The primary NSACF informs the NSACF in its response;

-	Based on the response from primary NSACF, the NSACF determines whether to accept or reject the NSAC request for UE registration. In addition, the NSACF may also update the local maximum number of Registered UEs or admission threshold respectively if the related updated value is received;

-	At any time, the Primary NSACF can update the NSACFs local maximum number of Registered UEs or admission threshold through the Nnsacf_NSAC_LocalNumberUpdate request/response service operation as described in clause 4.2.11.x of TS 23.502 [3].

-	The Primary NSACF subscribes to all the NACFs to obtain the number of registered UEs at all NSACFs.

# * * * * Next change * * * *

5.15.11.2.2	Hierarchical NSAC architecture

The main differences between the NSACFs deployed in a non-hierarchal architecture and NSACFs deployed in a hierarchal architecture is as follows:

-	The NSCAF is enhanced to support PDU session admission quota-based control.

-	When the local maximum number of PDU sessions is reached, the NSACF interacts with the primary NSACF to handle the request. The Primary NSACF either return an increased local maximum number to the NSACF, or reject the local maximum number value update request if all the global maximum number are consumed based on the status of established PDU sessions to the network slice.

-	Based on the response from primary NSACF, the NSACF updates the local maximum number if updated value is received from the primary NSACF. The NSACF updates local maximum number value (if received) and determines whether to accept or reject the NSAC request for PDU session establishment based on the local maximum number value.

-	The update of local maximum number value by the Primary NSACF can also happen at any time through the Nnsacf_NSAC_LocalNumberUpdate request/response service operation as described in clause 4.2.11.x of TS 23.502 [3].

-	The Primary NSACF subscribes to all the NACFs to obtain the number of established PDU sessions at all NSACFs.

# * * * * End of changes * * * *