3GPP SA WG2 Meeting #157	S2-2307125

22 - 26 May 2023, Berlin, Germany	(revision of S2-23xxxxx)

Title:	[DRAFT] Reply LS on the Design of RTP Header Extension for PDU set handling

Response to:	S4-230419 / S2-2306280

Release:	Release 18

Work Item:	5G_RTP

Source:	SA2

To:	SA4

Cc:	RAN2

Contact Person:

#### Name:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks SA4 for the information on the design of RTP header extension for PDU Set handling in their LS (S4-230419 / S2-2306280).

SA2 would like to provide the following answers to the SA4 questions:

Q1: SA4 would like to ask if it is possible for a single QoS flow to include both PDUs marked with PDU set header extension and unmarked PDUs?

In SA2’s understanding, the intent of Q1 is as follows: is it possible for a single QoS flow to include both PDUs marked with the new RTP header extension under definition in SA4 and PDUs that are not marked with the new RTP header extension under definition in SA4?

With this understanding the SA2’s answer is as follows: Mixing of PDUs that are marked with the new RTP header extension with PDUs that are not marked with the new RTP header extension in the same QoS Flow can happen as long as the information for QoS Flow matching (refer to the Packet Detection Information in the Packet Detection Rule in TS 23.501 Table 5.8.5.3-1) is the same for both.

Q2: Does SA2 expect the application server to decide which PDUs belong to a data burst and signal related information in the RTP header extension?

Response to Q2: As stated in a previous LS to SA4 (S2-2303849), for Rel-18 SA2 kindly asked SA4 to provide guidelines on a possible indication of the End of Data Burst by the application server using existing RTP header information (e.g., the timestamp, sequence number and "M" bit ) or define a 3GPP-specific RTP header extension if the current RTP header cannot provide such information.

2. Actions:

To SA4:

ACTION: 	SA2 kindly asks SA4 to take the information above into account.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#158			21 - 25 Aug 2023			Gothenburg, SE