SA WG2 Meeting #157	S2-2307178

# May 22-26, 2023, Berlin, DE

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4663 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Non-homogenous support of PDU set based QoS handling in NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Meta USA, CATT, Xiaomi,Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PDU set based QoS handling feature needs not be homogenously supported in the network. Before the SMF configures the PSA UPF with the PDU Set identification and marking for the QoS flow, the SMF needs to know whether the NG-RAN supports the PDU Set based QoS handling for the QoS flow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description on Non-homogenous support of PDU set based QoS handling in NG-RAN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SMF cannot know whether the PDU set based QoS handling is enabled in NG-RAN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR .... |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Start of Changes * * *

#### 5.37.5.x	Non-homogenous support of PDU set based QoS handling in NG-RAN

The PDU set based QoS handling does not need to be homogeneously supported in NG-RAN. Before the SMF configures the PSA UPF with the PDU Set identification and marking for the QoS flow, the SMF needs to know whether the NG-RAN supports the PDU Set based QoS handling for the QoS flow based on the N2 SM information - received from NG-RAN. Upon new QoS establishment and NG-RAN handover, the NG-RAN provides the SMF with information if it supports PDU Set based QoS handling.

# * * * End of Changes * * *