3GPP TSG SA WG-2 Meeting #157 	S2-2307192

Berlin, Germany, May 22 – 26, 2023

# Source:	Qualcomm Incorporated

# Title:	New SID on Enhancements in handling of Radio Capabilities

# Document for:	Approval

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Enhancements in handling of Radio Capabilities

Acronym:	FS_EHARC

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 840054 | Optimisations on UE radio capability signalling | Stage 2 for RACS in Rel-17 |

# 3	Justification

This study item is aiming to address the following distinct issues:

Issue #1: It is possible that certain UEs due to their subscription are only allowed in 5GS and not in LTE/EPS e.g. "CAG only" UEs. Nevertheless since the UE has radio capability to access LTE/EPS in certain cases it will attempt to do idle mode reselection to LTE and as a result be rejected. This wastes battery power for the UE and network resources for the network. It is therefore proposed that the network indicates to the UE due to its subscription it needs to disable its radio capabilities for specific RATs and as a result the UE does not need to perform inter-RAT measurements and mobility.

Issue #2: In certain deployments the viability of a particular application and the actions that an Application Function (AF) will take depend on the “alignment” of radio features deployed in network and UE. Network (PLMN and SNPN) may deploy a sub-set of the available radio features. UE operating in such network may or may not support the radio capabilities to operate such features. How an AF (internal or external to the network) know in advance before initiating a session for a particular application that there is “alignment” of radio features that would be required in order to support this application or the specific function is required.

Issue #3: When UE moves frequently between EPS and 5GS (in idle mode) a new retrieval of radio capability is needed from target system. This procedure would be optimised if like inside 5GS there is transfer of the radio capability container from AMF to MME and vice versa.

# 4	Objective

The aim of this study work is to investigate and identify potential architecture and system level enhancements to further evolve the handling of radio capabilities in 5GS and EPS. Specifically, the objectives include:

- WT#1: Identify how to prevent a UE to access a specific RAT/system in which it does not have subscription and therefore will be rejected. How to indicate to UE to disable certain radio capabilities in order to avoid attempting to register to this RAT/system and be rejected. How to define exceptions for emergency services.

- WT#2: How an AF (internal or external to the network) know in advance before initiating a session for a particular application that there is “alignment” of radio features that would be required in order to support this application or the specific function is required.

- WT#3: How to transfer the radio capability container or UE Radio Capability ID between AMF and MME (and vice versa) in order to avoid retrieval of radio capabilities from the UE (if possible) from the target system.

The work will be based on following deployment options: 1) RACS support in UE and NW, 2) No RACS in UE, but support of RACS in NW, 3) No RACS support.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 2 | 1 | No | N/A |
| WT#2 | 3 | 1 | Yes |  |
| WT#3 | 1 | 0.5 | No |  |

Total TU estimates for the study phase: 6

Total TU estimates for the normative phase: 2.5

Total TU estimates: N + M = 8.5

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xyz | Study on Enhancements in handling of Radio Capabilities | TSG#103 Mar. 2023 | TSG#104 Jun. 2024 |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

The following aspects involving other WGs may arise related to this SID:

-     Security aspects

-     RAN aspects

# 9	Supporting Individual Members

Supporting IM name

Qualcomm Incorporated