SA WG2 Meeting #157	S2-2307209

22 - 26 May, 2023, Berlin, Germany

# Source:	ZTE

# Title:	New SID on Edge Computing Phase 3

# Document for:	Information

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Edge Computing Phase 3

Acronym:	EDGE_Ph3

Unique identifier:	TBD

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x |  | x |  |
| No | x |  |  |  | x |
| Don't know |  |  | x |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| x | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification: None

3	Justification

In Edge Computing the SMF selects the DNAI and uses the DNAI to select the UPF for traffic offloading towards the Local Data Network. If multiple candidate DNAIs are available after considering the UE location, network topology and EAS deployment, the SMF selects one DNAI from the multiple ones based on operator's policy. For examples, the SMF can select the DNAI randomly, or based on selection weight factor if provided by AF, or select the DNAI closest to the UE location. Currently how to provide the weight factor by the AF has not been defined and needs to be further studied. In addition it is worthy of studying whether more information can be provided by AF to assist the SMF to select a proper DNAI.

When the vertical deploys the Edge Computing in Local Network, an AF needs to be deployed in Local Network and the AF provides information for traffic influence and/or EAS deployment information to operator network and the operator network can establish a data path for Edge Computing. However in reality the vertical may not be able to deploy an AF in the Local network. It is worthy of studying whether and how to establish the data path for Edge Computing without AF involvement.

When the vertical deploys the Edge Computing in Local Network, an UPF needs to be deployed in Local Network and this UPF needs to be controlled by the SMF in the operator network. This may add deployment complexity for Edge Computing since the operator network has to be impacted. It is worthy of studying whether and how to reduce the impact on the operator network when the vertical deploys Edge Computing.

4	Objective

The study item will study the potential system enhancements for edge computing support, including:

- Whether and how to provide more information including the weight factor of the DNAI from the AF to assist the SMF to select a proper DNAI.

- Whether and how to establish the data path for Edge Computing without AF involvement.

- Whether and how to reduce the impact on the operator network when the vertical deploys Edge Computing.

Existing solutions defined in Rel-15, Rel-16, Rel-17 and Rel-18 will be considered as baseline in this study.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or it depends on the completion of other WTs |
| --- | --- | --- | --- | --- |
| WT#1 |  |  | No | SC |
| WT#2 |  |  | No | SC |
| WT#3 |  |  | No | SC |

Total TU estimates for the study phase:

Total TU estimates for the normative phase:

Total TU estimates:

5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xxx | Study on Edge Computing Phase 3 |  |  |  |

6	Work item Rapporteur(s)

TBD

7	Work item leadership

SA2

8	Aspects that involve other WGs

The following aspects may arise related to this study:

- Security aspects are considered by SA3.

- Management and charging aspects are considered by SA5.

9	Supporting Individual Members

Supporting IM name