3GPP TSG-SA WG2 Meeting #157	 S2-2307246

# 22 - 26 May, 2023, Berlin, Germany

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4667 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve the EN on End of Data Burst Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It’s proposed to resolve the following EN on End of Data Burst Indication. Editor's note: Support of new RTP header extension for End of Data Burst indication by the application server depends on progress in SA WG4 5G_RTP WI. Based on S2-2306309, End of Data Burst indication can be provided in the RTP Header Extension. Therefore, it’s proposed to remove the EN and update text that UPF identifies the End of Data Burst indication in the RTP Header Extension and provides it to the NG-RAN over GTP-U. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the EN on End of Data Burst Indication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN is not addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.37.8.3	End of Data Burst Indication

An indication of End of Data Burst may be provided to the NG-RAN by the UPF, e.g. to configure UE power management schemes like connected mode DRX.

The PCF may provision the Protocol Description within PCC Rules based on the information provided by the AF and/or the local operator policies.

SMF should request the UPF to detect the last PDU of the data burst and the mark the End of Data burst in the GTP-U header of the last PDU in downlink, according to the PCC rule and/or the local operator policies.

According to the request and information from the SMF, the UPF identifies the last PDU of a Data burst in the DL traffic based on the End indication according to the Protocol Description and provides an End of Data Burst indication to the NG-RAN over GTP-U of the last PDU of a Data burst.

The UPF identifies the End of Data Burst indication in the RTP Header Extension and provides it to the NG-RAN over GTP-U.

NOTE:	There can be some packets from the Data Burst received by NG-RAN after the PDU with End of Data Burst Indication if packets are received out of sequence.

# * * * * End of change * * * *