3GPP TSG SA WG-2 Meeting #157 	S2-2307267

Berlin, Germany, May 22 – 26, 2023

# Source:	Qualcomm Incorporated

# Title:	New SID on Architecture Enhancements for Advanced Air Mobility in Cellular Environment

# Document for:	Approval

# Agenda Item:	xxx

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Architecture Enhancements for Advanced Air Mobility in Cellular Enviroment

Acronym:	AAM

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_UAV_Ph3 | SA1 | 960017 | Study on UAV Phase 3 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 980012 | Further Architecture Enhancement for UAV and UAM | XXXX |

# 3	Justification

Advanced Air Mobility (AAM) seeks to integrate Unmanned Aerial Systems (UAS) and other aerial vehicles (e.g. electric Vertical Take Off and Landing - eVTOLs) into the National Airspace System. AAM includes Urban Air Mobility (UAM), for which SA2 developed initial enablers in Release 17 and 18, and which involves transporting persons and cargo above the traffic within a city, with aerial mobility in wider areas (suburbs, villages and small towns in the countryside as well as islands or communities separated by mountainous regions and rivers). Enabling AAM is the next step in supporting the aviation vertical by 3GPP, in order to provide a complete set of tools for aerial vehicles and the UTM infrastructure to provide reliable and safe BVLOS operations.

ICAO, FAA, JARUS (Joint Authorities for Rulemaking on Unmanned Systems), and other regulatory bodies have been defining the concept of Command and Control Connectivity Service Provider (C2CSP) and associated requirements and expectations for providing and managing C2 connectivity for aerial vehicles. ACJA has published a “Landscape Whitepaper on UAS Cellular Ecosystem”(1) identifying the building blocks for the deployment of C2CSPs using cellular technologies. Identifying how to enable the C2CSP concept with 3GPP tools is the next steps in ensuring the UAS cellular ecosystem is completely defined based on 3GPP technologies.

In order to build on features developed for UAVs since Release 15 and to leverage the existing building blocks, SA1 in Release 19 is introducing a set of requirements to support the following:

-	enabling 5G network to provide further functionality for supporting UTM in flight mission planning, approval and flight monitoring to ensure minimization of risk of flight. This includes utilizing detailed location information to optimize the UAV fight path scheduling based on UAV tracking and propose flight path optimization to the UTM.

-	enabling UTM to plan and monitor the UAV connection to the network. SA1 has identified a set of tools developed by ACJA in “ACJA Interface for Data Exchange between MNOs and the UTM Ecosystem: Network Coverage Service Definition v1.00” (https://www.gsma.com/iot/wp-content/uploads/2021/02/ACJA-NetworkCoverage-Service-Definition-v1.00.pdf) for interfacing the UAV traffic management systems with the MNO network to collect additional data from connectivity providers. These tools match the QoE planning and monitoring defined in SA1 requirements but are not supported by network exposure mechanisms defined in 3GPP.

-	enabling MNOs to support the UTM in detection of UAVs without authorization.

-	enabling network-based mechanisms for DAA that can operate independently or in conjunction with traditional mechanisms of Detect and Avoid (DAA) between UAVs. Network-based DAA can leverage information gathered or generated by the 5GS (e.g. leveraging sensing capabilities) and provided to the UTM so that, e.g., collision warning can be notified by the UTM with network assistance.

# 4	Objective

The aim of this study work is to investigate and identify potential architecture and system level enhancements to further evolve the functionalities for the AAM aerial vehicles ecosystem. Specifically, the objectives include:

- WT#1: Enhance NEF services to support data services (e.g. service exposure) between MNOs and UTM functions to support intelligent pre-mission flight planning, in-mission flight monitoring, interfacing with UTM infrastructure (e.g. supporting multiple USS serving different geographical areas), and QoE estimation and planning

- WT#2: Enable network-assisted/ground-based DAA solutions that leverage information collected and generated in the 5GS (e.g. input from sensing and other potential "sensors" in RAN and UEs), and leverage service exposure and edge computing to provide aerial UEs and UTM with tactical deconfliction solutions​. The solution shall co-exist with and leverage, to the extent possible, Direct DAA solutions considered in Release 18.

- WT#3: study enhancements to multi-PLMN connectivity for connectivity redundancy to improve connection reliability based on SA1 requirements. The work will build upon ATSSS features in previous releases and leverage Release 19 work on ATSSS.​

- WT#4: Study support of the C2CSP model, considering existing 3GPP solutions and potentially extensions (e.g. IMS DC and IMS framework).​

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 1 | 0.5 | No |  |
| WT#2 | 1.5 | 1 | Maybe |  |
| WT#3 | 1 | 0.5 | Maybe |  |
| WT#4 | 1.5 | 1 | No |  |

Total TU estimates for the study phase: 5

Total TU estimates for the normative phase: 3

Total TU estimates: N + M = 8

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xyz | Study on architecture enhancements for Advanced Air Mobility in Cellular Environment | TSG#103 Mar. 2023 | TSG#104 Jun. 2024 | Faccin, Stefano, Qualcomm Incorporated, sfaccin@qti.qualcomm.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Faccin, Stefano, Qualcomm Incorporated,

sfaccin@qti.qualcomm.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

The following aspects involving other WGs may arise related to this SID:

-     Security aspects

-     RAN aspects

# 9	Supporting Individual Members

Supporting IM name

Qualcomm Incorporated