3GPP TSG-SA2 Meeting #157	S2-2307311

# Berlin, May 20 – 24, 2023									                                     (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0397 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of User Plane Logic of AMF and LMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | User Plane is an optional choice made by LMF, when there is user plane connection between LMF and UE, it may not be mandatory that AMF must choose the same LMF for successive LCS requests. For instance, the LMF may not be chosen due to other criteria with higher priority defined in clause 5.1 of TS 23.273. So, LMF can update the selection criteria in AMF based on user plane connection but it is not mandatory that AMF needs to follow LMF status update for LMF selection. When exchanging NAS/N1N2Transfer messages, LPP messages can also be transferred to reduce the latency. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The User Plane status is changed to LMF selection criteria of UP status to match the accurate AMF logic. - LPP message is allowed for the NAS/N1N2Transfer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The latency claimed by the user plane study can’t be fulfiled. AMF logic of LMF selection criteria is not consistent with UP connection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.1, 6.18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** 1st Change **********

### 6.18.1	LMF initiated User Plane Connection

LMF may trigger the user plane connection establishment after receiving a location request from AMF if target UE does not have user plane connection with LMF. AMF also subscribes from LMF for the selection criteria based on the status of LCS user plane connection for the target UE, using a Nlmf_Location_UPNotify Subscribe message if the UE supports user plane positioning with LMF. Figure 6.18.1-1 shows a procedure triggered by LMF to support positioning over the user plane connection between UE and LMF.

Figure 6.18.1-1: Positioning via a User Plane Connection between UE and LMF initiated by LMF

1.	Based on UE user plane positioning capability and other implementation factors, LMF decides whether to continue the position procedure via a user plane connection between UE and LMF.

Steps 2-7 are skipped if there is already a user plane connection context of the target UE in LMF and LMF determines to utilize the user plane connection for positioning.

NOTE:	LMF may depends on the UE capabilities stored in AMF and can select user plane positioning for specific positioning methods (e.g. motion sensor-based method) and it is based on implementation and local configuration to determine which positioning method requires user plane transport.

2.	[Conditional] If LMF decides to utilize user plane for positioning and there is no established secure user plane connection between the UE and LMF, LMF sends a user plane information to AMF to indicate UE to utilize user plane for positioning. The user plane information includes the user plane positioning address of the LMF, security related information. LMF may also include the positioning message(s) (e.g., LPP message) as payload.

Editor's note:	The security mechanism and security information provided by LMF to UE is subject to SA WG3's work. It is FFS how the UE user plane positioning capability and LMF user plane information is transferred, e.g. based on enhanced LPP, supplementary service or NAS message.

3.	[Conditional] When AMF receives the user plane information and optional positioning message(s) from LMF in step 2, AMF sends it to UE via a DL NAS TRANSPORT message.

4.	[Conditional] If there is no established applicable PDU session for the user plane positioning, the UE uses the URSP as defined in TS 23.503 [41] which includes user plane positioning related PDU session parameters, e.g. a dedicated DNN and S-NSSAI, to establish the PDU session for user plane positioning. UE may send an acknowledgement to LMF through AMF to indicate a success of utilization of a user plane connection for positioning service or a failure to utilize the user plane connection, e.g. no suitable PDU session established. When positioning message(s) is included in step 2 and 3, UE may also include the replied positioning message(s).

5.	[Conditional] AMF sends the acknowledgement received in step 4 to the LMF via Namf_N1messageNotify service. If the message from UE at step 4 has already fulfilled the positioning session, LMF replies the request from AMF. Then, LMF may skip the step 6~19; AMF may also continue to setup user plane connection without any positioning message.

6.	[Conditional] If LMF knows the UE IP address information, LMF may notify UE to setup the secure user plane connection through the known UE IP address.

Editor's note:	It is stage 3 to check and decide the protocol used by LMF to notify UE through the known UE IP address.

7.	[Conditional] UE establishes a secured user plane connection with LMF. If LMF send its FQDN to the UE, a DNS server/resolver is used to resolve the IP address of LMF (e.g. EASDF or local DNS for local LMF address resolution).

Editor's note:	IP address and/or FQDN as LMF address information needs to be checked/verified based on CT1/CT3 feedback.

8.	[Conditional] LMF indicates AMF in the Nlmf_Location_UPNotify message to update the LMF selection criteria that user plane connection between the UE and LMF has been established.

9.	[Conditional] The AMF stores the LCS-UP connection context as part of UE context.

10.	If LMF or UE determines to utilize the user plane connection for positioning and the secure user plane connection is established, LPP messages are transferred between UE and LMF for UE based positioning, UE assisted positioning and delivery of assistance data. Supplementary services event report messages from UE may also be transferred to LMF via the established user plane connection.

### 6.18.2	UE initiated User Plane Connection

UE may trigger the user plane connection establishment if the UE does not have user plane connection with LMF. Figure 6.18.2-1 shows a procedure triggered by UE to support positioning over the user plane connection between UE and LMF.

Figure 6.18.2-1: Positioning via a User Plane Connection between UE and LMF initiated by UE

1.	UE may send a user plane establishment request to AMF via NAS Message, if UE decides to prepare a user plane connection for upcoming positioning requests.

2.	[Conditional] If the UE is authorized based on UE Subscription to use the user plane positioning, AMF selects an LMF which capable to establish a user plane session for positioning with the UE.

3.	[Conditional] The AMF sends a Nlmf_Location_UPConfig Request towards LMF to request set up of an LCS-UP connection.

4.	[Conditional] If LMF accept to utilize user plane for positioning and there is no established secure user plane connection between the UE and LMF, LMF sends a user plane information to AMF to indicate UE to accept and utilize user plane for positioning. The user plane information includes the user plane positioning address of the LMF, security related information.

5.	[Conditional] When AMF receives the user plane information from LMF in step 4, AMF forwards it to UE via a DL NAS TRANSPORT message.

6.	[Conditional] UE establishes a secured user plane connection with LMF.

7.	[Conditional] LMF responds to AMF that user plane connection between the UE and LMF has been established.

8.	[Conditional] The AMF stores the LCS-UP connection context as part of  the criteria for LMF selection

9.	[Conditional] After the secure user plane connection is established, if LMF determines to utilize the user plane connection for positioning after receiving a positioning request from AMF, or UE determines to utilize the user plane connection for positioning, LPP messages are transferred between UE and LMF for UE based positioning, UE assisted positioning and delivery of assistance data. Supplementary services event report messages from UE may also be transferred to LMF via the established user plane connection.

********** End of Change ************