3GPP TSG-SA WG2 Meeting #157 	S2-2307465

MAY 22 – 26, 2023, Berlin, Germany	(was S2-2306889)

Source:	vivo

Title:	TS 23.586: Procedures for UE based SL Positioning for UE without NAS connection

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper tries to add Procedures for UE based SL Positioning for UE without NAS connection.

1. Introduction/Discussion

This paper adds the Procedures for UE based SL Positioning for UE without NAS connection based on the high level function descriptions in clause 5.5.5

### 5.5.5	UE based SL Positioning for UE without NAS connection

For UE based SL Positioning for UE without NAS connection, the procedure is similar to that of the Network based SL Positioning for UE without NAS connection (in clause 5.5.3), with the following exceptions:

-	The LMF may decide to choose a SL Positioning Server UE, e.g. Located UE, for result calculation, as described in clause 5.3.1.

-	The LMF may provide assistance information to the SL Positioning Server UE to assist the Sidelink Positioning result calculation.

Editor's note:	The contents of the assistance information will be defined based on RAN WG conclusions.

Taking this paper as baseline with merging S2-2307391

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

### 6.5.4	Procedures for UE based SL Positioning for UE without NAS connection

The Procedures for UE based SL Positioning for UE without NAS connection take the Procedures for Network based SL positioning for UE without NAS connection in clause 6.5.2 as baseline with following adaptations based on the functionalities defined in clause 5.5.5:

-	In Step 8: UE1(Located UE) may indicate that the assistance data or location calculation assistance is needed.

-	The LMF determines the UE based calculation taking the obtained capability of UE in step 10 and step 11 into account.

-	The LMF provides the assistance data to UE1(Located UE) in step 12~13, and requests the UE1(Located UE) to return the result to the LMF if the UE1(Located UE) is the SL location server UE enabled in step14.

-	If the UE1(Located UE) is not SL server UE enabled, the LMF may select a SL Positioning Server UE to calculate the result, in this case the LMF provides the assistant data and the location of the UE1(Located UE) to the selected SL Positioning Server UE in step 12-13, and ask for returning the calculation result in step14.

-	The UE1(Located UE) or a selected SL location server UE retunes the calculation result(i.e., the Target UE’s absolute location) to the LMF in step18.

# * * * * End of changes * * * *