3GPP TSG-WG SA2 Meeting #157 	S2-2307520

Berlin, Germany, May 22 – 26, 2023	(revision of S2-2307475)

Source:	CATT

Title:	ID alignment between UE Discovery and LMF operation

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes to use GMLC (instead of 5G DDNMF) for Application Layer ID to SUPI resolution in TS 23.586.

1. Discussion

It was agreed 5G DDNMF is not involved in UE discovery at the last SA2 meeting, but it was wrongly implemented in TS 23.586 v0.3.0 and therefore should be removed.

During the SA#157 drafting session, it is proposed that GMLC will be used for Application Layer ID to SUPI resolution, and detail solution is TBD.

2. Text Proposal

It is proposed to capture the following changes into TS 23.586.

# * * * * First change* * * *

4.2.1	Non-roaming reference architecture

Figure 4.2.1-1 shows the high-level view of the non-roaming 5G System architecture for Ranging based services and Sidelink Positioning with service-based interfaces within the Control Plane.

Figure 4.2.1-1 Reference architecture for Ranging based services and sidelink positioning for non-roaming and same PLMN operation in SBI representation

Figure 4.2.1-2 Non-roaming reference architecture for Location Services with ranging and sidelink positioning in reference point representation

NOTE: The full 5GS architecture is defined in TS 23.501[2].

4.2.2	Roaming reference architecture

Figure 4.2.2-1 shows the high-level view of the roaming 5G System architecture for Ranging based services and Sidelink Positioning with service-based interfaces within the Control Plane. In this figure, UE A use a subscription of the HPLMN.

Figure 4.2.2-1 Reference architecture for Ranging based services and sidelink positioning for roaming and same PLMN operation

4.2.3	Inter-PLMN reference architecture

Figure 4.2.3-1 shows a reference architecture for Ranging based services and Sidelink Positioning for Inter-PLMN non-roaming operation. In this case, the UE A and UE B have subscriptions from different PLMNs, i.e. PLMN-A and PLMN-B respectively.

Figure 4.2.3-1 Reference architecture for Ranging based services and sidelink positioning for inter-PLMN operations

Figure 4.2.3-2 shows a reference architecture for Ranging based services and Sidelink Positioning for Inter-PLMN roaming operation. In this case, the UE A and UE B have subscriptions from different PLMNs, i.e. PLMN-A and PLMN-B respectively. UE A is roaming and registered to PLMN-C.

Figure 4.2.3-1 Reference architecture for Sidelink Positioning and Ranging-based services for inter-PLMN operation with roaming

# * * * * Second change * * * *

4.2.5	Service-based interfaces

Nlmf:	In addition to the relevant services defined in TS 23.273 [8], if the LMF support Ranging/SL Positioning service, it may be used to provide service to other NFs related to it.

Nudm:	In addition to the relevant services defined in TS 23.501 [2] for Nudm, in the case of Ranging/SL Positioning service, services provided by UDM are used to get the related subscription information to AMF during Initial registration procedure or UE Configuration Update (UCU) procedure to inform AMF subscription information has changed.

Npcf:	In addition to the relevant services defined in TS 23.501 [2] for Npcf, in the case of Ranging/SL Positioning service, services provided by H-PCF are used to provide SL Positioning and Ranging Service related parameters to V-PCF for UE and NG-RAN in the roaming case.

Nudr:	In addition to the relevant services defined in TS 23.501 [2] for Nudr, in the case of Ranging/SL Positioning service, services provided by UDR are used to notify the PCF and the UDM of the update of the SL Positioning and Ranging-based service related information.

Namf:	In addition to the relevant services defined in TS 23.501 [2] for Namf, in the case of Ranging/SL Positioning service, services provided by AMF are consumed by PCF to provide the Ranging/SL Positioning service related parameters for the UE and the NG-RAN to AMF, and to enable the AMF create or update UE context related to Ranging/SL Positioning service.

Nnef:    In addition to the relevant services defined in TS 23.501 [2] for Nnef, in the case of Ranging/SL Positioning service, services provided by NEF are used by the Application Server to update Ranging/SL Positioning service related information of 5GC.

Nnrf:    In addition to the relevant services defined in TS 23.501 [2] for Nnrf, in the case of Ranging/SL Positioning service, services provided by NRF are used to discover the PCF that supports Ranging/SL Positioning.

# * * * * Third change* * * *

4.3.5	UDR

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the UDR performs the following functions:

-	Stores Ranging/SL Positioning service parameters.

-	Stores the mapping between Application Layer ID and GPSI provisioned by AF.

# * * * * Fourth change* * * *

4.3.8	LMF

In addition to the functions defined in TS 23.273 [8], the LMF supports the following:

-	The Network assisted SL Positioning;

-	Trigger Ranging/Sidelink positioning.

-	Exchange the Ranging/Sidelink Positioning capability.

-	Exchange the Ranging/Sidelink Positioning assistant data.

-	Exchange Ranging/Sidelink positioning signal measurement data/result.

-	Support of receiving stored Ranging/Sidelink Positioning capability from AMF and support of providing updated Ranging/Sidelink Positioning capability to AMF.

-	Determine the location of the Target UE based on the ranging/SL positioning measurement data or result reported by Target UE.

-	Interaction with GMLC to get the location of Located UE/Reference UE using the Application Layer ID.

-	Delivering the Ranging/Sidelink positioning service request/response for the Ranging service exposure to UE.

-

# * * * * Fifth change* * * *

4.3.9	GMLC

In addition to the functions defined in TS 23.273 [8], the GMLC supports the following:

-	enable trusted AFs and NFs to perform MT-LR by accessing GMLC directly with additional service parameters for Ranging/SL Positioning.

-	enable AFs and NFs to perform MT-LR by accessing GMLC through NEF with additional service parameters for Ranging/SL Positioning.

-	determine the serving AMF instances for the UEs involved in the Ranging/SL Positioning request and forward the request to respective serving AMF

-	receive response from AMF and event reports from and return Ranging/SL Positioning result to NFs and AFs.

-	performs Application Layer ID to SUPI resolution.

# * * * * Seventh change* * * *

4.3.10	AF

In addition to the functions defined in TS 23.501 [2], the AF supports the following:

-	provisions mapping between Application Layer ID and GPSI to UDR.

# * * * * End of changes* * * *