3GPP TSG-WG SA2 Meeting #156-e	S2-2307690

# Elbonia, April 17 – 21, 2023		(revision of S2-2307302)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0970 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3 address the ENs about restructuring and the scenario without TSCTSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Address the following ENs: Editor’s note: It is FFS whether this should be captured in TS 23.503 instead. - Keep the description about PCF in 503, but keep the description about TSCTSF in 501 Editor’s note: Further description of the scenario without TSCTSF is FFS - Option 1: Follow the TR conclusion, and clarify that the PCF maintains the status of the AF request information for a group, e. g. whether the group member is activated, de-activated, failed, whether to retry in case of failure of GBR QoS, AF request change due to group membership changes. - Option 2: Mandate to use the TSCTSF for AF request, TSCTSF is the entity to store the AF request information for the group and maintains the status of the AF request information. - Option 3: whehter to involve TSCTSF is determined based on operator configuration To unify the 5GC handling logic, it is proposed to use option 1 as the way forward. Editor’s note: Further description is FFS, to be done if this option is selected. - Remove As agreed in SA2#157 the EN: “API will be merged with the common one (Nnef_AFsessionWithQoS) at the next meeting” added in SA2#156e can be removed. QoS API introduced in GMEC will remain. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Move the support of Provisioning of traffic characteristics and monitoring of performance characteristics for a group from 23.501 into 23.503 to avoid duplicated description, keep TSCTSF logic in 23.501. 2. add a NOTE to clarify that the existing QoS Parameter Notification Control and existing AQP can be used formonitoring of specific performance characteristic. 3. Remove EN: “API will be merged with the common one (Nnef_AFsessionWithQoS) at the next meeting” added in SA2#156e. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unsolved issues remain |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.28 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.1.3.28	AF requested QoS for a UE or group of UEs not identified by a UE address

The AF may request that the data session(s) for a UE (identified by a GPSI) or a group of UEs (identified by an External Group ID) for a specific DNN and S-NSSAI are set up with a specific QoS (e.g. low latency or jitter) and priority handling. The request from the AF may also include parameters for QoS monitoring and parameters that describe the traffic characteristics.

According to the QoS parameters for AF session with QoS as described in clause 6.1.3.22,

the parameters that describe the traffic characteristics for AF session with QoS as described in clause 6.1.3.23a, and the parameters to

subscribe to be notified of the QoS Monitoring reports as described in clause 6.1.3.21, the AF request information may include:

-	DNN and S-NSSAI.

- 	Information about target UEs: External Group Identifier or GPSI.

-	Flow Descriptions as described in clause 6.1.3.6,

NOTE 1:	At the time of providing Flow Description from AF, the AF can only provide the server side information as Flow Description, e.g. 3-tuple(s) including protocol, server side IP address and port number. At the time of applying the Flow Description for generation of PCC rules, the PCF can supplement the UE side information, e.g. UE IP address and port.

-	Traffic characteristics as described in clause 6.1.3.23 or 6.1.3.23a,

-	QoS parameters (e.g. QoS Reference or individual QoS parameters or Alternative QoS Parameter Set) as defined in clause 6.1.3.22,

-	QoS parameters for monitoring as defined in clause 6.1.3.21,

NOTE 2:	The AF can support monitoring of specific performance characteristic by requesting the Notification control with Alternative QoS Profiles as defined in clause 5.7.2.4.1b of TS 23.501 [2] and deriving the specific degraded QoS characteristic (e.g. GFBR, PDB or PER) and the way it is degraded with the "GFBR can no longer be guaranteed" notifications and indications to Alternative QoS Profile.

-	Temporal invalidity condition (start-time, end-time), indicate the time period when there will be no user payload for the DNN/S-NSSAI and Flow Descriptions provided with the request (e.g. at night or on weekends),

-	Subscription to events as defined in clause 6.1.3.18.

The NEF determines whether or not to invoke the TSCTSF in the same way as for AF session with required QoS procedure, as described in step 2 of clause 4.15.6.6 in TS 23.502 [3].

In case the TSCTSF is not used, the NEF stores the AF request on UDR. The PCF receives the AF requested QoS information from the UDR as described in clause 4.15.6.14 of TS 23.502 [3]. If the AF requested QoS information contains temporal invalidity condition, the PCF activates, modifies, or removes PCC rules corresponding to the QoS information as needed based on the invalidity conditions.

In the case that the TSCTSF is used, the TSCTSF receives the AF requested QoS information from the NEF. The TSCTSF applies the AF requested QoS information as described in clause 5.20c of TS 23.501 [2] and clause 4.15.6.14 of TS 23.502 [3].

# * * * * End of changes * * * *