3GPP TSG SA WG2 Meeting #157	S2-2308089

# Berlin, DE, May 22nd – 26th, 2023	        (merge of S2-2306494 and S2-2307416 and S2-2306432)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3952 | rev | 7 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3 – 5GS to EPS mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Intel, Samsung, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Complete the new function to support URSP Provisioning in EPS after 5GS to EPS mobility and at initial attach in EPS, based on conclusions for KI#3 of FS_eUEPO. Change from rev. 1 to rev.2. Rev 2 addresses three ENs: - Editor’s Note: Whether enhancements in other procedures (e.g. UE Policy Association termination in EPC) are needed is FFS. - Editor’s Note: Home Routed roaming scenario is FFS. - Editor´s Note: It is FFS how the V-PCF for the PDU session determines the UE supports the provisioning of URSP rules in EPC. And in addition: - it clarifies the SMF+PGW-C provides a UE Policy Container within Npcf_SMPolicyControl_update by including a new PCRT (UE Policy) - How the UE reports the result of a URSP update in EPS is undefined - In LBO roaming It is undefined how the V-PCF for a PDU session discovers the V-PCF for the UE. - For 5GS to EPS mobility in LBO roaming it is undefined how the V-PCF for a UE discovers the H-PCF - Alignment with approved CR S2-2304027 in SA2#156. - Alignment with approved CR S2-2305477 in SA2#156. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the enhancement to the existing EPS procedures in new sub-clauses. Capture the enhancement to 5GS procedure in new sub-clause. Updates on existing Npcf_SMPolicyControl and Npcf_UEPolicyControl services - Regarding Editor’s Note: Editor’s Note: Whether enhancements in other procedures (e.g. UE Policy Association termination in EPC) are needed is FFS. Enhancements are included for “UE Policy association initiated by the PCF for the PDU sessionPDU Session” and “UE Policy association termination initiated by the PCF for the PDU session” - Regarding Editor’s Note: Editor´s Note: It is FFS how the V-PCF for the PDU session determines the UE supports the provisioning of URSP rules in EPC. During 5GS to EPS in LBO roaming the V-PCF for the PDU session has no access to UDR and it is not able to determine whether the UE supports provisioning of URSP rules in EPC. In this case it determines based on local configuration whether to establish a UE Policy association towards the V-PCF for a UE. If a UE Policy association is tried to be established for a UE that doesn’t support the delivery of URSP in EPS, when the V-PCF for a UE tries to establish the UE Policy association towards the H-PCF for a UE, this will reject the UE Policy association establishment. - Regarding Editor’s Note: Editor’s Note: Home Routed roaming scenario is FFS. The procedures have been updated including support for the Home Routed roaming scenario. - The SMF+PGW-C includes new PCRT “UE Policy” when forwarding a UE Policy Container towards the PCF for the PDU session. - In clause 4.11.0a.2a.0 (general) it is clarified that in EPS the PCF for a PDU session may establish a UE policy association towards the PCF for the PDU during either initial attach with default PDN connection establishment procedure or during the first request for PDN connectivity in EPS or during 5GS to EPS mobility. Also clarified the support for URSP delivery in EPS is supported for LBO and roaming scenarios. - At EPS to 5GS the PCF for the PDU session terminates the UE Policy association for EPS, as described in S2-2305477. - It is described the procedure for the UE to report the result of a URSP update in EPS. - It is clarified URSP delivery in EPS is also supported in both HR and LBO roaming scenarios. - It is clarified how the V-PCF for a PDU session discovers the V-PCF for a UE in LBO roaming. - For 5GS to EPS mobility in LBO roaming the V-PCF for a UE discovers the H-PCF for a UE by quering the BSF in the H-PLMN. - For Home Routed roaming, the V-PCF for a UE is not involved, therefore in this roaming scenario it is not possible to retrieve the Application guidance on URSP Rule for inbound roamers of the PLMN and provide it to the H-PCF for the UE to be used as input for the generation of the URSP rules for the UE as described in clause 6.6.2.2.3 of TS 23.503. - In alignment with approved S2-2304027 CR in SA2#156, the PCF for the UE doesn’t need remove the former 5GS UE Policy association when the EPS UE Policy association is established by the PCF for the PDU session at 5GS to EPS mobility. - Some other editorial changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete function of URSP delivery in EPS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a2, 4.11.0a.2a (new), 4.11.0a.2a.0 (new), 4.11.0a.2a.2 (new), 4.11.0a.2a.2.3 (new), 4.11.0a.2a.3 (new), 4.11.0a.2a.5 (new), 4.11.0a.2a.6 (new), 4.11.0a.2a.7(new), 4.11.0a.2a.8(new), 4.11.0a.2a.9(new), ,4.11.5.X (new), 4.11.5.X.1 (new), 5.2.5.4, 5.2.5.4.1, 5.2.5.4.3, 5.2.5.4.5 and 5.2.5.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * *First change * * * *

#### 4.11.0a.2	Interaction with PCC

When interworking with 5GS is supported and a "SMF+PGW-C" is selected for a PDN connection, policy interactions between PDN GW and PCRF specified in TS 23.401 [13] are replaced by equivalent interactions between SMF+PGW-C and PCF as follows:

-	IP-CAN Session Establishment procedure defined in TS 23.203 [24] is replaced by SM Policy Association Establishment Procedure as described in clause 4.16.4.The SMF+PGW-C includes the information elements received in Create Session Request message into the Npcf_SMPolicyControl_Create Service as follows: the SUPI contains the IMSI, the DNN contains the APN, the PEI contains either the IMEISV or IMEI, the Session AMBR contains the APN-AMBR and the default QoS information that contains the default EPS bearer QoS, note that QCI values are mapped into 5QI values. The SMF+PGW-C may receive PCC Rules and PDU Session Policy Information, 5G QoS information in the PCC Rule and in PDU Session Policy Information are mapped into EPS QoS information as defined in clause 4.11.1.1 and Annex C.

-	(PCEF-initiated) IP-CAN Session Modification procedure defined in TS 23.203 [24] is replaced by SM Policy Association Modification procedure as described in clause 4.16.5.1.

-	The SMF+PGW-C includes the information elements received in Modify Bearer Request or Modify Bearer Command message into the Npcf_SMPolicyControl_Update Service with the following modifications, the subscribed Session AMBR includes the subscribed APN-AMBR and subscribed default QoS information includes the default EPS bearer QoS, note that QCI values are mapped into 5QI values. The SMF+PGW-C includes the stored SUPI. The SMF+PGW-C may receive PCC Rules and PDU Session Policy Information, 5G QoS information in the PCC Rule and in PDU Session Policy Information are mapped into EPS QoS information as defined in clause 4.11.1.1 and Annex C.

-	The SMF+PGW-C includes the information elements received in Delete Bearer Command message into the Npcf_SMPolicyControl_Update Service with the following modifications, The SMF+PGW-C includes the stored SUPI.

-	(PCRF-initiated) IP-CAN Session Modification procedure defined in TS 23.203 [24] is replaced by SM Policy Association Modification procedure as described in clause 4.16.5.2. The SMF+PGW-C may receive PCC Rules and PDU Session Policy Information, 5G QoS information in the PCC Rule and in PDU Session Policy Information are mapped into EPS QoS information as defined in clause 4.11.1.1 and Annex C.

-	IP-CAN Session Termination procedure defined in TS 23.203 [24] is replaced by SM Policy Association Termination procedure as described in clause 4.16.6. The SMF+PGW-C includes the information elements received in Delete Session Request message by the SMF+PGW-C into the Npcf_SMPolicyControl_Delete Service. The SMF+PGW-C includes the stored SUPI.

URSP provisioning may be supported in EPS and the procedures are specified in clause 4.11.0a.2a.

# * * * * Next Change * * * *

#### 4.11.0a.2a		Interaction with PCC for URSP delivery via EPS

##### 4.11.0a.2a.0	General

To Support URSP provisioning in EPS, the SMF+PGW-C receives the Indication of URSP Provisioning Support in EPS PCO and ePCO support indication from the UE, if the SMF+PGW-C also supports URSP Provisioning Support in EPS and ePCO, it selects a PCF which also supports URSP Provisioning in EPS to establish the SM Policy Association based on the received Indication of URSP Provisioning Support in EPS PCO and local configuration.

When the SMF+PGW-C receives the UE Policy Container ePCO in Bearer Resource Command during UE requested bearer resource modification procedure, it forwards transparently the UE Policy Container to PCF for the PDU Session in the Npcf_SMPolicyControl_Update Request including indication that "UE Policy Container received" PCRT was met. When the PCF for Session Management receives UE Policy Container from PCF for the UE, it forwards the UE Policy Container to SMF+PGW-C in Npcf_SMPolicyControl_UpdateNotify Request. The PCF for the PDU Session retrieves the PCRTs for UE Policy from PCF for the UE and subscribe to the applicable PCRTs in EPC to SMF+PGW-C.

When URSP Provisioning is supported in EPS, the PCF for the PDU Session may establish the UE Policy Association with PCF for the UE:

- During EPS initial attach with default PDN connection establishment procedure or during the first request for PDN connectivity in EPS for a UE when the first UE Policy Container is received from the UE for an SM Policy association and forwards the UE Policy Container to PCF for the UE in Npcf_UEPolicyControl_Create Request.

- During 5GS to EPS mobility with N26 when the SMF+PGW-C notifies the PCF for the PDU Session about a RAT-Type change via Npcf_SMPolicyControl_Update.

The PCF for the UE generates the URSP and sends it to the PCF for the PDU Session in the UE Policy Container via Npcf_UEPolicyControl_UpdateNotify Request. The PCF for the UE triggers the re-evaluation of applicable URSPs and determines an update of URSP is needed for the UE when an event as described in clause 6.1.2.2.3 of TS 23.503 [20] occurs in the PCF for the PDU Session.

The procedures for the UE Policy association defined in clauses 4.16.11, 4.16.12 and 4.16.13 apply in general in EPS with the main differences below:

- AMF is replaced by the PCF for the PDU Session.

- Delivery of UE Policy Containers from the PCF for the UE to the PCF for the PDU Session is done via Npcf_UEPolicyControl service instead of via UE Configuration Update procedure.

URSP delivery via EPS is supported in roaming scenarios, both for a PDN connection established in Home Routed and for a PDN connection established in LBO.

This clause captures the enhancement to the interaction with PCC in clause 4.11.0a.2 to support URSP delivery via EPS.

# * * * * Next Change * * * *

##### 4.11.0a.2a.2	SMF initiated SM Policy Association Modification

###### 4.11.0a.2a.2.1 	PDN connection establishment in EPS

The following enhancements are applicable to clause 4.16.5.1 (SM Policy Association Modification procedure):

In addition to step 4 in clause 4.16.5.1 if the PCF receives both the indication that the PCRT "UE Policy Container received" was met and the UE Policy Container, steps 4a to 4d below are executed; otherwise step 5 in clause 4.16.5.1 is executed.

-	Step 4a: The PCF determines that the SM Policy Association is used to provision URSP rules to the UE by checking the UE Policy Container received from SMF+PGW-C in Npcf_SMPolicyControl_Update Request.

NOTE 1: If the PCF for the PDU Session is the same than the PCF for the UE the interactions in steps 4b-4d are internal to PCF.

-	Step 4b: The PCF for the PDU Session discovers the PCF for the UE by querying the BSF and using the SUPI received in Npcf_SMPolicyControl_Create Request as described. If no result is obtained from the BSF the PCF for the PDU Session selects the PCF for the UE by querying NRF.

- 	Step 4c:  The PCF for the PDU Session establishes a UE Policy Association towards the PCF for the UE as described in clause 4.11.0a.2a.5.

- 	Step 4d: If the PCF for the UE provides any Policy Control Request Trigger parameters in the Npcf_UEPolicyControl Create Response the PCF for the PDU Session takes them into account for the generation of Policy Control Request Triggers to the SMF+PGW-C.

###### 4.11.0a.2a.2.2 	5GS to EPS mobility

During 5GS to EPS mobility procedure, the PCF for the PDU Session determines whether to establish a UE Policy Association towards the PCF for the UE for the purpose of delivery of URSP in EPS. During EPS to 5GS mobility procedure the PCF for the PDU Session terminates the UE policy association if established during 5GS to EPS mobility.

The following enhancements are applicable to clause 4.16.5.1 (SMF initiated SM Policy Association Modification procedure):

In addition to step 4 in clause 4.16.5.1, the PCF determines that 5GS to EPS or EPS to 5GS mobility applies by checking a change in RAT and Access-Type. If 5GS to EPS mobility applies steps 4a to 4d below are executed; if EPS to 5GS applies the PCF for the PDU Session checks if a UE Policy association was created during 5GS to EPS mobility and in that case, it requests the termination of the UE Policy association as described in clause 4.11.0a.2a.8; otherwise step 5 in clause 4.16.5.1 is executed.

-	Step 4a: For non-roaming and Home Routed roaming the (H-)PCF for the PDU Session determines whether the UE supports provisioning of URSP Rules in EPC by checking the indication that is stored within the UE context Policy Subscription data in UDR. If the UE supports provisioning of URSP Rules in EPC the flow continues in step 4b.

For LBO roaming the V-PCF for the PDU Session has no access to UDR and it is not able to determine whether the UE supports provisioning of URSP rules in EPC. In this case it determines based on local configuration whether to establish a UE Policy association towards the V-PCF for the UE.

NOTE: If the PCF for the PDU Session is the same than the PCF for the UE the interactions in steps 4b-4d are internals to PCF.

-	Step 4b: For the non-roaming and Home Routed roaming the (H-)PCF for the PDU Session discovers the PCF for the UE by querying the BSF using the SUPI of the UE as described in 23.501 clause 6.3.7.4.

For LBO roaming the V-PCF for the PDU Session discovers and selects an instance of the V-PCF for the UE based on the available PCF instances obtained from the NRF or locally configured information in the V-PCF, depending on operator's policies, as described in 23.501 clause 6.3.7.1.

- 	Step 4c:  The PCF for the PDU Session establishes a UE Policy Association towards the PCF for the UE as described in 4.11.0a.2a.5. This request includes an indication about the trigger for the UE Policy Association Establishment ("5GS to EPS Mobility").

-	Step 4d: If the PCF for the UE provides any Policy Control Request Trigger parameters in the Npcf_UEPolicyControl Create Response the PCF for the PDU Session takes them into account for the generation of Policy Control Request Triggers to the SMF+PGW-C in Npcf_SMPolicyControl_Update answer.

###### 4.11.0a.2a.2.3 	UE reports URSP update

When the UE reports the result of a URSP update the following enhancements are applicable to clause 4.16.5.1 (SM Policy Association Modification procedure):

In addition to step 4 in clause 4.16.5.1 if the PCF for the PDU Session receives the "UE Policy" Policy Control Request Trigger and associated UE Policy Container, steps 4a to 4b below are executed; otherwise step 5 in clause 4.16.5.1 is executed.

-	Step 4a: The PCF for the PDU Session determines that a UE Policy association has been already established towards the PCF for the UE for the SM Policy Association as described in clause 4.11.0a.2a.2.1.

NOTE: If the PCF for the PDU Session is the same than the PCF for the UE the interactions in steps 4b is internal to PCF.

- 	Step 4b:  The PCF for the PDU Session forwards the UE Policy Container to the PCF for the UE in Npcf_UEPolicyControl_Update Request.

# * * * * Next Change * * * *

##### 4.11.0a.2a.3	PCF initiated SM Policy Association Modification

For the purpose of URSP delivery in EPC a new condition to initiate the SM Policy Association Modification procedure is added triggered by an interaction from the PCF for the UE. This interaction applies when the PCF for the PDU Session has previously established a UE Policy Association towards the PCF for the UE for the delivery of URSP Rules in EPS as defined in clauses 4.11.0a.5 and 4.11.0a.2a.2.

The following enhancements are applicable to clause 4.16.5.2 (PCF initiated SM Policy Association Modification procedure):

NOTE: If the PCF for the PDU Session is the same than the PCF for the UE the interactions in steps below are internals to PCF.

-	Step 1e: Alternatively, the PCF for the UE provides a UE Policy Container and/or an update in the Policy Control Request Triggers applicable to the UE by invoking Npcf_UEPolicyControl_UpdateNotify as described in clause 4.16.12.2 steps 3-4.

-	Step 4: If the PCF for the PDU Session receives a UE Policy Container from the PCF for the UE the PCF for the PDU Session first selects one of the ongoing PDU Sessions for the related UE in EPC and includes that UE Policy Container in Npcf_SMPolicyControl_update notify request for that PDN connection.

If the PCF for the PDU Session receives an update in the Policy Control Request Triggers applicable to the UE the PCF for the PDU Session determines whether an update on the current Policy Control Request Triggers needs to be sent to the SMF+PGW-C in Npcf_SMPolicyControl_UpdateNotify request.

# * * * * Next Change * * * *

##### 4.11.0a.2a.5	UE Policy Association establishment

To support the delivery of URSP in EPC as described in 23.503 clause 6.1.2.2.X 	(URSP Provisioning in EPS) the PCF for the PDU Session may establish a UE Policy Association towards the PCF for the UE.

This procedure may be triggered by the following procedures:

1.	Initial Attach to EPC.

2.	PDN Connectivity Request during the first request for PDN connectivity.

3.	5GS to EPS handover or 5GS to EPS Idle Mode mobility.

In the non-roaming case, the V-PCF for the UE is not involved, and the PCF for the PDU Session interacts with the PCF for the UE. For the Home Routed roaming scenarios the V-PCF for the UE is not involved and the H-PCF for the PDU Session interacts with the H-PCF for the UE. For the LBO roaming scenarios, the V-PCF for the PDU Session interacts with the V-PCF for the UE and the V-PCF for the UE interacts with the H-PCF for the UE:

The procedure is based on the one defined in clause 4.16.11 for a UE in 5GC with the following differences:

The PCF for the PDU Session determines the need of establishment of a UE Policy Association when:

- A UE Policy Container is received from the UE during SM Policy Association Update at Initial Attach with default PDN connection establishment to EPC or at first PDN Connectivity Request as described in clause 4.11.0a.2a.2.1, or

- During 5GS to EPS mobility as described in clause 4.11.0a.2a.2.2

2.	Same than in clause 4.16.11 step 2 with the following exceptions:

The PCF for the PDU Session includes the UE Policy Container in case of SM Policy Association update or 5GS to EPS Mobility indication in case of SM Policy Association update at 5GS to EPS mobility.
For LBO roaming the V-PCF for the PDU Session discovers and selects an instance of the V-PCF for the UE based on the available PCF instances obtained from the NRF or locally configured information in the V-PCF, depending on operator's policies, as described in 23.501 clause 6.3.7.1. The V-PCF for the PDU Session contacts the V-PCF for the UE and steps 3 and 4 are executed. 
For non-roaming or Home Routed roaming the (H-)PCF for the PDU Session contacts the (H-)PCF for the UE and step5 follows.

3.	Same than in clause 4.16.11 step 3 with the following exceptions:

The UE Policy Container is received at SM Policy Association instead of initial registration.When the UE Policy Association is being established due to 5GS to EPS mobility as described in clause 4.11.0a.2a.2.2, the V-PCF for a UE in the VPLMN needs to discover and select the same H-PCF for a UE that is serving the UE in the H-PLMN for 5GS by quering the BSF in the HPLMN. This is to be able to recover the information about the list of PSIs in the UE and the subscribed PCRTs from former UE Policy Association for the UE in 5GS as described in step 6. Therefore, the V-PCF for a UE in the VPLMN needs first to discover the BSF in the HPLMN.NOTE 1: For Home Routed roaming, the V-PCF for a UE is not involved, therefore in this roaming scenario it is not possible to retrieve the Application guidance on URSP Rule for inbound roamers of the PLMN and provide it to the H-PCF for the UE to be used as input for the generation of the URSP rules for the UE as described in clause 6.6.2.2.3 of TS 23.503.

4.	Same than in 4.16.11.

5.	Same than in clause 4.16.11 step 5 with the following exceptions:

The subscription to notification of N1 message delivery of policy information to the UE using Namf_Communication_N1N2MessageSubscribe service is not applicable.

6.	The Same than in clause 4.16.11 step 6 with the following exceptions:

If "5GS to EPS Mobility indication was received in Npcf_UEPolicyControl_Create in step 2 instead, the (H-)PCF recovers the information about the list of PSIs in the UE,the subscribed PCRTs in 5GS from former UE Policy Association for the UE and the indication about the support of provisioning of URSP rules in EPC. If the UE doesn’t support the provisioning of URSPs in EPS, the H-PCF rejects the establishment of the UE Policy Association.

NOTE 2: In LBO roaming the V-PCF for a UE may trigger the establishment of the UE Policy Association for a UE that doesn’t support the provisioning of URSP in EPS, as described in clause 4.11.0a.2a.2.2 step 4a.

The (H-)PCF provides the UE Policy Container in the Npcf_UEPolicyControl UpdateNotify Request.

7.	Same than in 4.16.11.

NOTE 3: The steps 8 and 9 are executed only in LBO roaming.

8.	The V-PCF for the UE forwards the UE Policy Container to the V-PCF for the PDU Session by invoking Npcf_UEPolicyControl UpdateNotify Request. The V-PCF for the UE checks the size limit as described in clause 6.1.2.2.2 of TS 23.503 [20].

9.	The V-PCF for the PDU Session sends a response to the V-PCF for the UE.

10.	If the UE provides a UE Policy Container with the result of the URSP update to the PCF for the PDU Session as described in section 4.11.0a.2a.2.3 the  PCF for the PDU Session forwards the UE Policy Container to the (V-) PCF for the UE invoking Npcf_UEPolicyControl_Update Request.

11.	The (V-) PCF for the UE sends a response to the (V-) PCF for the PDU Session.

NOTE 4: The steps 12 and 13 are executed only in LBO roaming.12. The V-PCF for the UE forwards the notification response to the H-PCF by invoking Npcf_UEPolicyControl_Update Request.

13. The H-PCF sends a response to the V-PCF for the UE.

# * * * * Next Change * * * *

##### 4.11.0a.2a.6	UE Policy Association Modification initiated by the PCF for the UE

The following impacts are applicable to clause 4.16.12.2 (UE Policy Association Modification initiated by the PCF procedure):

In the non-roaming case, the V-PCF is not involved, the AMF is replaced by the PCF for the PDU Session, and the role of the H-PCF is performed by the PCF for the UE.

For the Home Routed roaming scenarios, the V-PCF is not involved, the AMF is replaced by the H-PCF for the PDU Session, and the role of the H-PCF is performed by the H-PCF for the UE.
For the LBO roaming scenarios, the AMF is replaced by the V-PCF for the PDU Session, the role of the V-PCF is performed by the V-PCF for the UE and the role of the H-PCF is performed by the H-PCF for the UE.

-	Steps 1a, 1c, 2a, 2c and 2d: These steps are not applicable for Home Routed roaming, since the V-PCF is not involved

-	Step 3:  The (H-)PCF may create the UE policy container including UE policy information as defined in clause 6.1.2.2.2 of TS 23.503 [20]. The (H-) PCF checks whether the UE Policy Association for the UE is established for a UE in EPC by checking the RAT-Type of the UE Policy Association and in this case the (H-)PCF may send the UE Policy Container in the Npcf_UEPolicyControl UpdateNotify Request. The (H-)PCF may provide updated PCRTs the UE policy association.

For Home Routed roaming, the V-PCF for a UE is not involved, therefore the Service Parameters provided by the AF from the VPLMN are not used by the H-PCF as input for the generation of the URSP rules.

NOTE 1: The steps 5 and 6 are executed only in LBO roaming.

-	Step 5: In case the UE Policy Association is for a UE in EPC, the (V-) PCF for the UE provides the UE Policy Container and/or policy control triggers to the (V-) PCF for the PDU Session.

- Step 6: The (V-) PCF for the PDU session sends a response to the (V-) PCF for the UE.

Steps 7, 8 and 9 are replaced by steps 10-13 of procedure UE Policy Association Establishment in EPC from clause 4.11.0a.2a.5.

# * * * * Next Change * * * *

##### 4.11.0a.2a.7	UE Policy Association Modification initiated by the PCF for a PDU Session

This procedure addresses the scenario where a Policy Control Request Trigger condition is met by the PCF for a PDU Session.

The procedure is based on the one defined in clause 4.16.12.1.1 with the following differences:

In the non-roaming case, the V-PCF is not involved, the AMF is replaced by the PCF for the PDU Session, and the role of the H-PCF is performed by the PCF for the UE.

For the Home Routed roaming scenarios, the V-PCF is not involved, the AMF is replaced by the H-PCF for the PDU Session, and the role of the H-PCF is performed by the H-PCF for the UE.

For the LBO roaming scenarios, the AMF is replaced by the V-PCF for the PDU Session, the role of the V-PCF is performed by the V-PCF for the UE and the role of the H-PCF is performed by the H-PCF for the UE.

- Step 1: Same than in clause 4.16.12.1.1 step 1 with the following exceptions:

In LBO roaming the V-PCF for the PDU Session contacts the V-PCF for the UE and steps 2 and 3 are executed.  The V-PCF for the UE contacts with the H-PCF for the UE discovered during the establishment of the UE Policy association as described in clause 4.11.0a.2a.5.

For non-roaming or Home Routed roaming the (H-)PCF for the PDU Session contacts the (H-)PCF for the UE and step 4 follows.

- Steps 2, 3,4: Same than in clause 4.16.12.1.1.

- Step 5: The (H-)PCF may create the UE policy container including UE policy information as defined in clause 6.6 of TS 23.503 [20]. The (H-) PCF checks whether the UE Policy Association for the UE is established for a UE in EPC by checking the RAT-Type of the UE Policy Association and in this case the (H-)PCF may send the UE Policy Container in the Npcf_UEPolicyControl UpdateNotify Request.

- Step 6: Same than in clause 4.16.12.1.1.

- Steps 7, 8 and 9 are replaced by steps 10-13 of procedure UE Policy Association Establishment in EPC from clause 4.11.0a.2a.5.

# * * * * Next Change * * * *

##### 4.11.0a.2a.8	UE Policy Association Termination initiated by PCF for PDU session

The following case is considered for UE Policy Association Termination:

1.	UE Detach from the EPS.

2.	PDN connection for UE which is used for delivery of UE Policy Container in EPS is released.

3.	Changing of RAT type from EPS to 5GS, notified by SMF+P-GW-C.

Figure 4.11.0a.2a.8-1: UE Policy Association termination in EPS

This procedure applies for non-roaming scenarios, LBO roaming scenarios and Home Routed roaming scenario.

In the non-roaming case, the V-PCF for the UE is not involved, the role of the V-PCF for the PDU session is performed by the PCF for the PDU session and the role of the H-PCF for the UE is performed by the PCF for the UE. For the LBO roaming scenarios, the V-PCF for the PDU session interacts with the V-PCF for the UE and the V-PCF for the UE interacts with the H-PCF for the UE.

In the Home Routed roaming scenario, the V-PCF for the UE is not involved, the role of the V-PCF for the PDU session is performed by the PCF for the PDU session in HPLMN and the role of the H-PCF for the UE is performed by the PCF for the UE in HPLMN.

1.	The (V-)PCF for the PDU session terminates a UE Policy Association with the (V-)PCF for the UE when:

- The SMF+P-GW-C notifies the changing of RAT type from EPS to 5GS, or

- The SMF+P-GW-C terminates the SM policy association with (V-)PCF for the PDU session due to UE detached from EPS or the PDN connection for UE which is used for delivery of UE Policy Container in EPS is released.

2.	Same as in clause 4.16.13.1 step 2, replace the AMF to (V-)PCF for the PDU session.

3.	Same as in clause 4.16.13.1 step 3.

4.	Same as in clause 4.16.13.1 step 4.

5.	Same as in clause 4.16.13.1 step 5.

##### 4.11.0a.2a.9	UE Policy Association termination initiated by the PCF for a PDU Session

The procedure is based on the one defined in clause 4.16.13.2 (UE Policy Association Termination initiated by the PCF procedure) with the following differences:

The following cases are considered for UE Policy Association Termination:

1.	PDN Connection termination.

2.	EPS to 5GS mobility with N26.

This procedure applies for non-roaming scenarios, LBO roaming scenarios and Home Routed roaming scenario.

In the non-roaming case, the V-PCF is not involved, the AMF is replaced by the PCF for the PDU Session, and the role of the H-PCF is performed by the PCF for the UE.
For the LBO roaming scenarios, the AMF is replaced by the V-PCF for the PDU Session, the role of the V-PCF is performed by the V-PCF for the UE.

In the Home Routed roaming scenario, the V-PCF is not involved. The AMF is replaced by the PCF for PDU Session in HPLMN, and the role of the H-PCF for the UE is performed by the PCF for the UE in HPLMN.

NOTE 1: For both non-roaming and LBO roaming, If the (V-)PCF for the PDU Session is the same than the (V-) PCF for the UE, all the interactions between both happens internally in the (V-)PCF.

When the UE is in EPC the UE Policy Association has been established by the (V-) PCF for PDU Session as described in clause 4.11.0a.2a.6.

-	Step 4: Change the AMF to the PCF for PDU Session. The PCF for UE may notify the the PCF for PDU Session of the removal of the UE Policy Association via Npcf_UEPolicyControl_UpdateNotify service operation.

-	Step 5: Change the AMF to the PCF for PDU Session.

-	Step 6: Change AMF to the PCF for PDU Session and refer to the step 2 to step 5 in clause 4.11.0a.2a.8.

# * * * * Next Change * * * *

#### 4.11.5.x	UE Policy Association control

##### 4.11.5.x.1	UE Policy Association Establishment for URSP delivery in EPS

The following impacts are applicable to clause 4.16.11 (UE Policy Association Establishment procedure) during initial registration or during UE registration with 5GS when the UE moves from EPS to 5GS:

In addition to step 3 in clause 4.16.11, if the indication about the support of provisioning of URSP rules in EPC is received in step 2 within Npcf_UEPolicyControl_Create Request, that indication is added in the list of parameters to be stored in UDR using Nudr_DM_Create.

# * * * * Next Change * * * *

#### 5.2.5.4	Npcf_SMPolicyControl service

##### 5.2.5.4.1	General

Service description: NF Service Consumer, e.g. SMF, can create and manage a SM Policy Association in the PCF through which the NF Service Consumer receives policy information for a PDU Session.

As part of this service, the PCF may provide the NF Service Consumer, e.g. SMF, with policy information for the PDU Session that may contain:

-	PDU Session related policy information as defined in clause 6.4 of TS 23.503 [20].

-	PCC rule information as defined in clause 6.3 of TS 23.503 [20].

-	Policy Control Request Trigger information i.e. a set of Policy Control Request Trigger(s) as defined in clause 6.1.3.5 of TS 23.503 [20].

At PDU Session establishment the NF Service Consumer, e.g. SMF, requests the creation of a corresponding SM Policy Association with the PCF (Npcf_SMPolicyControl_Create) and provides relevant parameters about the PDU Session to the PCF.

When the PCF has created the SM Policy Association, the PCF may provide policy information for the PDU Session in the response.

When a Policy Control Request Trigger condition is met the NF Service Consumer, e.g. SMF requests the update(Npcf_SMPolicyControl_Update) of the SM Policy Association by providing information on the condition(s) that have been met as defined in clause 6.1.3.5 of TS 23.503 [20]. The PCF may provide updated policy information for the PDU Session to the NF Service Consumer in the response.

The PCF may at any time provide updated policy information for the  (Npcf_SMPolicyControl_UpdateNotify).

At PDU Session Release the NF Service Consumer, e.g. SMF requests the deletion of the corresponding SM Policy Association (Npcf_SMPolicyControl_Delete).

# * * * * Next Change * * * *

##### 5.2.5.4.3	Npcf_SMPolicyControl_UpdateNotify service operation

Service operation name: Npcf_SMPolicyControl_UpdateNotify

Description: Provides to the NF Service Consumer, e.g. SMF, updated Policy information for the PDU Session.

Inputs, Required: SM Policy Association ID.

Inputs, Optional: Policy information for the PDU Session as defined in TS 23.503 [20] and Policy Control Request Trigger(s) of SM Policy Association as defined in clause 6.1.3.5 of TS 23.503 [20].

Outputs, Required: Success or Failure.

Outputs, Optional: None.

See clause 4.16.5.2 for the usage of this service operation.

# * * * * Next Change * * * *

##### 5.2.5.4.5	Npcf_SMPolicyControl_Update service operation

Service operation name: Npcf_SMPolicyControl_Update.

Description: The NF Service Consumer can request the update of the SM Policy Association to receive updated Policy information for the PDU Session.

Inputs, Required: SM Policy Association ID.

Inputs, Optional: Information on the Policy Control Request Trigger condition that has been met, as defined in clause 6.1.3.5 of TS 23.503 [20].

W-5GAN specific PDU Session information provided by the SMF is specified in TS 23.316 [53].

Outputs, Required: Success or not.

Outputs, Optional: Policy information for the PDU Session as defined in TS 23.503 [20] and Policy Control Request Trigger(s) of SM Policy Association as defined in clause 6.1.3.5 of TS 23.503 [20] and UE Policy Container.

See clause 4.16.5.1 for the usage of this service operation.

NOTE:	When this service operation is invoked by SMF, race conditions apply, which are defined in TS 29.513 [47].

# * * * * Next Change * * * *

##### 5.2.5.6.2	Npcf_UEPolicyControl_Create service operation

Service operation name: Npcf_UEPolicyControl_Create

Description: NF Service Consumer can request the creation of a UE Policy Association by providing relevant parameters about the UE context to the PCF.

Inputs, Required: Notification endpoint, SUPI.

Inputs, Optional: H-PCF ID (if the NF service producer is V-PCF and AMF is NF service consumer), information provided by the AMF as define in clause 6.2.1.2 of TS 23.503 [20], such as Access Type, Permanent Equipment Identifier, GPSI, User Location Information, UE Time Zone, Serving Network (PLMN ID, or PLMN ID and NID, see clause 5.34 of TS 23.501 [2]), RAT type, UE policy information including the list of PSIs, OS id, UE capability of supporting to report URSP rule enforcement to network (see clause 6.6.2.4 of TS 23.503 [20]) and Internal Group (see TS 23.501 [2]), Satellite Backhaul Category (see clause 5.43.2 of TS 23.501 [2]), "5GS to EPS Mobility" indication.

Outputs, Required: Success or Failure, UE Policy Association ID.

Outputs, Optional: Policy Control Request Trigger of UE Policy Association. In the case of H-PCF is producer, UE policy information (see clause 5.2.5.6.1).

# * * * * End of changes * * * *