3GPP TSG-SA WG2 Meeting #157	S2-2308259

# Berlin, Germany,  May 22-26, 2023												  	         (was S2-2308215)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4163 | rev | 3 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NEF MemberUESelectionAssistance_Notify input extension |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-05-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NEF provides the MemberUESelectionAssistance service, where AF subscribes by providing an initial list of UEs and a set of filtering criteria, then the AF receives a list of candidate UEs that meet the criteria via NEF notification. However, in the case of multiple filtering criteria, if a filtering criterion is too tight or faulty, it may exclude many (or even all) UEs in the initial list given by the AF, resulting in a very short (or even empty) candidate UEs list without any explanation from NEF. To address this issue, this CR proposes that for each filtering criterion, NEF includes the number of UEs which don’t meet the criterion as an optional input of the Nnef_MemberUESelectionAssistance_Notify service. This additional information provides AF with insight into the impact of each criterion and allows for adjustments of the criterion in subsequent requests. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding optional input to Nnef_MemberUESelectionAssistance_Notify service indicating the number of UEs that do not meet each filtering criterion |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AF does not know the impact of each filtering criterion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.32.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

##### 5.2.6.32.4	Nnef_UEMemberSelectionAssistance_Notify service operation

Service operation name: Nnef_UEMemberSelectionAssistance_Notify

Description: NEF reports the UE member selection assistance information to the consumer that has previously subscribed.

Inputs, Required: Notification Correlation Information, one or more list(s) of recommended UE(s).

Inputs, Optional: A number for each filtering criterion that indicates the count of UEs in the initial list which do not meet the criterion (provided if there are multiple filtering criteria in the subscribe request)

NOTE: The number can be an indication for AF to revise the corresponding filtering criterion.

Outputs, Required: Operation execution result indication.

********** End of Changes **********