3GPP TSG-SA2 Meeting #157	S2-2307602

# Berlin, Germany, 22nd May 2023 - 26th May 2023	(revision of S2-2306768)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4015 | rev | 5 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  | ME |  | Radio Access Network |  |  |  |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | AMF/PCF interactions to support TNGF/N3IWF selection enhancement for support of S-NSSAI needed by UE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | [Nokia, Nokia Shanghai Bell], LG Electronics |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  |  |  | Date: |  | 2023-05-12 |  |  |  |
| Category: | B |  |  |  |  |  |  |  | Release: |  | Rel-18 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |  |

| Reason for change: | TR 23.700-17 conclusion states that: - The AMF may trigger the UE Policy Association Establishment procedure to provide the UE with updated WLANSP if the selected SSID (TNGF/N3IWF) does not support the slices requested by UE. The AMF requests the PCF to receive a notification when the PCF has completed the WLANSP update; once the AMF has received this notification from the PCF, the AMF can issue a registration reject. - However, there is no such trigger case available for AMF to trigger PCF to update WLANSP or ANDSP rules Same mechanisms have also been defined for N3IWF case Thus existing procedures need to be enhanced In Rev2 Accroding to exisitng procedure, the AMF can partly accept UE requested NSSAI instead of rejecting UE registration. However, according to current description, the AMF shall reject the UE registration if the selected N3IWF/TNGF does not support UE requested slices. This may block the registration of legacy UEs. For example, when a legacy UE selects N3IWF/TNAN based on legacy selection procedure, the selected AN node may not supports all the slices the UE requested. Then there is no option for the AMF except for rejecting UE registration. There should be an option to accept UE registration so that legacy UEs can be registered. |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | the AMF may trigger the UE PCF to update the UE policies related with (un)trusted non-3GPP access selection. The AMF requests the PCF to receive a notification when the PCF has completed the update of such UE policies. The PCF updates the UE policies related with (un)trusted non-3GPP access selection per the procedure defined in figure 4.2.4.3-1. When the update of such UE policies is completed, the PCF notifies the AMF by invoking Npcf_UEPolicyControl_UpdateNotify. In Rev2, The AMF may accept UE registration instead of rejecting UE registration when the selected AN node does not support all the slices requested by the UE. |  |  |  |
| Consequences if not approved: | Failure to support AMF/PCF interactions to support TNGF/N3IWF selection enhancement for support of S-NSSAI needed by UE |  |  |  |
| Clauses affected: | 4.12.2.2, 4.12a.2.2, 5.2.5.6.2, 5.2.5.6.3 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications |  |
| affected: |  | x | Test specifications |  |
| (show related CRs) |  | x | O&M Specifications |  |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

# * * * * First change * * * *

4.12.2.2	Registration procedure for untrusted non-3GPP access

The signalling flow in Figure 4.12.2.2-1 does not show all the details of a registration procedure via untrusted non-3GPP access. It shows primarily the steps executed between the UE and N3IWF. All the details of a registration procedure, including interactions with PCF, UDM, etc. are specified in clause 4.2.2.2.2.

Figure 4.12.2.2-1: Registration via untrusted non-3GPP access

1.	The UE connects to an untrusted non-3GPP Access Network with any appropriate authentication procedure and it is assigned an IP address. For example, a non-3GPP authentication method can be used, e.g. no authentication (in the case of a free WLAN), EAP with pre-shared key, username/password, etc. When the UE decides to attach to 5GC network, the UE not operating in SNPN access mode for NWu interface selects an N3IWF in a 5G PLMN, as described in clause 6.3.6 of TS 23.501 [2]. When the UE decides to attach to 5GC network, the UE operating in SNPN access mode for NWu interface selects an N3IWF in an SNPN, as described in clause 6.3.6.2a of TS 23.501 [2].

NOTE 1:	The UE Selection of a N3IWF that supports the S-NSSAIs needed by the UE is enabled based on ANDSP configuration defined in TS 23.501 [2]. The N3IWF selection based on this information is documented in TS 23.501 [2].

2.	The UE proceeds with the establishment of an IPsec Security Association (SA) with the selected N3IWF by initiating an IKE initial exchange according to RFC 7296 [3]. After step 2, all subsequent IKE messages are encrypted and integrity protected by using the IKE SA established in this step.

3.	The UE shall initiate an IKE_AUTH exchange by sending an IKE_AUTH request message. The AUTH payload is not included in the IKE_AUTH request message, which indicates that the IKE_AUTH exchange shall use EAP signalling (in this case EAP-5G signalling). If the UE supports MOBIKE, it shall include a Notify payload in the IKE_AUTH request, as specified in RFC 4555 [40], indicating that MOBIKE is supported. In addition, as specified in TS 33.501 [15], if the UE is provisioned with the N3IWF root certificate, it shall include the CERTREQ payload within the IKE_AUTH request message to request the N3IWF's certificate. In the case of WLAN access, if the UE has an MPS subscription, the UE shall include a Notify payload in the IKE_AUTH request indicating its MPS subscription.

NOTE 2:	Based on operator policy, the N3IWF can use the MPS subscription indication at this time to handle this UE with priority.

4.	The N3IWF responds with an IKE_AUTH response message, which includes an EAP-Request/5G-Start packet. The EAP-Request/5G-Start packet informs the UE to initiate an EAP-5G session, i.e. to start sending NAS messages encapsulated within EAP-5G packets. If the N3IWF has received a CERTREQ payload from the UE, the N3IWF shall include the CERT payload in the IKE_AUTH response message containing the N3IWF's certificate. How the UE uses the N3IWF's certificate is specified in TS 33.501 [15].

5.	The UE shall send an IKE_AUTH request, which includes an EAP-Response/5G-NAS packet that contains the Access Network parameters (AN parameters) and a Registration Request message. The AN parameters contain information that is used by the N3IWF for selecting an AMF in the 5G core network. This information includes e.g. the GUAMI, the Selected PLMN ID (or PLMN ID and NID, see clause 5.30 of TS 23.501 [2]), the Requested NSSAI and the Establishment cause. The Establishment cause provides the reason for requesting a signalling connection with 5GC. Whether and how the UE includes the Requested NSSAI as part of the AN parameters is dependent on the value of the Access Stratum Connection Establishment NSSAI Inclusion Mode parameter, as specified in clause 5.15.9 of TS 23.501 [2]. The registration request may contain an indication that the UE supports N3IWF selection based on the slices the UE wishes to use over untrusted non-3GPP access (i.e. that the UE supports Extended Home N3IWF identifier configuration and Slice-specific N3IWF prefix configuration). If at step 1 the UE selects the N3IWF based on Tracking/Location Area of same PLMN as described in clause 6.3.6 of TS 23.501 [2], the UE may include this TA in the last visited TAI in registration request in order to help the AMF to determine the target N3IWF as described in step 17.

NOTE 3:	The N3IWF does not send an EAP-Identity request because the UE includes its identity in the first IKE_AUTH. This is in line with RFC 7296 [3] clause 3.16.

6.	The N3IWF shall select an AMF based on the received AN parameters and local policy, as specified in clause 6.3.5 of TS 23.501 [2]. The N3IWF shall then forward the Registration Request received from the UE to the selected AMF within an N2 message. This message contains N2 parameters that include the Selected PLMN ID and optionally the Selected NID and the Establishment cause.

NOTE 4:	The Selected NID is present when the UE connects to an SNPN via Untrusted non-3GPP access.

7.	The selected AMF may decide to request the SUCI by sending a NAS Identity Request message to UE. This NAS message and all subsequent NAS messages are sent to UE encapsulated within EAP/5G-NAS packets.

8.	The AMF may decide to authenticate the UE by invoking an AUSF. In this case, the AMF shall select an AUSF as specified in clause 6.3.4 of TS 23.501 [2] based on SUPI or SUCI.

The AUSF executes the authentication of the UE as specified in TS 33.501 [15]. The AUSF selects a UDM as described in clause 6.3.8 of TS 23.501 [2] and gets the authentication data from UDM. The authentication packets are encapsulated within NAS authentication messages and the NAS authentication messages are encapsulated within EAP/5G-NAS packets. After the successful authentication:

-	In step 8h, the AUSF shall send the anchor key (SEAF key) to AMF which is used by AMF to derive NAS security keys and a security key for N3IWF (N3IWF key). The UE also derives the anchor key (SEAF key) and from that key it derives the NAS security keys and the security key for N3IWF (N3IWF key). The N3IWF key is used by the UE and N3IWF for establishing the IPsec Security Association (in step 11).

-	In step 8h, the AUSF shall also include the SUPI, if in step 8a the AMF provided to AUSF a SUCI.

NOTE 5:	EAP-AKA' or 5G-AKA are allowed for the authentication of UE via non-3GPP access, as specified in TS 33.501 [15]. Figure 4.12.2.2-1 only shows authentication flow using EAP-AKA'. Authentication methods other than EAP-AKA' or 5G-AKA are also allowed for UE accessing SNPN services via a PLMN, as specified in TS 33.501 [15], Annex I, as well as for UE accessing SNPN services directly via Untrusted non-3GPP access.

9a.	The AMF shall send a NAS Security Mode Command to UE in order to activate NAS security. If an EAP-AKA' authentication was successfully executed in step 8, the AMF shall encapsulate the EAP-Success received from AUSF within the NAS Security Mode Command message.

9b.	The N3IWF shall forward the NAS Security Mode Command message to UE within an EAP/5G-NAS packet.

9c.	The UE completes the EAP-AKA' authentication (if initiated in step 8), creates a NAS security context and an N3IWF key and sends the NAS Security Mode Complete message within an EAP/5G-NAS packet.

9d.	The N3IWF relays the NAS Security Mode Complete message to the AMF.

10a.	Upon receiving NAS Security Mode Complete, the AMF shall send an NGAP Initial Context Setup Request message that includes the N3IWF key.

10b.	This triggers the N3IWF to send an EAP-Success to UE, which completes the EAP-5G session. No further EAP-5G packets are exchanged.

11.	The IPsec SA is established between the UE and N3IWF by using the common N3IWF key that was created in the UE in step 9c and received by the N3IWF in step 10a. This IPsec SA is referred to as the "signalling IPsec SA". After the establishment of the signalling IPsec SA, the N3IWF notifies the AMF that the UE context (including AN security) was created by sending a NGAP Initial Context Setup Response. The signalling IPsec SA shall be configured to operate in tunnel mode and the N3IWF shall assign to UE an "inner" IP address. If the N3IWF has received an indication that the UE supports MOBIKE (see step 3), then the N3IWF shall include a Notify payload in the IKE_AUTH response message sent in step 11a, indicating that MOBIKE shall be supported, as specified in RFC 4555 [40].

All subsequent NAS messages exchanged between the UE and N3IWF shall be sent via the signalling IPsec SA and shall be carried over TCP/IP. The UE shall send NAS messages within TCP/IP packets with source address the "inner" IP address of the UE and destination address the NAS_IP_ADDRESS that is received in step 11a. The N3IWF shall send NAS messages within TCP/IP packets with source address the NAS_IP_ADDRESS and destination address the "inner" IP address of the UE. The TCP connection used for reliable NAS transport between the UE and N3IWF shall be initiated by the UE right after the signalling IPsec SA is established in step 11a. The UE shall send the TCP connection request to the NAS_IP_ADDRESS and to the TCP port number specified in TS 24.502 [41].

12.	The AMF determines the allowed subset of the Requested NSSAI that is allowed by the Subscribed S-NSSAI(s); the AMF may detect that the N3IWF used by the UE is not compatible with this allowed subset and based on operator's policy configured in the AMF, the AMF determines whether different N3IWF should be used. If the UE supports slice-based N3IWF selection and the AMF determine to use different N3IWF, then proceed with steps 15-19. Otherwise, i.e. if the AMF determine to use the selected N3IWF that supports part of the allowed subset, the AMF proceeds with step 13 and 14. In this case, steps 15-19 are skipped.

NOTE 6:	The AMF considers the subscribed S-NSSAI(s) before determining to trigger the UE PCF to avoid triggering the UE PCF to update the UE policies for Requested S-NSSAIs that the UE is not subscribed for.

13.	The AMF sends the NAS Registration Accept message in an N2 message sent to the N3IWF. The N2 Message includes the Allowed NSSAI for the access type for the UE. The Allowed NSSAI is a subset of the slices supported by the selected N3IWF.

14.	The N3IWF forwards the NAS Registration Accept message to UE via the established signalling IPsec SA. If the NAS Registration Accept message is received by the N3IWF before the IPsec SA is established, the N3IWF shall store it and forward it to the UE only after the establishment of the signalling IPsec SA.

Steps 15 to 19 correspond to the case where the AMF has detected that the N3IWF used by the UE is not compatible with the subset of the requested NSSAI that is allowed by the subscribed S-NSSAI(s).

15.	If the UE Registration Request contains an indication that the UE supports N3IWF selection based on the slices the UE wishes to use over untrusted non-3GPP access and the AMF is able to select a UE PCF that supports UE policies for slice specific N3IWF selection, the AMF may trigger UE policy association establishment if a suitable UE policy association does not exist yet.

Then the AMF triggers the UE PCF to update the UE policies for slice specific N3IWF selectionby either indicating the request in Npcf_UEPolicyControl_Create or, if a UE policy already exists, by issuing a Npcf_UEPolicyControl_Update.

The AMF requests the PCF to receive a notification when the PCF has completed the update of these UE policies.

NOTE 7:	The UE is assumed to inform PCF whether the UE supports Extended Home N3IWF identifier configuration and Slice-specific N3IWF prefix configuration as part of the UE policy update procedure. Details will be specified in Stage 3 specifications.

16.	The PCF updates the UE policies for slice specific N3IWF selection per the procedure defined in figure 4.2.4.3-1. When the update of these UE policies is completed, the PCF notifies the AMF by invoking Npcf_UEPolicyControl_UpdateNotify.

17.	The AMF sends via the N3IWF a UE Registration Reject indicating that the UE selected N3IWF was not appropriate for the requested slices that the UE is allowed to access to. The AMF optionally may provide target N3IWF information (FQDN and/or IP address) to the UE within the Registration Reject message.

NOTE 8:	The AMF may determine a target N3IWF that supports the subset of the requested NSSAI that is allowed by the subscribed S-NSSAI(s) based on the list of supported TAs and the corresponding list of supported slices for each TA obtained in N2 interface management procedures as specified in TS 38.413 [10]. To determine the target N3IWF, the AMF may take into account UE location corresponding to last visited TAI included in Registration Request as described at step 5 when the UE selects N3IWF based on Tracking/Location Area of same PLMN.

18.	If supported by the UE and if the UE received target N3IWF information in step 17, the UE connects to the target N3IWF, otherwise the UE may perform N3IWF selection again using the updated N3IWF selection information received in step 16. The UE uses the target N3IWF information in the Registration Reject only for the N3IWF selection directly following the rejected registration and UE shall not store for future use.

The AMF provides the Access Type set to "Non-3GPP access" to the UDM when it registers with the UDM and the RAT type determined as specified in clause 5.3.2.3 of TS 23.501 [2].

NOTE 9:	The Access Type and the RAT type are is set to "Untrusted Non-3GPP access" even when the UE accesses SNPN services via PLMN over 3GPP access

# * * * * Next changes * * * *

4.12a.2.2	Registration procedure for trusted non-3GPP access

The UE connects to a trusted non-3GPP Access Network (TNAN) and it also registers to 5GC over via this TNAN, by using the EAP-based procedure shown in the figure 4.12a.2.2. This procedure is very similar with the 5GC registration procedure over untrusted non-3GPP access in clause 4.12.2.2. The link between the UE and the TNAN can be any data link (L2) that supports EAP encapsulation, e.g. PPP, PANA, Ethernet, IEEE 802.3, IEEE 802.11, etc. The interface between the TNAP and TNGF is an AAA interface.

Figure 4.12a.2.2-1: Registration via trusted non-3GPP access

0.	The UE which is not operating in SNPN access mode for Yt interface selects a PLMN and a TNAN for connecting to this PLMN by using the Trusted Non-3GPP Access Network selection procedure specified in clause 6.3.12 of TS 23.501 [2]. During this procedure, the UE discovers the PLMNs with which the TNAN supports trusted connectivity (e.g. "5G connectivity").

The UE operating in SNPN access mode for Yt interface selects an SNPN and a TNAN for connecting to this SNPN by using the Trusted Non-3GPP Access Network selection procedure specified in clause 5.30.2.13 of TS 23.501 [2]. During this procedure, the UE discovers the SNPNs with which the TNAN supports trusted connectivity (e.g. "5G connectivity").

NOTE 1:	In this Release, it is assumed that when the trusted non-3GPP access is a trusted WLAN access, the UE is configured (e.g. with the WLANSP rules defined in TS 23.503 [20]) to select an TNAN(SSID and TNGF) associated with a non-3GPP Tracking Area, which supports one or more of the UE's subscribed S-NSSAIs.

1.	A layer-2 connection is established between the UE and the TNAP. In the case of IEEE Std 802.11 [48], this step corresponds to an 802.11 Association. In the case of PPP, this step corresponds to a PPP LCP negotiation. In other types of non-3GPP access (e.g. Ethernet), this step may not be required.

2-3.	An EAP procedure is initiated. EAP messages are encapsulated into layer-2 packets, e.g. into IEEE 802.3/802.1x packets, into IEEE 802.11/802.1x packets, into PPP packets, etc. The NAI provided by the UE not operating in SNPN access mode for Yt interface indicates that the UE requests "5G connectivity" to a specific PLMN (e.g. NAI = "<any_username>@nai.5gc. mnc<MNC>.mcc<MCC>.3gppnetwork.org"). In the case of WLAN access, if the UE has an MPS subscription, the UE shall also include an indication of its MPS subscription in the username part of the NAI as per TS 23.003 [33]. The NAI provided by the UE operating in SNPN access mode for Yt interface indicates that the UE request "5G connectivity" to a specific SNPN (e.g. NAI = "<any_username>@nai.5gc. nid<NID>.mnc<MNC>.mcc<MCC>.3gppnetwork.org"). If the WLANSP rule contains information including TNGF ID to use for specific slices and the UE supports such information, the UE builds the realm of NAI taking the TNGF ID into account (e.g. NAI = "<any_username>@ tngfid<TNGF ID>. nai.5gc. mnc<MNC>.mcc<MCC>.3gppnetwork.org"). This NAI triggers the TNAP to send an AAA request to a TNGF, which operates as an AAA proxy. Between the TNAP and TNGF the EAP packets are encapsulated into AAA messages. The AAA request also include the TNAP identifier, which can be treated as the User Location Information.

NOTE 2:	In this Release, it is assumed that when the trusted non-3GPP access is a trusted WLAN access, the TNAP selects a TNGF based on the realm (e.g. MCC, MNC and TNGF ID) provided by the UE and also based on the SSID selected by the UE. In a deployment a TNGF may be reached over different SSID(s) where the TNGF supports a Tracking Area and be associated with a set of slices, or an SSID may provide access to one or more TNGF(s), where each of these TNGF(s) can support a different Tracking Area and a different set of slices.

NOTE 3:	Based on operator policy, after receiving the indication of MPS subscription from the UE, the TNAN can treat this UE with priority.

4-10.	An EAP-5G procedure is executed as the one specified in clause 4.12.2.2 for the untrusted non-3GPP access with the following modifications:

-	The registration request may contain an indication that the UE supports TNGF selection based on the slices the UE wishes to use over trusted non-3GPP access (i.e. that the UE supports Extended WLANSP rule).

-	A TNGF key (instead of an N3IWF key) is created in the UE and in the AMF after the successful authentication. The TNGF key is transferred from the AMF to TNGF in step 10a (within the N2 Initial Context Setup Request). The TNGF derives a TNAP key, which is provided to the TNAP. The TNAP key depends on the non-3GPP access technology (e.g. it is a Pairwise Master Key in the case of IEEE Std 802.11 [48]). How these security keys are created, it is specified in TS 33.501 [15].

-	In step 5 the UE shall include the Requested NSSAI in the AN parameters only if allowed, according to the conditions defined in clause 5.15.9 of TS 23.501 [2], for the trusted non-3GPP access. The UE shall also include a UE Id in the AN parameters, e.g. a 5G-GUTI if available from a prior registration to the same PLMN or SNPN. If the UE in SNPN access mode for Yt interface performs the Registration procedure for UE onboarding, the UE shall include an indication in the AN parameters that the connection request is for onboarding.

-	In the N2 message sent in step 6b, the TNGF includes a UE Location Information (ULI) that contains a "null" IP address (e.g. 0.0.0.0) because the UE is not yet assigned an IP address. After the UE is assigned an IP address, the TNGF includes this address in subsequent N2 messages. This N2 message also includes the Selected PLMN ID and optionally the Selected NID, and the Establishment cause.

NOTE 4:	The Selected NID is present when the UE connects to an SNPN via Trusted non-3GPP access.

-	If the UE in SNPN access mode for Yt interface performs the Registration procedure for UE onboarding, the interaction between AMF and AUSF (step 8a and step 8c in Figure 4.12a.2.2-1) is replaced with step 9-1 or step 9-2 or step 9-3 in Figure 4.2.2.2.4-1, depending on the 5GC architecture that is used for UE onboarding.

-	After receiving the TNGF key from AMF in step 10a, the TNGF shall send to UE an EAP-Request/5G-Notification packet containing the "TNGF Contact Info", which includes the IP address of TNGF. After receiving an EAP-Response/5G-Notification packet from the UE in step 10c, the TNGF shall send message 10d containing the EAP-Success packet.

11.	The TNAP key is used to establish layer-2 security between the UE and TNAP. In the case of IEEE Std 802.11 [48], a 4-way handshake is executed, which establishes a security context between the WLAN AP and the UE that is used to protect unicast and multicast traffic over the air.

12.	The UE receives IP configuration from the TNAN, e.g. with DHCP.

13.	At this point, the UE has successfully connected to the TNAN and has obtained IP configuration. The UE sets up a secure NWt connection with the TNGF as follows:

The UE initiates an IKE_INIT exchange using the IP address of TNGF received during the EAP-5G signalling, in step 10b. Subsequently, the UE initiates an IKE_AUTH exchange and provides its identity. The identity provided by the UE in the IKEv2 signalling should be the same as the UE Id included in the AN parameters in step 5. This enables the TNGF to locate the TNGF key that was created before for this UE, during the authentication in step 8. The TNGF key is used for mutual authentication. NULL encryption is negotiated between the UE and the TNGF, as specified in RFC 2410 [49].

In step 13c, the TNGF provides to UE (a) an "inner" IP address, (b) a NAS_IP_ADDRESS and a TCP port number and (c) a DSCP value. After this step, an IPsec SA is established between the UE and TNGF. This is referred to as the "signalling IPsec SA" and operates in Tunnel mode. Operation in Tunnel mode enables the use of MOBIKE [40] for re-establishing the IPsec SAs when the IP address of the UE changes during mobility events. All IP packets exchanged between the UE and TNGF via the "signalling IPsec SA" shall be marked with the above DSCP value. The UE and the TNAP may map the DSCP value to a QoS level (e.g. to an EDCA Access Class [48]) supported by the underlying non-3GPP Access Network. The mapping of a DSCP value to a QoS level of the non-3GPP Access Network is outside the scope of 3GPP.

Right after the establishment of the "signalling IPsec SA", the UE shall setup a TCP connection with the TNGF by using the NAS_IP_ADDRESS and the TCP port number received in step 13c. The UE shall send NAS messages within TCP/IP packets with source address the "inner" IP address of the UE and destination address the NAS_IP_ADDRESS. The TNGF shall send NAS messages within TCP/IP packets with source address the NAS_IP_ADDRESS and destination address the "inner" IP address of the UE.

This concludes the setup of the NWt connection between the UE and the TNGF. All subsequent NAS messages between UE and TNGF are carried over this NWt connection (i.e. encapsulated in TCP/IP/ESP).

14.	After the NWt connection is successfully established, the TNGF responds to AMF with an N2 Initial Context Setup Response message.

15.	The AMF determines the allowed subset of the Requested NSSAI that is allowed by the Subscribed S-NSSAI(s); the AMF may detect that the TNGF used by the UE is not compatible with this allowed subset and based on operator's policy configured in the AMF, the AMF determines whether different TNGF should be used. If the UE supports slice-based TNGF selection and the AMF determine to use different TNGF, then proceed with steps 17-21. Otherwise, i.e. if the AMF determine to use the selected TNGF that supports part of allowed the subset, the AMF proceeds with step 16. In this case, steps 17-21 are skipped.

NOTE 5:	The criteria for the AMF to determine that the TNGF used by the UE is not compatible with the subset of the requested NSSAI that is allowed by the subscribed S-NSSAI(s) is based on local AMF policies. For example the AMF can determine that the TNGF used by the UE is compatible as soon as there is one supported slice in common.

16.	The NAS Registration Accept message is sent by the AMF and is forwarded to UE via the established NWt connection. Now the UE can use the TNAN (a) to transfer non-seamless offload traffic and (b) to establish one or more PDU Sessions.

Steps 17 to 21 correspond to the case where the AMF has detected that TNGF used by the UE is not compatible with the subset of the requested NSSAI that is allowed by the subscribed S-NSSAI(s).

17.	If the UE Registration Request contains an indication that the UE supports TNGF selection based on the slices the UE wishes to use over trusted non-3GPP access and AMF is able to select a UE PCF that supports UE policies for slice specific trusted access selection, the AMF may trigger UE policy association establishment if a suitable UE policy association does not exist yet.

Then the AMF triggers the UE PCF to update the UE policies for slice specific trusted access selection by either indicating the request in Npcf_UEPolicyControl_Create or, if a UE policy already exists, by issuing a Npcf_UEPolicyControl_Update. The AMF requests the PCF to receive a notification when the PCF has completed the update of these UE policies.

NOTE 6:	The UE is assumed to inform PCF whether the UE supports Extended WLANSP or ANDSP as part of the UE policy update procedure. Details will be specified by CT WG1.

18.	The PCF updates the UE policies for slice specific trusted access selection per the procedure defined in figure 4.2.4.3-1.

19.	When the update of these policies is completed, the PCF notifies the AMF by invoking Npcf_UEPolicyControl_UpdateNotify.

20.	The AMF sends via the TNGF a UE Registration Reject indicating that the selected TNGF was not appropriate for the requested slices that the UE is allowed to access to. The AMF may provide target TNAN information (SSID, TNGF ID) to the UE within the Registration Reject message indicating the UE to build the NAI based on the TNGF ID.

NOTE 7:	The AMF may determine a target TNGF that supports the subset of the requested NSSAI that is allowed by the subscribed S-NSSAI(s) based on the list of supported TAs and the corresponding list of supported slices for each TA obtained in N2 interface management procedures as specified in TS 38.413 [10], and considering UE location.

21.	If supported by the UE and if the UE received target TNAN information in step 20, the UE connects to the target TNAN, otherwise the UE may perform TNAN selection again using the updated WLANSP rule received in step 18. If the target TNAN information includes TNGF ID, the UE shall build the NAI based on TNGF ID. The UE uses the target TNAN information in the Registration Reject only for the TNAN selection directly following the rejected registration and UE shall not store it for future use.

# * * * * Next changes * * * *

5.2.5.6.2	Npcf_UEPolicyControl_Create service operation

Service operation name: Npcf_UEPolicyControl_Create

Description: NF Service Consumer can request the creation of a UE Policy Association by providing relevant parameters about the UE context to the PCF.

Inputs, Required: Notification endpoint, SUPI.

Inputs, Optional: H-PCF ID (if the NF service producer is V-PCF and AMF is NF service consumer), information provided by the AMF as define in clause 6.2.1.2 of TS 23.503 [20], such as Access Type, Permanent Equipment Identifier, GPSI, User Location Information, UE Time Zone, Serving Network (PLMN ID, or PLMN ID and NID, see clause 5.34 of TS 23.501 [2]), RAT type, UE policy information including the list of PSIs, OS id, UE capability of supporting to report URSP rule enforcement to network (see clause 6.6.2.4 of TS 23.503 [20]) and Internal Group (see TS 23.501 [2]), Satellite Backhaul Category (see clause 5.43.2 of TS 23.501 [2]), request to update the UE policies, request to be notified when updated UE policies have been provided to the UE.

Outputs, Required: Success or Failure, UE Policy Association ID.

Outputs, Optional: Policy Control Request Trigger of UE Policy Association. In the case of H-PCF is producer, UE policy information (see clause 5.2.5.6.1)

# * * * * Next changes * * * *

5.2.5.6.3	Npcf_UEPolicyControl_UpdateNotify service operation

Service operation name: Npcf_UEPolicyControl_UpdateNotify

Description: Provides to the NF Service Consumer updated Policy information for the UE context evaluated based on the information previously provided by the PCF.

NOTE:	This notification corresponds to an implicit subscription.

Inputs, Required: Notification endpoint, UE Policy Association ID.

Inputs, Optional: Policy Control Request Trigger of UE Policy Association. In the case of H-PCF is producer, UE Access and PDU session related information as defined in clause 5.2.5.1, indication of successful delivery of UE policies.

NOTE:	The indication of successful delivery of UE policies is used e.g. as defined in clauses 4.12.2.2 and 4.12a.2.2.

Outputs, Required: Success or failure.

Outputs, Optional: None.

# * * * * End of changes * * * *