3GPP TSG-WG SA2 Meeting #157	S2-2307609

# Berlin, May 22th – 26th, 2023						                                        (revision of S2-2306360)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4566 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23501_CR_Update to support overload control when UEs leaving coverage. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is valuable if the network can control the time range during which mass UEs initiate NAS signalling before leaving coverage, by considering e.g. the amount of users under the satellite coverage and the processing capacity of network. The signalling load is different between the NAS signalling for leaving coverage and re-gaining coverage, as re-gaining coverage may need more steps for subsequent possible services. The network should determine indepedent maximun time range for UEs to initiate NAS signalling before leaving coverage, comparing to the maximum waiting time for UEs to initiate NAS signalling after re-gaining coverage. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It proposes the network to determines a maximum leading time for the UE to decide a random time when to initiate NAS signalling before leaving coverage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not support overload control when UEs leaving coverage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.13.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X |  |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4.13.5	Overload control

The AMF and UE may only use the procedure defined in this clause if both the UE and AMF indicate "Discontinuous Coverage Supported", see clause 5.4.13.1.

In order to avoid a large number of UEs causing excessive signalling load on the network before leaving coverage or re-gaining coverage after being out of coverage, the AMF may accordingly determine a maximum leading time offset or a maximum waiting time offset controlling when UEs are allowed to initiate NAS signalling with the network, as described in this clause.

In this case, the AMF determines the maximum leading time offset and/or the maximum waiting time offset based on network configuration, the amount of users under the satellite coverage, priority users and priority service as specified in TS 23.122 [17] and TS 24.501 [47]. The AMF sends this maximum leading time offset and/or this maximum waiting time offset to individual UEs during the Registration procedure or UE Configuration Update procedure.

If the UE receives a maximum leading time offset and/or a maximum waiting time offset from the network in a Registration Accept or UE Configuration Update Command message, the UE shall replace any previously received maximum leading time offset and/or maximum waiting time offset on the same RAT type and PLMN with this one.

When the UE knows a later time at which coverage will be lost and when the UE does not send Mobility Registration Update to the AMF in advance (see clause 5.4.13.1), the UE sets the leading time value to a random value up to and including the latest maximum leading time offset for this PLMN and RAT type. The UE shall send the Mobility Registration Update to the AMF indicating the loss of coverage and the start time of Unavailability Period (see clause 5.4.1.4) at the leading time before losing coverage.

Upon returning in coverage after being out of coverage due to discontinuous coverage, the UE sets the discontinuous coverage wait timer value to a random value up to and including the latest maximum waiting time for this PLMN and RAT type, and starts this timer. The UE shall not initiate any NAS signalling on that RAT Type and PLMN while the discontinuous coverage wait timer is running.

The UE shall stop the discontinuous coverage wait timer and initiate NAS signalling if the UE receives paging message, has pending emergency services or when UE enters a TAI outside the registration area.

Editor's note:	It is for CT WG1 to decide whether the maximum wait time will require a new IE or reuse existing IE used for MINT i.e. disaster return wait range information. SA WG2 specifications will be aligned based on CT WG1 decision.

# * * * * End of changes * * * *