3GPP TSG-SA WG2 Meeting #157	S2-2307724

# May 22 – 26, 2023, Berlin, Germany	         (revision of S2-2304671, 5588, 6621, 7673, 7703)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0338 | rev | 5 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LCS services assisted by NWDAF analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Samsung, Nokia, Nokia Shanghai Bell, Huawei, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For verifying UE location via satellite access, the KI#9 has concluded that AMF may use the analytics from NWDAF to assist with UE location verification, which needs to be captured in the LCS specification. “- The AMF is the entity in charge of providing the location verification decision, in line with Rel-17 mechanism of UE location verification: - The AMF may trigger location service procedures as defined in TS 23.273 [5] to determine the UE location verification decision and optional TAI determination. Location information received at AMF is provided by LMF via the NI-LR procedure. The LMF may decide specific positioning methods to be used for verification based on RAN WG decisions. - The AMF may receive assistance information from NWDAF (i.e. analytics containing UE location information) to perform the location verification decision.” Besides, to address the procedure between LCS(AMF, LMF) and NWDAF, a new clause is added to capture how NWDAF analytics helps location services in positioning method determination(LMF) and UE location verification for NR satellite access(AMF). |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Introducing NWDAF analytics assisting with UE location verification for NR satellite access in general subclause and 5GC-NI-LR procedure. 2. Proposing a new procedure clause to capture how NWDAF analytics helps location services in positioning method determination(LMF) and UE location verification for NR satellite access(AMF). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NWDAF analytics assisting with UE location verification is not captured in TS 23.273. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.7, 5.16, 6.10.1, 6.x(new), 4.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | CR Rev-5: - Rewrite clause 5.16, to add select or adjust of positioning method - Add a NOTE to give an example of how to utilize LCS accuracy analytics - Add functionality of LMF on determination of indoor or outdoor |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.3.7	Access and Mobility Management Function, AMF

The AMF contains functionality responsible for managing positioning for a target UE for all types of location request. The AMF is accessible to the GMLC and NEF via the Namf interface, to the RAN via the N2 reference point and to the UE via the N1 reference point.

Functions which may be performed by an AMF to support location services include the following.

-	Initiate an NI-LR location request for a UE with an IMS emergency call or to know a UE geographical area with NR satellite access for PLMN selection verification.

-	Receive and manage location requests from a GMLC for a 5GC-MT-LR and deferred 5GC-MT-LR for periodic, triggered and UE available location events.

-	Receive and manage location requests from a UE for a 5GC-MO-LR.

-	Receive and manage Event Exposure request for location information from an NEF.

-	Select an LMF.

-	Receive updated privacy requirements from a UE and transfer to a UDR via UDM.

-	Support cancelation of periodic or triggered location reporting for a target UE.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	When assistance data is broadcast by 5GS in ciphered form, the AMF receives ciphering keys from the LMF and forwards to suitably subscribed UEs using mobility management procedures.

-	Store UE Positioning Capability received from an LMF and send the UE Positioning Capability along with the received location request to an LMF.

-	Receive UL NAS Transport including a PRU Association, Association Update, or Disassociation Request (contained in an LCS supplementary service message) from a UE.

-	AMF may verify whether a UE can serve as a PRU based on UE subscription data after receiving the PRU Association Request, Association Update, or Disassociation. AMF may also use local policy to determine if UEs are allowed to serve as a PRU.

-	Sends the PRU Association Request or PRU Disassociation Request to LMF and may include a UE verification indication indicating whether this UE is authorized to serve as a PRU.

-	Support triggering LMF to establish a User Plane Connection to UE if UE requested that.

-	Store in UE context that UE has a maintained User Plane connection with certain LMFs.

NOTE:	Details of UE Positioning Capability is defined in TS 37.355 [20].

-	Support of local configuration of a mapping table of UE identifier ranges and LMF identifier(s) or querying the UDM the LMF identifier for a UE for a 5GC-MO-LR.

-	Support the 5G to EPS Handover by providing the target MME ID to GMLC as part of the LCS Service response.

-	Support interaction with NWDAF to obtain UE related analytics to assist with UE location verification for NR satellite access.

# * * * * Second change * * * *

## 5.16	Location services assisted by NWDAF

LMF and AMF may utilize analytics from NWDAF to assist with location services as described in clause 6.x.

NWDAF may provide analytics for Location Accuracy as specified in TS 23.288 [37] to LMF.

When LMF receives analytics for Location Accuracy from the NWDAF, it may use the analytics to determine the indoor/outdoor environment, to select or adjust appropriate positioning methods, for the requested location accuracy.

NWDAF may provide UE related analytics as specified in clause 6.7 of TS 23.288 [37] to AMF. The UE location information received in analytics can be used by AMF to perform the UE location verification for NR satellite access.

NWDAF may request an aggregated UE location report from LMF (via GMLC) to include multiple UE location results. NWDAF may also provide a reporting time to indicate the latest time for the reporting.

# * * * * Third change * * * *

### 6.10.1	5GC-NI-LR Procedure

Figure 6.10.1-1 shows a Network Induced Location Request (NI-LR) procedure for a UE in the case where the UE initiates an emergency session or other session using NG-RAN. The procedure assumes that the serving AMF is aware of the regulatory service associated with the session (e.g. emergency session initiation - e.g. due to supporting an Emergency Registration procedure or assisting in establishing an emergency PDU Session). The procedure can also be used to verify UE location for NR satellite access.

Figure 6.10.1-1: 5GC Network Induced Location Request (5GC-NI-LR) for a UE

1.	A trigger for AMF to initiate the 5GC-NI-LR procedure happens, e.g. the UE registers to the 5GC for emergency services or requests the establishment of a PDU Session related to an applicable regulatory service (e.g. emergency session initiation) or the AMF decides to verify UE location via LCS procedure for a UE registered for NR satellite access.

2.	For verifying UE location via LCS service for NR satellite access this step is mandatory, for other triggers this step is optional. The AMF selects an LMF based on NRF query or configuration in AMF and invokes the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE. The service operation includes a LCS Correlation identifier, the serving cell identity of the Primary Cell in the Master RAN node and the Primary Cell in the Secondary RAN node when available based on Dual Connectivity scenarios, and an indication of a location request from a regulatory services client (e.g. emergency services) and may include an indication if UE supports LPP, the required QoS and Supported GAD shapes, the UE Positioning Capability if available. When AMF needs to know the geographical area of the UE to check whether the PLMN is allowed to operate in the area, an indication of this is included. When AMF needs to verify the location of the UE when a UE registers via NR satellite access, the AMF includes in indication in the request to the LMF that the request is for UE location verification. If any of the procedures in clause 6.11.1 or 6.11.2 are used the service operation includes the AMF identity. Optionally, the AMF may also request UE related analytics containing UE location information (e.g., UE mobility analytics) from NWDAF to assist with UE location verification for NR satellite access as described in clause 6.x.2.

3.	[Conditional] If step 2 occurs, the LMF performs one or more of the positioning procedures described in clause 6.11.1, 6.11.2 and 6.11.3. If the AMF included an indication at step 2, the LMF maps the UE location to a geographical area where a PLMN is or is not allowed to operate.

4.	[Conditional] If step 3 occurs, the LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF to return the current location of the UE. The service operation includes the LCS Correlation identifier, the location estimate, its age and accuracy and may include information about the positioning method and the timestamp of the location estimate. The service operation also includes the UE Positioning Capability if the UE Positioning Capability is received in step 3 including an indication that the capabilities are non-variable and not received from AMF in step 2. When UE geographical area determination for location verification is indicated at step 2, the service operation also returns the geographical area where a PLMN is or is not allowed to operate determined at step 3.

NOTE 1:	Any remaining procedures for regulatory services other than emergency services are not addressed in this flow. The remaining steps are applicable for emergency services.

5.	[Conditional] For emergency services, the AMF selects an GMLC based on NRF query or configuration in AMF. The information regarding the endpoint in the GMLC to deliver the event notification, is obtained from the NRF as specified in clause 7.1.2 of TS 23.501 [18] or from local configuration in the AMF. AMF invokes the Namf_Location_EventNotify service operation towards the selected GMLC to notify the GMLC of an emergency session initiation. The service operation includes the SUPI or the PEI, and the GPSI if available, the identity of the AMF, an indication of an emergency session and any location obtained in step 3.

6.	[Conditional] For emergency services, the GMLC forwards the location to an external emergency services client or may wait for a request for the location from the external emergency services client (not shown in Figure 6.10.1-1) before forwarding the location.

7.	[Conditional] For emergency services, the emergency services session and emergency PDU Session are released.

8.	[Conditional] For emergency services, the AMF invokes the Namf_Location_EventNotify service operation towards the GMLC to notify the GMLC that the emergency session was released to enable the GMLC and LRF to release any resources associated with the emergency session.

# * * * * Forth change * * * *

## 6.X	Procedure for NWDAF assistance to location services

### 6.x.1	General

This clause specifies procedures for NF in the LCS system to interact with NWDAF for data analytics. General framework for NF in the LCS system to interact with NWDAF for data analytics refers to the clause 6.1.1 and 6.1.2 in TS 23.288 [37].

Clause 6.x.2 describes the procedures of Location Accuracy Analytics retrieved by LMF.

Clause 6.x.3 describes the procedures of UE Mobility Analytics retrieved by AMF.

NOTE: in this release of specification, only AMF and LMF can be NF service consumers to NWDAF.

The procedure in Figure 6.X-1 can be used by a 5GC NF involved in 5GC location services to get optional assistance from NWDAF as shown below.

Figure 6.X-1: Procedure for NWDAF assistance to location services

1.	The AMF may invoke Nlmf_Location_DeterminationLocation service operation towards the LMF to request the current location of the UE.

2.	Assistance from NWDAF for location services may be requested from the LMF and/or AMF.

3.	The LMF may trigger UE positioning.

4.	The LMF may return the location estimate to AMF as specified in clause 8.3.2.

### 6.x.1	Location Accuracy Analytics Retrieval by LMF

LMF may request Location Accuracy Analytics from NWDAF to retrieve the location accuracy.

The procedure to request Location accuracy analytics by LMF is referred to TS 23.288[37], clause 6.17.4. In Figure 6.17.4-1 the Analytics consumer is replaced by LMF. Pre-condition of the procedure is NWDAF has a trained supervised ML model for deriving Location Accuracy Analytics.

NOTE:  To assist the determination of the positioning methods for the requested LCS accuracy with the LCS accuracy analytics, LMF may know the accuracy of the positioning method(s) and NLOS/LOS measurement indication if the positioning method is NR based.

### 6.x.2	UE Mobility Analytics Retrieval by AMF

AMF may request assistance for UE location verification for NR satellite access as described in clause 6.10.1 by requesting or subscribing to UE mobility analytics from NWDAF. With NWDAF-based UE location statistics and predictions and UE location estimated by LMF, AMF can further assist UE location verification for NR satellite access.

# * * * * Fifth changes * * * *

### 4.3.8	Location Management Function, LMF

The LMF manages the overall co-ordination and scheduling of resources required for the location of a UE that is registered with or accessing 5GCN. It also calculates or verifies a final location and any velocity estimate and may estimate the achieved accuracy. The LMF receives location requests for a target UE from the serving AMF using the Nlmf interface. The LMF interacts with the UE in order to exchange location information applicable to UE assisted and UE based position methods and interacts with the NG-RAN, N3IWF or TNAN in order to obtain location information.

The LMF shall determine the result of the positioning in geographical co-ordinates as defined in TS 23.032 [8] and/or in local co-ordinates as defined in TS 23.032 [8]. If requested and if available, the positioning result may also include the velocity of the UE. The coordinate type(s) is determined by LMF when receiving a location request, based on LCS Client type and supported GAD shapes. If the location request indicates regulatory LCS Client type the LMF shall determine a geographical location and optionally a location in local coordinates. For location request indicates a value added LCS Client type, the LMF may determine the UE location in local coordinates or geographical co-ordinates or both. If the supported GAD shapes is not received or Local Co-ordinates is not included in the supported GAD shapes, the LMF shall determine a geographical location.

NOTE 1:	Some RAT independent position methods (e.g. GNSS based position methods) can only determine a UE location in geographical co-ordinates. In such a case, the LMF may translate a UE location in geographical co-ordinates into a location in local co-ordinates when an origin for the local co-ordinates has known global coordinates. When an origin for the local co-ordinates does not have known global coordinates, position methods that can only determine a UE location in geographical co-ordinates cannot be used to determine a UE location in local co-ordinates.

Additional functions which may be performed by an LMF to support location services include the following.

-	Support a request for a single location received from a serving AMF for a target UE.

-	Support a request for periodic or triggered location received from a serving AMF for a target UE.

-	Determine type and number of position methods and procedures based on UE and PLMN capabilities, QoS, UE connectivity state per access type, LCS Client type, co-ordinate type and optionally service type and indication of requiring reliable UE location information.

-	Report UE location estimates directly to a GMLC for periodic or triggered location of a target UE.

-	Support cancelation of periodic or triggered location for a target UE.

-	Support the provision of broadcast assistance data to UEs via NG-RAN in ciphered or unciphered form and forward any ciphering keys to subscribed UEs via the AMF.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	Support of receiving stored UE Positioning Capability from AMF and support of providing updated UE Positioning Capability to AMF.

-	Map the UE location to a geographical area where the PLMN is or is not allowed to operate based on the request from AMF.

-	Support determination of a UE location at a scheduled location time.

-	Support determination of indoor or outdoor for a location estimate.

-	Determine whether to use user plane or control plane for positioning.

-	Support handling of 5GC-MT-LR, 5GC-NI-LR, 5GC-MO-LR and deferred 5GC-MT-LR for periodic or triggered location over a user plane connection between UE and LMF.

Editor's note:	LMF functionality of user plan connection handling will be updated according to the protocol stack decision made by CT WG1 and SA WG3 groups.

-	Support collection of GNSS assistance data from AFs.

-	Support service level PRU Association, PRU Association update or PRU Disassociation.

-	LMF supports verification of a PRU initiated Association or Disassociation by checking whether there is an PRU verified indication from AMF.

-	LMF stores the received PRU information contained in service level PRU Association message.

-	LMF may indicate support of PRU function to NRF via NF profile and may further send the PRU information to NRF via NF profile update.

-	LMF may request a PRU to associate to a new LMF by returning a Routing ID of the new LMF.

-	Support selection of a PRU based on stored PRU information if the LMF needs to obtain the location measurements from the PRU to assist positioning of a target UE.

-	Support to obtain PRU location measurements as described in clause 5.4.5 of TS 38.305 [9] by triggering the procedure in clause 6.11.

-	Support to obtain PRU location measurements from other PRU serving LMF(s).

-	As a serving LMF of target UE(s), support discovery and selection of other PRU serving LMF(s) by querying the NRF and support to request PRU location measurements from the selected LMF(s).

-	As a serving LMF of PRU(s), support to provide PRU location measurements to other LMF(s) after receiving a request from other LMF(s).

-	Support to determine UE location by considering obtained PRU location measurements.

Editor's note:	Whether and how AF provides PRU information (e.g. known location) on behalf of a PRU to network is FFS.

NOTE 2:	Country, area within a country, or an international area can be supported as different types of geographical area.

-	Support a request for user plane reporting from a UE to an LCS Client or AF for a periodic or triggered 5GC-MT-LR. Subsequently, support the transfer of cumulative event reports from the target UE via control plane back to the H-GMLC and LCS Client or AF. Also support any request for assistance data received in a cumulative event report.

-	Determine UE location for a UE connecting to a MBSR based on location and velocity of the MBSR and the timing of the location estimations for the target UE and MBSR.

# * * * * End of changes * * * *