SA WG2 Meeting #157	S2-2307924

# Berlin, Germany, May 22nd – 26th, 2023           	  			  			       (revision of S2-2306836)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4621 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Informative annex on PIN traffic routing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The general session management principles as described in clause 5.6, the QoS model as defined in clause 5.7 and the User Plane management for 5GS as defined in clause 5.8 are applicable to PIN-DN communication and PIN-indirect communication. If a PIN has multiple PEGC UEs, 5G VN group communication mechanisms can also be used for PIN indirect communication. Currently in Annex P (Informative), only a PIN Reference Architecture is defined. This CR proposes the add one more informative annex to clarify the PIN session management and traffic routing aspects |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause Annex P.y is added under informative Annex P, to explain the PIN session management and traffic routing aspects. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification for PIN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex P.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changes in rev1: - Removed the text “Ethernet PDU sessions are not supported” as this needs to go into a normative clause - added a NOTE that the Figure does not show traffic from a PEMC |  |  |  |  |  |  |  |  |

FIRST CHANGE (all texts are new)

# P.y	Session management and traffic routing for PIN

The general session management principles as described in clause 5.6, the QoS model as defined in clause 5.7 and the User Plane management for 5GS as defined in clause 5.8 are applicable to PIN-DN communication and PIN-indirect communication.

If a PIN has multiple PEGC UEs, 5G VN group communication mechanisms can be used for PIN indirect communication (i.e. communication between PIN Elements belonging to the same PIN but served by different PEGC UEs). In this case a dedicated SMF set is used for managing the PIN related PDU Sessions from all the PEGCs of that PIN and the PDU session management principles for 5G VN-LAN-type services as specified in clause 5.29.3 are applicable. The user plane handling for 5G LAN-type services as specified in 5.29.4 are applicable with following differences:

-	For PIN indirect communication N19-based traffic forwarding is not used i.e. the PIN traffic is forwarded using

-	N6-based traffic forwarding method, where the UL/DL traffic for the PIN communication is forwarded to/from the DN;

-	local switching as depicted in Figure P.y-1 below, following the principles of local swithing of traffic for 5G VN LAN-type service

Figure P.y-1: Local-switch based user plane architecture for PIN

NOTE:	Figure P.y-1 does not show traffic from a PEMC

The SMF configures the UPF(s) to apply N6-based traffic forwarding to route traffic between PDU Sessions of different PEGC UEs of a PIN as specified in clause 5.8.2.13. The SMF can apply local switching as specified in clause 5.8.2.13 in order to enable UPF locally forward uplink stream from one PDU session of one PEGC UE of a PIN as downlink stream of PDU session of one or more PEGC UE(s) of the same PIN. For local switching of PIN traffic between PIN related PDU sessions from different PEGC UEs of a single PIN, based on the DNN and S-NSSAI that is used for the PDU session related to PIN, the SMF provides a Network Instance to the UPF in FAR and/or PDR via N4 Session Establishment/Modification procedures.

END OF CHANGES