3GPP TSG SA WG2 Meeting SA2#157		S2-2308146

May 22 – 26, 2023, Berlin, Germany

Title:	Proposed method for AMF discovery and subscription by the TSCTSF

Release:	Rel-18

Work Item:	TRS_URLLC

Source:	SA2

To:	CT4

Cc:	-

Contact person: 	 devaki.chandramouli@nokia.com

Attachment:

# 1	Overall description

SA2 is discussing two methods for the TSCTSF to discover AMF to support coverage area filters as the attached CR updates in clause 5.27.1.10. The coverage area defines a spatial validity condition for the UE that is resolved at the TSCTSF subscribing to “UE presence in AoI” at the AMF. The methods the TSCTSF can use to discover and subscribe to the AMF are as follows:

- discover the AMF(s) serving the list of TA(s) that comprise the spatial validity using the NRF and subscribes to the discovered AMF(s) to receive notifications about presence of any UE and list of UE(s) in an Area of Interest events (as described in clause 5.3.4.4 of TS 23.501 [2]). It is required that the subscribed event remain sticky to the AMF even if it is does not serve a given UE. It is required that the subscribed event applies only to the list of UEs in the subscribe request. The TSCTSF may include an "Adjust AoI based on RA" indicator in the subscription to the UE mobility event notification. An Area of Interest (AoI) for each AMF is represented by a list of TA(s), wherein the Area of Interest is identical to the requested coverage area, or the Area of Interest is a TA. (or)

- determine the serving AMF for each of the targeted UE(s) using the UDM and subscribe to the serving AMF to receive notification about presence of the UE in an Area of Interest (as described in clause 5.3.4.4 of TS 23.501 [2]). The TSCTSF may include an "Adjust AoI based on RA" indicator in the subscription to the UE mobility event notification. An Area of Interest (AoI) for each AMF is represented by a list of TA(s), wherein the Area of Interest is identical to the requested coverage area, or the Area of Interest is a TA.

SA2 question: There were concerns related to excessive signaling and failure issues thus SA2 would like to kindly request CT4 to provide feedback regarding the two methods from stage 3 perspective.

# 2	Actions

To CT4

ACTION: 	SA2 kindly asks for feedback as requested above.

# 3	Dates of next TSG SA WG2 meetings

TSG SA WG2 Meeting #158   August 21 – 25, 2023 	Goteborg, Sweden

TSG SA WG2 Meeting #159   October 21 - 25, 2023	Xiamen, China