3GPP TSG-SA WG2 Meeting #157	S2-2308209

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-2307953)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4194 | rev | 2 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLsys: KI#5 – Applying PDTQ Policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | After AF negotiate PDTQ policy, when the AF wants to make use of PDTQ Policy for existing or new sessions, AF will invoke Npcf_PolicyAuthorization_Create/Update or Nnef_AFsessionWithQoS_Create/Update with negotiated PDTQ Policy as described in clause 6.1.2.7 of TS 23.503. When the AF wants to make use of the PDTQ policy for existing or new sessions, then the AF will invoke, for each UE, the Nnef_AFsessionWithQoS/Npcf_PolicyAuthorization during the recommended time window. However, corresponding descriptions to apply PDTQ policy is not implemented in TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add description to apply negotiated PDTQ policy |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.15.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.16.15.1	General

The intent of this clause is to specify generic service procedures to enable the AF to negotiate viable time window for the planned application data transfer with specific QoS requirements and operational conditions via the support of the NEF.

The PDTQ policies are defined for a specific ASP which include the respective Desired Time Windows, the optional Network Area Information, the Request for Notification if the AF accepts that the PDTQ policy can be re-negotiated using the PDTQ warning notification procedure.

The Network Performance analytics or DN Performance analytics for NWDAF as described in TS 23.288 [50] will be subscribed by the PCF to assist its decision to derive the PDTQ policies for the selected time window(s).

One or more negotiated PDTQ policies could be provided by PCF to AF via NEF together with the PDTQ Reference ID. If the AF receives more than one PDTQ policies from the PCF, the AF will select one of them and inform the PCF about the selected PDTQ policy which will then be stored in the UDR to be applied to the AF session when requested by the AF. Prior to the start of the Desired Time Window for the planned application data transfer, the AF requests the PCF to set up the AF session with required QoS. The PCF will then determine the appropriate PCC rules according to the AF request.

* * * * End of Changes * * * *