SA WG2 Meeting #S2-158	S2-2308320

21 - 25 August, 2023, Goteborg, Sweden

3GPP TSG-RAN3 Meeting #120	R3-233347

Incheon, Korea, 22th May - 26th May 2023

Title:	Reply LS on INACTIVE eDRX above 10.24sec and SDT

Response to:	R3-232506 / S2-2305619

Release:	Release 18

Work Item:	NR_RedCap_Ph2, NR_redcap_enh-Core

Source:	RAN3

To:	SA2, CT4

Cc:	RAN2

Contact Person:

#### Name:	Yazid Lyazidi

E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 thanks RAN2 for their LS (R2-2302082 / S2-2303947) on “INACTIVE eDRX above 10.24sec and SDT” and thanks SA2 for their reply LS (R3-232506 / S2-2305619) on that matter.

RAN3 would like to inform SA2, RAN2 and CT4 that RAN3 discussed the changes to the NGAP signalling based on the updated call flows from SA2 in TS 23.502:

- RAN3 has agreed to include the RAN configured Paging Time Window (PTW) in the MT COMMUNICATION HANDLING REQUEST message, when the NG-RAN sends this message to request CN applying HLCOM.

- Following SA2 and CT4 updates, RAN3 has agreed to include the QFI, PPI, ARP and PDU session ID of the QoS Flow in the DL DATA NOTIFICATION message from AMF to NG-RAN requesting connection resume. Concerning signalling 5QI, RAN3 is still discussing it and would like to check with SA2, if the signalled 5QI can be different than the one associated with the stored QFI in the RAN UE context.

- Furthermore, RAN3 has discussed the need to signal the DL data size in the DL DATA NOTIFICATION message to help NG-RAN making decision if MT-SDT paging can be performed based on the QFI associated with the configured SDT radio bearer. RAN3 would like to ask SA2 and CT4 whether it is feasible that DL data size can be signalled to the NG-RAN (e.g. for QoS flows that have buffered data in the CN) along with the paging differentiation parameters to assist NG-RAN for MT-SDT paging decision.

2. Actions:

To SA2, CT4:

ACTION: 	RAN3 kindly asks SA2 and CT4 to take the information above into account and to feedback on the two questions.

3. Date of Next RAN3 Meetings:

3GPP RAN3#121   			21 – 25 August 2023 			Toulouse, FR

3GPP RAN3#121bis                                09 – 13 October 2023			Xiamen, China