3GPP SA WG2 Meeting #158	S2-2308750

Goteborg, August 21 – 25 2023	(revision of S2-2306435)

# Source:	Ericsson, AT&T, Verizon, Oracle

# Title:	Minimize the number of Policy Associations

# Document for:	Approval

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Minimize the Number of Policy Associations

Acronym:	TEI19_MINPA

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

UE Policy Association Establishment - At Initial Registration or at mobility from EPS to 5GS, the UE can send the UE Policy Container to the network for the purpose to both report the list of stored UE Policies, implicitly requests UE Policies and to report some capabilities, such as ANDSP support, or the support for reporting the enforcement of a URSP Rule. The reception of the UE Policy Container at the AMF triggers the UE Policy Association Establishment procedure to deliver the UE Policy Container to the PCF. The UE Policy Association is maintained until the UE deregisters or the PCF changes while the UE is registered, the purpose is to deliver changes in the UE context, such as change of the connectivity state of the UE, to the PCF, this helps the PCF to update the UE if needed. The UE sends the UE Policy Container without knowing if the PCF will send UE Policies to the UE.

If the AMF does not receive a UE Policy Container, the AMF may still attempts to establish a UE Policy Association with the PCF based on local configuration.

In some cases the establishment of the UE Policy Association may be unnecessary, e.g., some IoT devices may not need any UE Policies. This can negatively impact network performance and increases signalling.

AM Policy Association Establishment - The AMF determines based on local configuration whether AM Policies are needed, this local configuration, e.g. per Allowed NSSAI, may trigger of AM Policy Association Establishment  procedure with the PCF. The PCF may not have any AM Policies for this SUPI.

Similar to what is explained above for UE Policy Association Establishment, this may lead to waste of network resources (i.e. the AM Policy Association remains unnecessaryily established) and may lead to performance degradation.

# 4	Objective

The objective is to have the UE Policy Association established only when there are UE Policies to be sent to the UE. Similar for AM Policies Association.

The respective NF consumer (i.e. AMF) should be able to determine the above by examining the combination of user subscription information and local configuration.

Specifically, the following enhancements are needed for the AMF to be able to determine whether an establishment of the UE Policy Association is required:

- The Access and Mobility Subscription Data Type is extended with an indication of whether the UE Policy Association is enabled, disabled or no control of UE Policies. The UE Policy Association indication is enabled for SUPIs that are subscribed to a service that requires sending UE Policies to the UE. The UE Policy Association indication is disabled for SUPIs that are not subscribed to any service that requires sending UE Policies to the UE. The UE Policy Association indication is “no control of UE Policies” for SUPIs ensures backward compatibility to enable AMF to use the existing local configuration and the presence of the UE Policy Container.

- At initial registration or EPC to 5GS  mobility, the AMF triggers the UE Policy Association Establishment to the PCF using both the reception of UE Policy Container and the value of the new indication received from UDM. If the UE policy Container is not received the AMF triggers the UE Policy Association Establishment to the PCF using both the local configuration and the new indication received from UDM.

- The AMF stores the UE Policy Container, if present, if no UE Policy Association is established during the lifetime of the UE registration and transfer it as part of the UE Context,

- At change of the value indication, the AMF re-evaluates the need of the UE Policy Association using the UE Policy Container stored in the UE Context, the local configuration and the new value of the indication.

Similar enhancements are needed for the AMF to be able to determine whether an establishment of the AM Policy Association is required:

- The Access and Mobility Subscription Data Type is extended with an indication of whether the AM Policy Association is enabled or disabled. The AM Policy Association is enabled if the SUPI subscribes to a service that requires to provision AM Policies to the AMF. The AM Policy Association indication is disabled for SUPIs that are not subscribed to any service that requires sending AM Policies to the AMF. The AMPolicy Association indication is “no control of AM Policies” for SUPIs ensures backward compatibility to enable AMF to use the existing local configuration.

- At initial registration or EPC to 5GS  mobility, the AMF triggers the AM Policy Association Establishment to the PCF using using both the local configuration and the new indication received from UDM.

- At change of the value indication, the AMF re-evaluates the need of the UE Policy Association using the UE Policy Container stored in the UE Context, the local configuration and the new value of the indication.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.502 TS 23.501 | Clause 5.2.3.3.1. Update of the Access and Mobility Subscription Data Subset to indicate if the UE Policy Association is enabled or disabled. Or no control of UE Policies. Similar for AM Policy Association. Clause 4.16.11. Define that the AMF selects the PCF for the UE if the new indication is set to enabled, or if the indication is set to disabled but the local configuration is set to enable. Similar for AM Policy Association in Clause 4.16.1. Clause 4.16.11/13. Define that the AMF establish the UE Policy Association if the indication is enabled and terminates it if disabled. Similar for AM Policy Association in Clause 4.16.1/3. Clause 4.2.2.2.2. Updates to the PCF Discovery and selection during Registriation, the AMF may need to use the PCF Selection Assistance Info to select the same PCF as selected for a PDU Session. | TBD | This will be handled as “TEI19”. |

This TEI19_WID requires 1 TUs

# 6	Work item Rapporteur(s)

Ericsson, Belen Pancorbo

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None identified.

# 9	Supporting Individual Members

Supporting IM name

Ericsson

AT&T

Verizon Wireless

Oracle