3GPP TSG-WG SA2 Meeting #158 	S2-2309077

# Goteborg, Sweden, August 21 – 25, 2023 	(revision of )

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4843 | rev | - | Current version: | 18.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Time-based handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN added the time-based trigger condition to NG HO for NR NTN as described in S2-2308326/R3-233527. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the time-based trigger condition to the principle. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA2 Spec will not match with RAN spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

5.4.11.X	Support for N2 handover for NR satellite access

In case of NG HO for NTN, it is beneficial to signal the handover time window start and duration (as defined by RAN2) to the target node, if so configured by the source node. Handover window start and duration are added to the source to target transparent container.

If the handover request message contains within the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE the Time Based Handover Information IE, the target NG-RAN node shall, if supported, consider that the UE will appear in the target cell within the handover time window indicated.

>>>> End of Changes <<<<