SA WG2 Meeting #158 meeting	S2-2309174

# Aug 21 – 25th, 2023  					                       (revision of S2-22xxxxx)

Source:	China Mobile

Title:	Enhancement on SMF selection for MBS service

Document for:	Approval

Agenda Item:	10.6

Work Item:	TEI19_SSM

Abstract:		TEI19 on SMF selection for MBS service

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: TEI19 on SMF selection for MBS service

Acronym: TEI19_SSM

Unique identifier: to be allocated by MCC

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

{Not applicable for Feature nor for a Study Item}

{For a Building Block: list here the parent Feature }

{For a Work Task: list here the parent Building Block }

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

- Justification

The MBS service is quite useful for saving resource of wireless, core network and transmission, and has explicit commercial requirement in Public Safety, CIoT and V2X. But when the operator introduces the MBS service, the network deployment is progressed step by step. Specially, for the SMF, of which the number is huge, it is almost impossible to upgrade all these SMF at once. One possible approach is to upgrade one/two SMF for each SMF pool. In this case, an upgraded SMF supporting the MBS service should be selected to serve the UE  when a UE request the multicast MBS service.

About the SMF selection for MBS service, the current statement in TS 23.237 is that “the AMF selects an SMF capable of handling Multicast MBS sessions based on DNN and S-NSSAI”,  namely, in order to introduce MBS service, operator needs to configure specific DNN/S-NSSAI for MBS service, which would need operator to reserve the DNN/S-NSSAI resource and invest a huge amount of effort. Therefore, in the early stage of introducing MBS service, it is difficult to select an SMF capable of handling Multicast MBS sessions based on DNN and S-NSSAI.

This work item proposes to enhance the existing mechanism, when registering with NRF, the SMF reports the capability of supporting the MBS service, then when a UE registers and AMF retrieves subscription data from UDM, if the MBS subscription data indicates that the UE is authorized to use Multicast MBS service, no mater whether the UE is requesting the the MBS service, the AMF would select an SMF supporting the MBS service to ensure that the MBS service would be provided successfully, in case the MBS service is requested.

4	Objective

The object of this WID is to study the enhancement of MBS service management:

- Adds subscription information of 5G multicast data in UDM/UDR, which can be pre-configured by operators or dynamically provisioned by AF.

- SMF registers with NRF the capability of supporting MBS service.

- AMF can select an SMF capable of handling MBS sessions based on subscription information.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2: The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Enhancement of SMF discovery and selection | TSG#103 Mar. 2024 |  |
| 23.502 | Enhancement of MBS subscription data | TSG#103 Mar. 2024 |  |
| 23.247 | Enhancement of establishment of a PDU Session that can be associated with multicast session(s) | TSG#103 Mar. 2024 |  |

6	Work item Rapporteur(s)

NA

7	Work item leadership

SA2

8	Aspects that involve other WGs

NA

9	Supporting Individual Members

Supporting IM name

China Mobile