3GPP SA WG2 Meeting #158	S2-2309219

# Goteborg, Sweden,  August 21 - August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4414 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for the Unavailability Period Feature for Dual Registered UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SUECR |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is not specified how to handle the UE’s EMM state when the UE is in an unavailability period. If the UE enters an unavailability period that is longer than the UE’s periodic TAU timer, the UE may be implicitly detached. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a section under “4.11.2 Interworking procedures without N26 interface” to say that, when the UE is dual registered, the AMF may send the Unavailability information to the HSS+UDM. The HSS+UDM may then send the Unavailability information to the MME so that the MME can adjust the UE’s implicit detach timer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE and network behaviour would not be specified for the case where a dual registered UE uses the Unavailability Period Feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

#### 4.11.2.x	Support of the Unavailability Period Feature

If a Registration Request includes an Unavailability Period Duration and/or Start of Unavailability Period and indicates that the UE is in the EMM-REGISTERED state and the AMF is configured to support 5GS-EPS interworking without N26 procedure, the AMF sends an Nudm_UECM_Registration Request message to the HSS+UDM to provide the Unavailability Period Duration and/or Start of Unavailability Period to the HSS+UDM. The HSS+MME sends an Insert Subscriber Data message to the MME to provide the Unavailability Period Duration and/or Start of Unavailability Period. The MME may use the Unavailability Period Duration and/or Start of Unavailability Period to adjust the UE’s Implicit Detach timer such that the MME does not implicitly detach the UE while the UE is unavailable. If the AMF indicated Interworking without N26 interface is supported in the Registration Accept message and the UE’s periodic TAU timer expires while the UE is unavailable, the UE may initiate a Tracking Area Update procedure once the event which makes the UE unavailable is completed in the UE. If the AMF indicated Interworking without N26 interface is not supported in the Registration Accept message and the UE’s periodic TAU timer expires while the UE is unavailable, the UE may consider itself implicitly detached from EPC once the event which makes the UE unavailable is completed in the UE.

*************************** End of Changes ****************************