3GPP TSG-WG SA2 Meeting #158	S2-2309721

# Goteborg, Sweden, August 21 – 25, 2023		(revision of S2-2308615)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0280 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Group Paging for Multicast MBS session data reception in RRC_INACTIVE state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, ZTE, Nokia,CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 38.331 running CR R2-2306855 has been endorsed in RAN2, which records the following agreements: Paging message -- ASN1START -- TAG-PAGING-START …………………… GroupPaging-r18 ::= SEQUENCE { inactiveReceptionAllowed-r18 ENUMERATED {true} OPTIONAL -- Need N } -- TAG-PAGING-STOP -- ASN1STOP As highlighted in green, when the Group Paging is triggered due to the MBS session activation, the NG-RAN nodes need indicate in the radio interface whether UE is allowed to receive the MBS session data in RRC-Inactive state. Hence even the RRC-Inactive UE receives the group paging message, it can be aware that it does not need go back to RRC-Connected state if it have related valid PTM configuration information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update clause 6.17 to clarifiy how NG-RAN notifies the group member UE that it is allowed to be receive in RRC-inactive state during MBS session activation. - Clarify that UE can keep in RRC Inactive state to receive the MBS session data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with the agreement as described in R2-2306855. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.17	Support of Multicast MBS session data reception in UE with RRC_INACTIVE state

To provide multicast MBS service to more UEs in a cell, the NG-RAN may decide to move some UE(s) receiving multicast MBS data from RRC_CONNECTED to RRC_INACTIVE state if the UE(s) is capable to receiving MBS data in RRC_INACTIVE state.

The decision in NG-RAN may use the following information provided from 5GC:

-	Existing MBS session QoS parameters, e.g. the most demanding ARP and 5QI of all MBS QoS Flow within the MBS session.

-	MBS assistance information for the MBS session, the MBS assistance information for the MBS session is an optional parameter and associated with one MBS session, which consists of an indication that the UE is preferred to be kept in connected when the related MBS session that the UE joined is active. When the NG-RAN node receives this information, the NG-RAN may determine to keep the UE in RRC_CONNECTED state even if the MBS session data is supported to be received in RRC_INACTIVE state.

Editor's note:	The MBS assistance information protocol details require RAN WG feedback, e.g. whether the indication is enough.

NOTE 1:	How the NG-RAN nodes perform those decisions is up to NG-RAN implementation.

NOTE 2:	The "RRC Inactive Assistance Information" in clause 5.3.3.2.5 of TS 23.501 [5] is sent by AMF to NG-RAN, whether and how it is used by NG-RAN for deciding whether to send a UE to RRC_INACTIVE state is decided by NG-RAN.

The MBS session QoS parameters (e.g. ARP and 5QI) are provided to NG-RAN by the MB-SMF during user plane establishment for shared delivery.

Per the MBS session that the UE joined, the related "MBS assistance information for the MBS session" is provided to NG-RAN by the SMF if the MBS assistance information is available in the SMF and the MBS session that the UE joined is included in the MBS assistance information. The SMF gets from the UDM the "MBS assistance information", which is provisioned by the AF via the NEF to the UDM as part of the MBS subscription data and includes all the MBS session ID(s), where the UE is preferred to be kept connected when the related MBS session that the UE joined is active (as specified in clause 6.4). The SMF provides the "MBS assistance information for the MBS session" to the NG-RAN as part of the associated PDU session information within the N2 SM information in the procedures where the associated PDU session information need be sent to NG-RAN node, e.g. PDU Session modification for UE joining, handover procedure.

When an MBS session is to be activated, if there are UE(s) that joined the MBS session, the 5GC activates the MBS Session in the NG-RANs serving the joined UE(s). When the NG-RAN triggers the group paging to activate the MBS session and decides to enable Multicast MBS session data reception for UE with RRC_INACTIVE state, it also indicates that the MBS session data is allowed to be received in RRC_INACTIVE state. Hence the joined UEs in RRC_INACTIVE state in the cells, where the delivered MBS session is allowed to be received in RRC_INACTIVE state, is able to stay in RRC_INACTIVE state and receive MBS Session data.

When an UE in RRC_INACTIVE state is receiving ongoing MBS session data, if the UE moves to a new cell within the RNA, or if the UE moves outside the current RNA but within the current Registration Area, or if the UE moves out of the current Registration Area, the UE should be able to receive the MBS session data if applicable in the new area.

NOTE 3:	The scenario of the UE moving to a new cell within the RAN Notification Area is specified in RAN specifications.

# * * * * End of changes * * * *