Draft new Recommendation ITU-T Y. 3159 (ex. Y.IMT-2020-NSL-fra)

Framework for classifying network slice level in future networks including IMT-2020

Summary

The objective of this Recommendation is to specify a framework for classifying network slice level in future networks including IMT-2020. This framework guides the network slice deployment and management. A method for classifying network slice level of future networks including IMT-2020 is introduced.

Keywords

Framework, IMT-2020, isolation, network slice level

Table of Contents

1.	Scope	3

2.	References	3

3.	Definitions	3

3.1.	Terms defined elsewhere	3

3.2.	Terms defined in this Recommendation	4

4.	Abbreviations and acronyms	4

5.	Conventions	5

6.	Overview of framework for classifying network slice level	5

6.1.	Principle for classifying network slice level	5

6.2.	Classification of network slice level	6

7.	Methods for classifying network slice level	9

7.1  Network slicing isolation capability	9

7.1.1 RAN slice isolation solution	9

7.1.2 Transport network slice isolation solution	10

7.1.3 Core network slice isolation solution	11

7.2 Security capabilities of IMT-2020 network slices	12

7.3 Exposure of network management and orchestration capabilities	12

8.	Security considerations	14

Bibliography	14

Draft new Recommendation ITU-T Y. 3159 (ex. Y.IMT-2020-NSL-fra)

Framework for classifying network slice level in future networks including IMT-2020

- Scope

This Recommendation specifies a framework for classifying network slice level in future networks including IMT-2020. This framework guides the network slice deployment and management. This Recommendation also introduces a method for classifying network slice level of future networks including IMT-2020.

- References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.3100]	Recommendation ITU-T Y.3100 Corrigendum 1 (2018), Terms and definitions for IMT-2020 network.

[ITU-T Y.3102]	Recommendation ITU-T Y.3102 (2018), Framework of the IMT-2020 network.

[ITU-T Y.3104]	Recommendation ITU-T Y.3104 (2018), Architecture of the IMT-2020 network

[3GPP TS 23.501]		3GPP TS 23.501 V18.0.0 (2022), System architecture for the 5G System (5GS); Stage 2 (Release 18).

[3GPP TS 28.530]		3GPP TS 28.530 V17.3.0 (2022), Management and orchestration; Concepts, use cases and requirements

- Definitions

  - Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	application domain [b-ITU-T Y.4100]: An area of knowledge or activity applied for one specific economic, commercial, social or administrative scope.

NOTE – Transport application domain, health application domain and government application domain are examples of application domains.

3.1.2	network slice [ITU-T Y.3100]: A logical network that provides specific network capabilities and network characteristics.

NOTE 1 – Network slices enable the creation of customized networks to provide flexible solutions for different market scenarios which have diverse requirements, with respect to functionalities, performance and resource allocation.

NOTE 2 – A network slice may have the ability to expose its capabilities.

NOTE 3 – The behaviour of a network slice is realized via network slice instance(s).

3.1.3	IMT-2020 [ITU-T Y.3100]: Systems, system components, and related technologies that provide far more enhanced capabilities than those described in [b-ITU-R M.1645].

3.1.4	network function [ITU-T Y.3100]: In the context of IMT-2020, a processing function in a network.

NOTE 1 – Network functions include but are not limited to network node functionalities, e.g., session management, mobility management and transport functions, whose functional behaviour and interfaces are defined.

NOTE 2 – Network functions can be implemented on a dedicated hardware or as virtualized software functions.

NOTE 3 – Network functions are not regarded as resources, but rather any network functions can be instantiated using the resources.

3.1.5	orchestration [ITU-T Y.3100]: In the context of IMT-2020, the processes aiming at the automated arrangement, coordination, instantiation and use of network functions and resources for both physical and virtual infrastructures by optimization criteria.

3.1.6	Service level agreement (SLA) [b-ITU-T Y.3106]: A formal agreement between two or more entities reached after a negotiating activity with the scope to assess service characteristics, responsibilities and priorities of every part. A SLA may include statements about performance, tariffing and billing, service delivery and compensations.

NOTE – Definition based on [b-ITU-T E.860].

  - Terms defined in this Recommendation

None

- Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

API 		Application Programming Interface

ASF		Authentication Server Function

eMBB		enhanced Mobile Broadband

E2E		End-to-End

FlexE		Flexible Ethernet

FlexO		Flexible Optical Transport Network

IMT-2020	International Mobile Telecommunication 2020

IoT		Internet of Things

KPI		Key Performance Indicator

MTN 		Metro Transport Network

NF		Network Function

NR		New Radio

NRF		Network Function Registry Function

NSSF		Network Slice Selection Function

OAM		Operation, Administration and Management

OTN		Optical Transport Network

PCF                 Policy Control Function

QoS		Quality of Service

RB		Radio Bear

SLA		Service Level Agreement

SMF		Session Management Function

S-NSSAIs	Single- Network Slice Selection Assistance Information

UE		User Equipment

UPF		User Plane Function

USM		Unified Subscription Management Function

VPN		Virtual Private Network

- Conventions

In this Recommendation:

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted, if conformance to this Recommendation is to be claimed.

- Overview of framework for classifying network slice level

  - Principle for classifying network slice level

Unlike a traditional network that works in "one pipe and best effort" mode, the IMT-2020 network in combination with network slicing aims to provide various end-to-end logical private networks that run on a shared infrastructure and a shared network to cater for the specific requirements of vertical industries. As defined in [3GPP TS 23.501], network slices may differ for supported features and network functions optimisations, in which case such network slices may have e.g. different S-NSSAIs with different slice/service types. The operator can deploy multiple network slices delivering exactly the same features but for different groups of uses, e.g. as they deliver a different committed service and/or because they are dedicated to a customer, in which case such network slices may have e.g. different S-NSSAIs with the same slice/service type but different slice differentiators. Based on analytics of performance indicators, flexible functions, network requirements, and OAM modes, users who uses network slice as a service can be the communication service provider or the communication service customer defined in [3GPP TS 28.530], here for IMT-2020 network can fall into two types:

Public network users:

The services for public network users that traditional networks provided are all retained in IMT-2020 network at a consistent or even better user experience.

Application domain users:

- Application domain users can be divided into general application domain users and special application domain users.

General application domain users have general requirements for service isolation and quality, and may call for differentiation in customization in terms of such as connection management;

Special application domain users who have specific requirements (for example, high isolation or high service quality assurance) demand extremely high security, such as power grids, government, and army.

In accordance with public network users and application domain users, the network architecture forms public networks and application domain networks, where public networks and application domain networks share core network hardware, transport resources, and radio resources as shown in Figure 1. This sharing mechanism gives full play to network scale effect. IoT numbers and public network numbers are used to separate UEs on different networks, and NEs, resources, and base stations can be exclusively used by different networks, enabling flexible architectures and configuration modes.

Figure 1 Architecture of Public networks and Application domain networks

To meet these IMT-2020 network slicing requirements and those concerning isolation, deployment, and operation, two relatively independent IMT-2020 networks slices are available: public network slices, application domain network slices. This Recommendation defines multi-level slices with varying capabilities to meet corresponding requirements of users.

The network functions interact with each other to provide the IMT-2020 network services specified in [ITU-T Y.3102]. Interaction procedures among network functions in order to provide network services are specified in clause 7 of [ITU-T Y.3104].

  - Classification of network slice level

The clause provides the overall classification of network slice level. Depending on the existing network capabilities, the RAN, transport network, core network, security, and operation capabilities are holistically considered for different slice levels to match the most possible network deployment policies. A total of five slice capability levels are provided to meet the two types of IMT-2020 network slicing requirements, as shown in the following table.

Table 1 The classification of network slice level

| Network Slice Level | Title | Type | Definition | Customized Resources | Services Experience |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Isolation | Security | Exposure of network management and orchestration capabilities | Customized service |
| L0 | Public network | General | Based on IMT-2020 public network infrastructure , no special requirements | Fully shared | Basic safety | None | Default |
| L1 |  | High-quality users | Based on IMT-2020 public network infrastructure, superimposed customization requirements | Fully shared | Encryption | None | Differentiation |
| L2 | General application domain | General | Based on IMT-2020 application domain network infrastructure, providing value-added services. | Fully shared | Boundary isolation | Visible | Differentiation |
| L3 |  | High-quality users | Based on the IMT-2020 application domain network infrastructure, it provides exclusive resources and advanced services. | Partially exclusive | Boundary isolation + Encryption | Manageable | Differentiation |
| L4 | Special application domain | Special application domain users | Based on the construction of IMT-2020 application domain private network facilities, it provides exclusive resources and reliability services for all resources. | Completely independent | Advanced Security | Manageable | Differentiation |

- Slice level:

- Five slice levels, including L0 to L4, are defined.

- Network type:

- Two network types, including public network and application domain network.

- Level classification:

Public networks have two levels: general and high-quality users. Application domain networks have three levels: general, high-quality users, and special.

- Slice level definition:

It defines each slice level.

- Resource customization:

Three options are available, including fully shared, partially exclusive, and completely independent. In initial construction, public networks and application domain networks are separated, delivering original isolation. Resource customization refers to the degree of resource isolation among the RAN, transport network, and core network.

- Service experience:

- Security:

Five options are available, including basic security, enhanced Mobile Broadband (eMBB) enhanced security, service feature security, high-level service feature security, and high-level security.

- Exposure of network management and orchestration capabilities:

Three options are available, including no need for exposure, application domain visualization for exposure information, and application domain manageability for exposure information.

- Customized service:

Except general users on public networks that have no special requirements, L1 to L4 require different assurance capabilities which are to be tailored to specific scenarios and requirements.

In addition to the aforementioned basic network capabilities (network resources, isolation, exposure, security, and customized services), different vertical industries have their own customization requirements. Network slicing needs to provide customization capabilities, which can be found as the attributes of network slices/services in [b-GSMA NG.116], such as support for non-IP transmission, clock synchronization, and high-speed device processing, and such capabilities can be added to each of capability levels.

- Methods for classifying network slice level

This clause provides detailed solutions to the aforementioned five capability levels in terms of RAN, transport network, core network, and management system in the preceding dimensions.

## 7.1  Network slicing isolation capability

### 7.1.1 RAN slice isolation solution

The table 2 provides the classification of RAN slice level. The RAN slice isolation solution aims to isolate and guarantee resources for network slices on RAN. Slice-specific QoS assurance, air interface dynamic radio bear (RB) resource sharing, and static RB resource reservation are available, differentiating in service latency, reliability, and isolation requirements.

NOTE: The specific technology of RAN is out of scope of this Recommendation.

Table 2 Suggestions on RAN slice levels

| Network Slice Level | Title | Type | RAN Slice Capability | Application domain |
| --- | --- | --- | --- | --- |
| L0 | Public network | General | Capability 1: RB resource sharing and QoS-specific slice | Enterprise broadband |
| L1 |  | High-quality users |  |  |
| L2 | General application domain | General |  |  |
| L3 |  | High-quality users | Capability 2: partially reserved RB resources and dynamically shared slices Or Capability 3: resource-exclusive slice | For Capability 2：Basic service assurance is required. Example services include smart grid inspection and media live broadcast. For Capability 3： High requirements are posed on service isolation and bandwidth. Examples include power grid distribution automation, government, and public security private networks. |
| L4 | Special application domain | Special application domain users |  |  |

### 7.1.2 Transport network slice isolation solution

The table 3 provides the classification of TN slice level. The isolation on transport networks between RANs and CNs can be hard isolation or soft isolation, depending on latency, slice security and reliability requirements. The isolation technology can be i) physical isolation: metro transport network (MTN) interface isolation adapting Flex Ethernet (FlexE) [b-ITU-T G.8310][b-ITU-T G.8312], optical transport network (OTN) isolation including Flex OTN (FlexO) in G.709 series [b-ITU-T G.709], or ii) soft isolation: MPLS in G.8100 series [b-ITU-T G.8110]/VPN+QoS [b-ITU-T Y.1311.1] isolation.

Table 3 Suggestions on TN slice levels

| Slice Level | Title | Type | TN Slice Capability | Application domain |
| --- | --- | --- | --- | --- |
| L0 | Public network | General | softwarized sharing | Internet access and OTT video |
| L1 |  | High-quality users | softwarized sharing +physical isolation (tunnel isolation, Gbps-level services) | Cloud gaming, home cloud VR, and application domain applications such as mobile rescue, drones, and mobile surveillance |
| L2 | General application domain | General | soft isolation +physical isolation (physical isolation, tunnel isolation, Gbps-level services) | Power grid, manufacturing, healthcare, mining, port, and IoV |
| L3 |  | High-quality users | Partial physical isolation | Government and enterprise private line, meter reading and collection, video surveillance, and live broadcast |
| L4 | Special application domain | Special application domain users | E2E physical isolation | Private lines for government, finance, securities, and power grid customers |

### 7.1.3 Core network slice isolation solution

The table 4 provides the classification of CN slice level. The core network slice isolation solution helps isolate core network slice resources from the networking and guarantee the customized services. The resource view mainly includes the hardware resource layer, virtual resource layer, and NF layer allocated for slice isolation.

- Hardware resource layer：It mainly refers to a variety of servers, which can support both "shared" and "dedicated" isolation modes.

- Virtual resource layer: The communication functions are carried on the general hardware through virtual machine, container and other virtualization technologies, which can realize rapid development, deployment and flexible scaling up of new business. Virtual resource layer can also support both "shared" and "dedicated" isolation modes.

- NF layer: The network function/virtual network function layer of the core network can support the on-demand isolation mode at different levels to ensure the independence of different slices.

Table 4 Suggestions on CN slice levels

| Slice Level | Title | Type | CN Slice Capability | Application domain |
| --- | --- | --- | --- | --- |
| L0 | Public network | General | NF layer, virtual resource layer and hardware resource layer shared | Internet access and OTT video |
| L1 |  | High-quality users | NF layer, virtual resource layer and hardware resource layer shared or partially shared | Cloud gaming, home cloud VR, and application domain applications such as mobile rescue, drones, and mobile surveillance |
| L2 | General application domain | General | NF layer, virtual resource layer and hardware resource layer shared or partially shared | Power grid, manufacturing, healthcare, mining, port, and IoV |
| L3 |  | High-quality users | NF layer, virtual resource layer and hardware resource layer shared or partially shared | Government and enterprise private line, meter reading and collection, video surveillance, and live broadcast |
| L4 | Special application domain | Special application domain users | NF layer, virtual resource layer and hardware resource layer isolation | Private lines for government, finance, securities, and power grid customers |

## 7.2 Security capabilities of IMT-2020 network slices

To support E2E security protection for different services, flexible security architecture is required to provide multi-level slice security assurance. When vertical application domain users have specific security requirements, they can request customized network slices with different security protection levels from carriers. Differentiated security capabilities include security management capabilities, security capabilities of network protocols and network device resource security capabilities.

NOTE: The specific technology of security capabilities of IMT-2020 network slices is out of scope of this Recommendation.

## 7.3 Exposure of network management and orchestration capabilities

The table 5 provides the classification of orchestration and management levels for exposure. As defined in Y.3108, in terms of interaction with CEF, the network management and orchestration capabilities [ITU-T Y.3110] expose a set of management data required by the customer and authorized by the network operator. For example, network operation status and current network performance can be exposed to third parties through CEF within operator’s policy.

The exposure of network management and orchestration capabilities for network slices can be divided into several scenarios based on the operator’s policy.

Table 5 Suggestions on orchestration and management levels for exposure

| Slice Level | Title | Type | Degree of Capability Openness | Visible for exposure information | Manageable for exposure information |
| --- | --- | --- | --- | --- | --- |
| L0 | Public network | General | None | None | Managed by operator. |
| L1 |  | High-quality users | None | None | Managed by operator. |
| L2 | General application domain | General | User Portal ( KPI visualization ) | Users can view slice status, UE information, bills and obtain SLA reports through the network slice provider’s Portal. | Managed by operator. |
| L3 |  | High-quality users | User Portal (KPI visualization + self-service business ) | Users can view slice status, UE information, bills and obtain SLA reports through the network slice provider’s Portal. | It can be implemented through self-service Portal: - Business subscription, modification, and termination, etc. - UE lifecycle management. Such as UE account opening and cancellation, shutdown and recovery, etc. - Simple failure diagnosis. Such as real-time diagnosis, diagnosis based on historical information correlation (non-real-time diagnosis), location services, etc. |
| L4 | Special application domain | Special application domain users | Open API capabilities ( KPI visualization + self-service business ) | Users implement slice business monitoring using APP through open API. | Users implement slice business management using APP through open API. |

- Security considerations

This Recommendation defines methods for classifying network slice level, including slice isolation solution for RAN, Transport and core network, the security capabilities, OAM modes. Thus, it is assumed that security considerations in general are based on the security of IMT 2020 network management and orchestration [ITU-T Y.3111].

Bibliography

[b-ITU-T G.709]	Recommendation ITU-T G.709 (2017) Amendment 3 (2021), Interfaces for the optical transport network.

[b-ITU-T G.8110]	Recommendation ITU-T G.8110/Y.1370 (2005), MPLS layer network architecture.

[b-ITU-T G.8310]	Recommendation ITU-T G.8310 (2020), Architecture of the metro transport network.

[b-ITU-T G.8312]	Recommendation ITU-T G.8312 (2021), Interface for the metro transport network.

[b-ITU-T Y.1311.1]	Recommendation ITU-T Y.1311.1 (2001), Network-based IP VPN over MPLS architecture.

[b-ITU-T E.860]		Recommendation ITU-T E.860 (2002), Framework of a service level agreement.

[b-ITU-T Y.3106]		Recommendation ITU-T Y.3106 (2019), Quality of service functional requirements for the IMT-2020 network

[b-ITU-T Y.4100]		Recommendation ITU-T Y.4100/Y.2066 (2014), General requirements of the Internet of things.

[b-GSMA NG.116] 		GSMA NG.116 v3.0 (2020), Generic Network Slice Template

_______________________