3GPP TSG-WG SA2 Meeting #158 	S2-2308455

# Goteborg, Sweden, August 21 – 25, 2023	(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4263 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Counting of UEs with at least one PDU Session/Connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNSAC |  |  |  |  |  | Date: |  |  | 2023-07-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR corrects mistakes for the handling by NSACF configured for the number of UEs with at least one PDU Session/PDU Connection for a slice subject to EPC interworking when the UE initiates multiple PDU Sessions/PDU connections from different accesses and when the UE tears down one connection in case the UE has multiple PDU Sessions/PDN connections from different accesses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement the following logic associated with the update flag values of increase, decrease or update: - When the UE has already a PDU Session/PDN Connection established and initiates an additional PDU Session/Connection from a second access only a new entry is added but the count is not increased. - When the UE has multiple entries associated with different accesses and tears down one PDU Session/PDN Connection, the count is not decreased but rather the entry associated with the access is deleted. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.5.9a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

#### 4.11.5.9a	Network Slice Admission Control in 5GS for maximum number of UE with at least one PDU session and one PDN connection

Support of NSAC in conjunction with interworking with EPC for maximum number of UE with at least one PDU session/PDN connection is described in clause 5.15.11.5a of TS 23.501 [2].

For deployment option 1 of two separate NSACF, Same mechanisms in clause 4.2.11.2 is used with the following additions:

-	The SMF+PGW-C invokes this procedure when the UE establishes the first PDU Session/PDN Connection from a new access and for every access when the last PDU Session/PDN Connection associated with the network slice is released. In this case, the AMF in figure 4.2.11.2-1 is replaced with SMF+PGW-C.

-	Step 2: The SMF+PGW-C includes in the message the S-NSSAI, identity of SMF+PGW-C, UE ID and update flag. The update flag may include either ‘increase’, ‘decrease’                     or ‘update’ values.

-	Step 2: The AMF ID is replaced by the SMF+PGW-C ID.

-	Step 3: The ‘update’ value indicates that for existing UE registration the Access Type is to be replaced with a new Access Type during inter access mobility.

-	If the update flag parameter from the SMF+PGW-C indicates update value, the NSACF locates the existing entry with UE ID and NF ID and replaces the Access Type in the existing entry without increasing the count.

-	If the update flag from the SMF+PGW-C indicates increase value, and an entry did not exist, the NSACF creates a UE entry only if the maximum number of Registered UEs with at least one PDU Session/PDU Connection has not been  reached and increases the number of UE registered with at least one PDU Session/PDN Connection.  If an entry existed,  the NSACF stores the NF ID and the Access Type as a new entry keeping the existing NF ID and Access Type  as is. The NSACF does not increase the number of UE with at least one PDU session/PDN connection for that S-NSSAI.

-	If the update flag parameter from the SMF+PGW-C anchoring the PDU session/PDN connections  indicates decrease and if there is only one entry associated with the UE ID, the NSACF removes the UE ID from the list of UEs registered with the network slice for each of the S-NSSAI(s) indicated in the request from the SMF+PGW-C and decreases the number of UEs per network slice that is maintained by the NSACF for each of these network slices. If there are multiple entries associated with the UE ID, the NSACF removes the entry associated with the NF ID but the UE ID is kept in the list of UEs registered with the S-NSSAI. The NSACF does not decrease the count for number of UEs with at least one PDU session/PDN connection.

-    The NSACF determines whether or not to accept the request as described in clause 5.15.11.5a of TS 23.501 [2].

NOTE 1:	EAC mode is not applicable here.

For deployment option 2 of single NSACF, same mechanisms in clause 4.2.11.4 are used with the following additions:

-	The NSACF also counts the maximum number UE with at least one PDU session/PDN connection based on the update value received from SMF+PGW-C.

-	If the update flag parameter from the SMF+PGW-C anchoring the PDU session/PDN connections indicates increase value and the maximum number of UE with at least one PDU session/PDN connection has already been reached, then the NSACF returns a result parameter indicating that the maximum number of PDU Sessions per network slice has been reached. If the maximum number of UE with at least one PDU session/PDN connection has not been reached, the NSACF checks the UE ID. If the UE ID is located, the NSACF stores the NF ID and the Access Type as a new entry keeping the existing NF ID and Access Type  as is. The NSACF does not increase the number of UE with at least one PDU session/PDN connection for that S-NSSAI. If the NSACF did not locate the UE ID, the NSACF creates an entry for the UE ID, stores the NF ID and Access Type and increases the number of PDU Sessions for that S-NSSAI.

-	If the update flag parameter from the SMF+PGW-C anchoring the PDU session/PDN connections  indicates decrease and if there is only one entry associated with the UE ID, the NSACF removes the UE ID from the list of UEs registered with the network slice for each of the S-NSSAI(s) indicated in the request from the SMF+PGW-C and decreases the number of UEs per network slice that is maintained by the NSACF for each of these network slices. If there are multiple entries associated with the UE ID, the NSACF removes the entry associated with the NF ID but the UE ID is kept in the list of UEs registered with the S-NSSAI. The NSACF does not decrease the count for number of UEs with at least one PDU session/PDN connection.

-	If the update flag parameter from the SMF/SMF+PGW-C anchoring the PDU session/PDN connections indicates update, the NSACF locates the existing entry with UE ID and NF ID and replaces the Access Type in the existing entry.   NSACF will not increase the count again.

-    The NSACF determines whether to accept the request as described in clause 5.15.11.5a of TS 23.501 [2].

-	The NSACF performs other actions in addition to the above  to keep track of the last PDU Session/Connection released so as to decrease the count for the number of Registered UEs with at least one PDU Session/one PDN Connection.

NOTE 1:	EAC mode is not applicable here.

## END of CHANGES