3GPP TSG-WG SA2 Meeting #158	S2-2308537

Goteborg, Sweden, Aug 21 – 25, 2023

Title:	[DRAFT] Reply LS on network slice replacement procedure

Response to:	LS (S4-231048/S2-2308338) on network slice replacement procedure from SA4

Release:	Rel-18

Work Item:	eNS_Ph3

Source:	[Huawei to be] SA2

To:	SA4

Cc:

Contact Person:

Name:	Haiyang Sun

Tel. Number:

E-mail Address:	sunhaiyang3@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.503 CR 1095

1. Overall Description:

SA2 would like to thank SA4 for their network slice replacement procedure.

Specific to the SA4 questions in S2-2308338/S4-231048, SA2 would like to provide the following feedback:

Question #1: Can SA2 confirm that the Alternative network slice provides similar performance as that of the original network slice that it replaces?

Answer:  The alternative network slice should support minimum requirements for the services that the original network slice supports.

Question #2: Is the network slice replacement procedure transparent to the UE application?

Answer: Yes. The UE application is not aware of the Alternative S-NSSAI.

For existing PDU Session associated with an S-NSSAI that is replaced with the Alternative S-NSSAI, if the SMF determines that the PDU Session needs to be re-established, the SMF will trigger the re-establishment of the PDU Session. However, the UE application doesn’t know the re-establishment of the PDU Session is due to network slice replacement.

Question #3: Are the Application Function and the Application Provider notified when the network slice replacement procedure is invoked by the 5G System?

Answer: Based on the descriptions in clause 6.1.3.18 of TS 23.503, if the AF subscribes to notifications of events from the PCF when the PDU Session is transferred from S-NSSAI to Alternative S-NSSAI, the PCF reports the Alternative S-NSSAI to the AF. Further clarifications are added as in the attachment.

SA2 specifications do not define or refer to Application Provider.

Question #4: Is the Application Provider made aware of the Alternative network slice information prior to the network slice replacement procedure being invoked by the 5G System (e.g., through OAM)?

Answer:  SA2 specifications do not define or refer to Application Provider. There is no procedure defined to notify the AF before network slice replacement procedure.

2. Actions:

To SA4 group.

ACTION: 	SA2 asks SA4 group to take the above feedback into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #159		October 09 – 13, 2023			Xiamen, CN

TSG-SA2 Meeting #160		November 13 – 17, 2023			Chicago, USA