3GPP SA WG2 Meeting #159	S2-2308684

Goteborg, Aug 21 – 25 2023	(revision of )

# Source:	Nokia, Nokia Shanghai Bell, AT&T

# Title:	Non Home Network Data Services

# Document for:	Information

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Non Home Network Data Services

Acronym:	TEI19_NOHODS

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

Current release of the specification enables the PDU Session to be served by the home network (i.e. non roaming or home routed roaming) or by the visited network (i.e. local breakout). Decision of the approach implemented is based on operator policies, negotiations and UE subscription data.

Using any of the above scenarios, the UE can establish a PDU Session and can access the services provided by the specific DNN and S-NSSAI in accordance with the operator defined policies.

In all these cases, the serving entities for that particular DNN are either in the HPLMN or VPLMN based on where the UE is registered, the policies and the UE subscription information. The user plane (N6) is termintated at V-PLMN if it is LBO or at the H-PLMN if this is home routed roaming or non roaming scenario.

Now, there are certain use cases that require an operator to have PDU sessions for certain DNN services to be controlled by another partner PLMN network (user plane N6 termination is at the partner PLMN). Current implementations have N6 termination only at home network or at the VPLMN.

Therefore, it is seen important to introduce a mechanism that enables an operator to provide PDU Session control and N6 termination to a partner network irrespective of the user is registered in the partner network, home network or roaming in a different VPLMN.

It is also required that such kind of service that needs N6 termination from the partner network can be available based on the specific UE subscription.

Figure 1: Subscriber is registered in the HPLMN

Figure 2: Subscriber is registered in the VPLMN

# 4	Objective

The objective of this work item is to enhance the PDU session establishment procedures, enabling the UE to establish PDU session with the partner network irrespective of where the UE has registered to the network. This means irrespective of the user being registered in the HPLMN or VPLMN, based on the subscription, the UE shall establish the PDU session at the partner network (Non Home Network Services).

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.502 | Enhance UE subscription data in UDM to also contain an indication per subscribed DNN indicating whether routing of PDU Session to a partner PLMN is required and the corresponding partner PLMN ID. Specify a new PDU session establishment procedure based on 4.3.2.2.2 to enable the UE to establish a PDU session that is served by the partner PLMN instead of the HPLMN. Specify potential changes needed for discovery of SMF from partner PLMN, potential changes in interaction between (V-)AMF and (V-)SMF, between (V-)SMF and partner PLMN SMF and potential interaction needed between the partner PLMN SMF and the UDM, H-PCF. | TBD | This will be handled as “TEI19”. |
| TS 23.501 | Potential updates to brief the use case scenario. | TBD | This will be handled as “TEI19”. |

This TEI19_WID requires 2 TUs

# 6	Work item Rapporteur(s)

Nokia, Nokia Shanghai Bell, Srinivas.Garikipati@Nokia.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None identified.

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

AT&T