3GPP TSG-SA2 Meeting #158	S2-2308758

# Gothenburg,Sweden, 21st-25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0877 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MLModelManagement_Delete service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 08-11-2023 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR provides correction in clause 10.3.3 to align with the description in clause 6.2B.6 on ML Model removal from ADRF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction in clause 10.3.3 and editorial fixes for consistent specificiation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Correction for alignment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1, 10.3.1, 10.3.2, 10.3.3, 10.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

## 10.1	General

Table 10.1-1 shows the ADRF services and ADRF service operations.

ADRF service operations may be used to store data or analytics in the ADRF, retrieve data or analytics from an ADRF, or delete data or analytics from an ADRF.

ADRF service operations may also be used to store ML Model(s) or ML Model address(es) in the ADRF or delete ML Model(s) from an ADRF.

Table 10.1-1: NF services provided by ADRF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadrf_DataManagement | StorageRequest | Request / Response | DCCF, NWDAF, MFAF |
|  | StorageSubscriptionRequest | Request / Response | DCCF, NWDAF |
|  | StorageSubscriptionRemoval | Request / Response | DCCF, NWDAF |
|  | RetrievalRequest | Request / Response | DCCF, NWDAF |
|  | RetrievalSubscribe | Subscribe / Notify | DCCF, NWDAF |
|  | RetrievalUnsubscribe |  | DCCF, NWDAF |
|  | RetrievalNotify |  | DCCF, NWDAF |
|  | Delete | Request / Response | DCCF, NWDAF |
| Nadrf_MLModelManagement | StorageRequest | Request / Response | NWDAF |
|  | Delete | Request / Response | NWDAF |

* * * * Next Change * * * *

## 10.3	Nadrf_MLModelManagement service

### 10.3.1	General

Service Description: This service enables the consumer to store and update and remove ML Model(s) or ML Model address(es) from an ADRF.

### 10.3.2	Nadrf_MLModelManagement_StorageRequest service operation

Service operation name: Nadrf_MLModelManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store or update ML Model(s). ML Model(s) or ML Model address(es) stored in NWDAF containing MTLF are provided to the ADRF in the request message.

Inputs, Required: NF instance ID of the NWDAF containing MTLF and set of:

-	the tuple (unique ML Model identifier and address (e.g. URL or FQDN) of Model file and Storage size required for each of the ML Model).

NOTE:	The ADRF can download ML Model files based on the ML Model addresses provided by the NWDAF containing MTLF. How the ADRF downloads the ML Model and locally stores the ML Model is left for implementation.

Inputs, Optional: None.

Outputs Required: Result Indication, one of the following:

-	one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of Model file stored in ADRF.

Outputs, Optional: Storage Transaction Identifier.

### 10.3.3	Nadrf_MLModelManagement_Delete service operation

Service operation name: Nadrf_MLModelManagement_Delete

Description: This service operation instructs the ADRF to delete stored ML Model file(s) or ML Model address(es).

Inputs, Required:

-	Storage Transaction Identifier; or

-	one or more unique ML Model identifier(s).

When a Storage Transaction Identifier is given, ADRF shall delete all the models that stored in the corresponding storage transaction.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication (i.e. ML Model deleted, ML Model not found, ML Model found but not deleted).

Outputs, Optional: None.

### 10.3.4	Nadrf_MLModelManagement_Retrieval Request service operation

Service operation name: Nadrf_MLModelManagement_Retrieval Request

Description: The consumer NF uses this service operation to request the ADRF to get the ML Model(s) stored in ADRF.

Inputs, Required:

-	Storage Transaction Identifier; or

-	one or more tuples of unique ML Model identifier(s).

Inputs, Optional: None.

Outputs Required:

-	one or more tuples of unique ML Model identifiers and address (e.g. URL or FQDN) of Model file stored in ADRF.

Outputs, Optional: None.

* * * * End of Change * * * *