3GPP TSG-WG SA2 Meeting #158	S2-2308778

# Gothenburg Aug 21 – Aug 25, 2023		(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4784 | rev | - | Current version: | 18.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Method for exchange info to determine SNPN selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. In Annex N.6, there is one editor’s note regarding how localized service provider or SNPN operator, who is providing access to localized service, provides information to the home network operator (PLMN or subscribed SNPN), so that the home network operator can generate the corresponding Credentials Holder controlled priority lists for localized services. Editor's note: The exchange between the Localised Service Provider or the SNPN providing access to localized service and the HPLMN or the subscribed SNPN acting as Credentials Holder is FFS. The SNPN selection information is provisioned to the UE via SOR procedure, and there are established procedures in TS 23.122 regarding how the UDM obtains the priority lists for SNPN selection. But the creation of such network selection priority lists (i.e. by UDR, or SoR-AF) is proposed to not be included the scope of 3GPP, as home network operator may need to consider many different critieria to determine the distribution of the end users which may not be constant all the time i.e. it is not as simple as assumed by previous proposals and would require to be studied separately and that has not been done. Another aspect to consider is that whatever input the LSP or hosting network provides to the home network would be just another input for the SoR-AF to generate the priority lists dynamically, while the functional description of SoR-AF and how SoR-AF obtains such input is not in 3GPP scope. So it is proposed to change the editor’s note to a note and stating that the input for creating the network priority list is out of 3GPP scope. 2. The information provided by localized service provider or the SNPN operator providing access to localized service should also include the GIN which may be broadcasted by the hosting network SNPN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Remove the editor’s note and propose it is out of scope how home network operator obtains input for creating network selection list. 2. Add GIN as part of the information provided by localized service provider or SNPN operator who providing access to localized service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The editor’s note remains unresolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex N.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## N.6	Configuration of Credentials Holder for determining SNPN selection information

To enable the HPLMN or the subscribed SNPN acting as Credentials Holder to generate and provision UEs with SNPN selection information for discovery and selection of SNPNs providing localised services, based on service agreement between the Localised Service Provider or the SNPN providing localized services and the HPLMN or the subscribed SNPN acting as Credentials Holder, the Localised Service Provider or the SNPN providing access to localized services can provide configuration information for SNPN selection to the HPLMN or the subscribed SNPN acting as Credentials Holder. The configuration information for SNPN selection may contain at least one of the following parameters:

a.	SNPN ID or GIN of the SNPN providing access to one or more localized services;

b.	Identification of each localized service;

c.	validity information for each localized service, e.g. the validity time information or/and location assistance information; and/or

d.	List of UE IDs (e.g. GPSIs or External Group ID) identifying the UEs subscribed with a localized service.

NOTE:	How HPLMN or subscribed SNPN obtains the information above is out of 3GPP scope.

The operator of the HPLMN or the subscribed SNPN acting as Credentials Holder then may use the information received from the SNPN providing localised services and/or Localized Service Provider to create or update the Credentials Holder controlled prioritized lists of preferred SNPNs/GINs for accessing Localized Services and provision the UEs using the Steering of Roaming procedure as defined in TS 23.122 [17].

# * * * * End of changes * * * *