3GPP SA WG2 Meeting #158	S2-2308801

21-25 Aug 2023, Goteborg, Sweden                  	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1339 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on IMS Data Channel Service Subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause AC.3: The Service Subscription for IMS DC and the Data Channel subscription information is describied as following: "Service Subscription for IMS Data Channel shall be an extension to the MMTEL service profile in HSS. The Data Channel subscription information shall be used by the IMS AS during IMS session initiation or update to authorize subscribers to use the DC service." There is no MMTEL service profile definition in TS 23.228; and according to discussion, the Data Channel session also can be separated with MMTEL session in the future releases. So it is proposed to extend IMS Service Profile to include Service Subscription for IMS DC. The IMS Service Profile is described in Clause 4.3.3.4 in TS 23.228: "The IMS Service Profile is a collection of service and user related data as defined in TS 29.228 [30]." Regarding DC subscription information, it has the same meaning with DC subscription data. Since subscription data is used in other related clauses AC.7.1, AC.7.2.1 and AC.7.2.2. It is proposed to use subscription data to align with the term used in other clauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify MMTEL service profile to IMS Service Profile; modify subscription information to subscription data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MMTEL service profile is incorrect; subscription information is not aligned with subscripion data used in other clauses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AC.3	IMS Data Channel Service Subscription

Service subscription for IMS Data Channel shall be an extension to the IMS Service Profile in HSS. The Data Channel subscription data shall be used by the IMS AS during IMS session initiation or update to authorize subscribers to use the DC service.

IMS AS Service specific data is enhanced with DC specific service data, optionally stored in HSS (e.g. as repository data) and retrieved by IMS AS using N71 interface.

DCSF service specific data used by DCSF for data channel management, may be stored and retrieved from HSS using the N72 interface. DCSF service specific data are out of scope of 3GPP.

END OF CHANGES