SA WG2 Meeting #158	S2-2308824

Goteborg, Sweden, August 21 – 25, 2023	(revision of S2-2300xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4339 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF provides service experience type to NEF for member selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to section 4.15.13.5.2, the NEF may use the Service Experience type provided by the AF consumer in the filtering information to filter the target UE. But, it is unclear how to the NEF to use this Service Experience type to filter the UE. Firstly, the Service Experience type should be associated with each UE in the candidate list. And then, the NEF can consume the filter information to filter the target UE. So, in the data analytics from NWDAF, i.e. the observed Service Experience, the Service Experience type (e.g.: AIML traffic) should be provided to NEF, and the NEF use the filter information in AF to filter the target UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the sentence of provision Service Experience type associated with each UEs, and clarify how to NEF to use the filter information to filter the target UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to NEF to filter the target UE by Service Experience type is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

4.15.13.5.2	Member UE Assistance with Service Experience filtering criteria

Figure 4.15.13.5.2-1: 5GC assistance to Member UE selection based on Service Experience

1.	AF subscribes the Member UE selection assistance functionality by sending Nnef_MemberUESelectionAssistance_subscribe request including the Application Identity, AoI, DNAI(s), Service Experience Type and contribution weights associated to location, time window, Application ID and Service Experience type.

2.	NEF verifies the authorization of the AF Request and identifies which information needs to be collected and executed based on the Service Experience filtering criteria provided by the AF.

3.	If an Application ID is present in the request from the AF, the NEF derives the S-NSSAI, DNN and DNAI(s) to which this Application ID have access. Due to the absence of a time window in the request from the AF, NEF discovers querying UDM and NRF the SMFs that are deployed in the Area of Interest. The NEF may also restrict the discovery to those SMFs that serve some S-NSSAI and DNN combination (this S-NSSAI and DNN combination has been derived from the Application Id or AF Id).

4.	NEF sends an Analytics request/subscribe to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe, including Analytics ID = Service Experience, Application ID, S-NSSAI, AoI, DNAI(s), and target UEs based on the initial list obtained from the AF.

5-7.	Procedures as specified in clause 6.4.6 of TS 23.288 [50] are followed.

8.	The NWDAF provides the data analytics, i.e. the observed Service Experience (which can be a range of values) to the consumer NF by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 4.

9.	Based on the Analytics report received from the NWDAF, NEF consolidates results and derives the list(s) of candidate UE(s). The NEF may receive the analytic data from NWDAF in step 8 including the Service Experience Type (e.g. AI/ML traffic) associated with each UE. By comparing the Service Experience Type corresponding to each UE provided by NWDAF with the filtering information provided by AF, the target UEs are filtered out from the candidate list of UEs by NEF. The NEF may use the Service Experience type provided by the AF consumer in the filtering information and use operator policies to interpret a customized MoS. The NEF may use the contribution weight associated to an application and a Service Experience Type (e.g. AI/ML traffic) and apply them to a location and time window, as provided by the AF consumer, to be used as reporting thresholds when selecting candidate Member UEs, e.g. the NEF may select a UE with a specific Service Experience Type as a Member UE candidate, if the associated Service Experience fulfils the threshold for Service Experience filtering criteria provided by the AF.

10.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and additional information.

* * * End of Change * * *