SA WG2 Meeting #S2-158	S2-2308950

21 - 25 August, 2023, Goteborg, Sweden	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0021 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the UE Ranging SL Positioning capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the clause 4.3.1, the UE Ranging SL positoning capability includes Capability of supporting Ranging/SL Positioning over PC5 and Capability of supporting SL Positioning Server UE over PC5. In the 24.501, the SLPSPC5 and RSPPC5 are separate IE and not part of 5G ProSe Capability/V2X capability in the "5GMM capability". In the N1 interface, the UE covery the UE Ranging/SL Positioning capability to AMF, there is no separate PC5 Capability. During the registration proceudre, AMF may include Ranging/SL Positioning Support indicator in the Registration Accept message to indicate that the serving network supports Ranging/SL Positioning. The related description is missing in the UE and AMF functionality and Registration procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Calrifying that UE Ranging/SL Positioning Capability is in 5GMM capability. During registration, the AMF may include Ranging/SL Positioning Support indicator in the Registration Accept. UE Ranging/SL Positioning Capability contain Capability of supporting SL Positioning Server UE over PC5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE Ranging SL Positioning capability description is not align. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4, 4.3.1, 4.3.3, 5.2.3, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 4.2.4	Reference points

SR1:	The reference point between the Sidelink (SL) Positioning and Ranging function in the UE and the SL Positioning/Ranging Server. This reference point is out of scope of this specification. It may be used for the configuration and application layer signalling.

SR5:	The reference point between the Sidelink (SL) Positioning and Ranging function in UEs. It is carried over PC5-U of the PC5 reference point.

PC5:	The reference point between the UEs. It also supports the Sidelink Positioning and Ranging operation defined by RAN WGs.

N1:	In addition to the relevant functions defined in TS 23.501 [2] for N1, in the case of Ranging/SL Positioning Service, it is also used to convey the Ranging/SL Positioning policy and parameters (including service authorization) from AMF to UE, and to convey the UE's Ranging/SL Positioning Capability defined in clause 4.3.1 from UE to AMF.

N2:	In addition to the relevant functions defined in TS 23.501 [2] for N2, in the case of Ranging/SL Positioning Service it is also used to convey the Ranging/SL Positioning policy and parameters (including service authorization) from AMF to NG-RAN.

Next CHANGE

### 4.3.1	UE

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the UE may support the following functions:

-	Reporting the following Ranging/SL Positioning related capabilities to 5GC over the N1 reference point:

-	Capability of supporting Ranging/SL Positioning over PC5;

NOTE:	Based on Ranging/SL Positioning control, a UE capable of Ranging/SL Positioning may take different roles in the operation, e.g. Target UE, Reference UE, Located UE.

-	Capability of supporting SL Positioning Server UE over PC5.

-	Procedures for Ranging/SL Positioning over PC5.

-	Procedures to Network based SL Positioning, and Network assisted SL Positioning;

-	Procedures to Ranging/SL Positioning service exposure.

-	Indicating UE Policy Provisioning Request in UE Policy Container for UE triggered Ranging/SL Positioning Policy provisioning, which requests one or multiple types of policies/parameters as listed below:

-	Policy/parameters for Ranging/SL Positioning over PC5;

-	Policy/parameters for Located UE;

-	Policy/parameters for Target UE in addition to the functions defined in TS 23.273 [8] clause 4.3.5;

-	Policy/parameters for SL Positioning Client UE;

-	Policy/parameters for SL Positioning Server UE.

-	Receiving the Ranging/SL Positioning Policy from 5GC over N1 reference point.

-	Configuration of parameters for Ranging/SL Positioning over PC5. These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over SR1 reference point from the Ranging/SL Positioning Application Server.

-	Receiving the serving network supporting Ranging/SL Positioning in Registration Accept.

Next CHANGE

### 4.3.3	AMF

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the AMF performs the following functions:

-	Select a PCF supporting Ranging/SL positioning Policy/Parameter provisioning based on indication of Ranging/SL positioning Capability in the "5GMM capability" in the Registration Request.

-	Store the UE Ranging/SL positioning Capability.

-	Select the LMF supporting Ranging/SL positioning and forward the Ranging/SL positioning Capability of Target UE to LMF.

-	Select a GMLC supporting Ranging/SL positioning and request the GMLC for Ranging/SL Positioning result.

-	Forward the Ranging/SL positioning Capability to PCF.

-	Obtain from UDM the subscription information related to Ranging/SL positioning and store them as part of the UE context data.

-	Provision the NG-RAN with indication about the UE authorization status about Ranging/SL Positioning over PC5 (i.e. as Ranging/SL positioning-enabled UE).

-	Provision the NG-RAN with PC5 QoS parameters related to RSPP transport over PC5.

-	Indicate the UE serving network supports Ranging/SL Positioning.

NOTE:	The Ranging/SL Positioning Support is configured in the AMF according to network capabilities.

Next CHANGE

### 5.2.3	SL Positioning Server UE Discovery & Selection

SL Positioning Server UE Discovery & Selection is performed by the Target UE, when it meets one or more of the following criteria:

-	The Target UE and the discovered SL Reference UEs are currently not served by a network supporting Ranging/SL Positioning (e.g. because they are out-of-coverage or the serving network does not support Ranging/SL Positioning). SL Reference UE and Target UE may indicate whether its serving network supports Ranging/SL Positioning or not to each other. The indication of serving network not supporting Ranging/SL Positioning can be reused to notify each other when UE has no NAS connection available.

Editor's note:	Need to determine if such indication is provided during discovery or capability exchange, and whether or not it requires RAN WG2 involvement.

-	The Target UE is not capable of performing SL Positioning Server UE functionalities.

-	The SL Reference UEs are not capable of performing SL Positioning Server UE functionalities.

The discovery of SL Positioning Server UE follows the same principles as specified in clause 5.2.1. The UE can indicate its role “SL Positioning Server UE” in its list of supported roles during discovery, if it is authorized to be a SL Positioning Server UE in a given PLMN as per the Authorization and Provisioning for Ranging/SL positioning service as specified in clause 5.1.

The Target UE shall discover and select a SL Positioning Server UEs that are in the same or different serving PLMN of the Target UE and the Reference UE(s).

Editor's note:	Other criteria for the selection of SL Positioning Server UE is FFS, and can be coordinated with RAN WGs.

Next CHANGE

### 6.3.2	Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [3] clause 4.2.2.2 with the following additions:

-	The UE includes the Ranging/SL positioning Capability as part of the "5GMM capability" in the Registration Request message. The AMF stores the Ranging/SL positioning Capability for Ranging/SL Positioning operation:

-	The Ranging/SL Positioning Capability indicates whether the UE supports one or more of the following Ranging/SL Positioning capabilities:

-	Capability of supporting Ranging/SL Positioning over PC5.

-	Capability of supporting SL Positioning Server UE over PC5.

-	The AMF obtains the Ranging/SL positioning subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [3].

-	The AMF determines whether the UE is authorised to use Ranging/SL positioning services based on UE's Ranging/SL positioning Capability and the Ranging/SL positioning Service Authorisation included in the subscription data received from UDM. The AMF stores the authorized Ranging/SL positioning services Capability.

-	The AMF sends the authorized Ranging/SL positioning Capability for Ranging/SL positioning operation to PCF. Based on the received Ranging/SL positioning Capability from the AMF, the PCF provides the PC5 QoS parameters related to Ranging/SL positioning to AMF. The AMF stores such information as part of the UE context.

-	If the UE is authorised to use Ranging/SL positioning services, then the AMF shall include in a NGAP message sent to NG-RAN:

-	"Ranging/SL positioning services authorised" information, including one or more of the following:

-	whether the UE is authorized to use Ranging/SL Positioning over PC5;

-	whether the UE is authorized to act as a Located UE;

-	whether the UE is authorized to act as a SL Positioning Server UE.

-	the PC5 QoS parameters related to Ranging/SL positioning used by the NG-RAN for the resource management of UE's PC5 transmission for Ranging/SL Positioning services in network scheduled mode.

Editor's note:	"Ranging/SL positioning services authorised" information needs the coordination with RAN WGs.

-	If the UE is authorised to use Ranging/SL positioning services, AMF may include Ranging/SL Positioning Support indicator in the Registration Accept message to indicate that the serving network supports Ranging/SL Positioning.

-	If the UE is authorised to use Ranging/SL positioning services, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [3].

END OF CHANGES